3GPP TSG-SA WG6 Meeting #66	S6-251358

Göteborg, Sweden, 7th – 11th April 2025	(revision of S6-251041)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.392 | CR | 0001 | rev | 01 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MMTel Enabler architecture fixes in section 6.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MMTEl_APP |  |  |  |  |  | Date: |  |  | 2025-03-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MMTel Enabler architecture description in Section 6.2.1 of TS 23392 shows the connection between the MMTel Enablement Server with the EPC/5GC through the N33 interface to consume the IMS session management services. The N33 interface is exposed by the NEF, but the NEF and procedures for NEF exposure of IMS session management and event notification are not reflected in the text. This CR proposes the addition of the text describing this in Section 6.2.1. Revision: removed the clarification sentence on DCMTSI client |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the NEF and the reference and description of the NEF exposure of IMS session management via N33 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete MMTel Enaber architecture description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 6.2	MMTel Enabler architecture

### 6.2.1	General

Figure 6.2-1 shows the MMTel Enabler architecture.

In this release, the MMTel Enabler Server is hosted within the PLMN operator network which supports the MMTel services capabilities exposure.

Figure 6.2.1-1: MMTel Enabler architecture

The MMTel functional entities on the UE and the server sides are MMTel Enabler Client and MMTel Enabler Server respectively.

The MMTel Enabler Client communicates with the MMTel Enabler Server over the MMTel-1 reference point to support the application layer interactions needed in MMTel service.

NOTE	The interaction between MMTel Enabler Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

The MMTel Enabler Client also communicates with the DCMTSI client over the MMTel-4 reference point to downloading behavior on DCMTSI Client based on the application policy provided by the MMTel Enabler Server. The DCMTSI client is specified in 3GPP TS 26.114 [4].

The Application Server communicates with the MMTel Enabler Server over the MMTel-2 reference point to invoke the MMTel service related capabilities provided by MMTel Enabler Server,

The Controlling Application Server communicates with the MMTel Enabler Server over the MMTel-3 reference point to manage the information needed in MMTel service.

The MMTel Enablement Server communicates with the EPC/5GC via N33 to consume the IMS session management services as defined in 3GPP TS 23.502 [12].

The Application Server communicates with the Application Client on the UE over the App-1 reference point to provide the Data Channel Application specific logic.

The MMTel Enabler Client(s) interacts with SEAL Clients over the SEAL-C reference point specified for each SEAL service. The MMTel Enabler Server(s) interacts with SEAL Servers over the SEAL-S reference point specified for each SEAL service. The interaction between a SEAL Client and the corresponding SEAL Server is supported by SEAL-UU reference point specified for each SEAL service as specified in 3GPP TS 23.434 [6].

* * * END of Changes * * * *