3GPP TSG-SA WG6 Meeting #66	S6-251427

Göteborg, Sweden, 7th – 11th April 2025	(revision of S6-251359)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.392 | CR | 0002 | rev | 01 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MMTel Enabler Server and Client descriptions update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MMTel_APP |  |  |  |  |  | Date: |  |  | 2025-03-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The text in Section 6.3.2 describes the DCSF as exposing N33, which is not true as N33 is an interface exposed by the NEF, not by the DCSF (alignment with TS 23.228 Annex AC). In section 6.3.3, among the description of the different clients it is not clear that the only IMS enabled client is the DCMTSI client Revision: update all interfaces to latest from 23.228. Merge in S6-251205 and S6-251167, add co-signer. Revision: remove track marks in metadata of the CR |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the text in 6.3.2 related to consumed interfaces from core and IMS. Add clarification in 6.3.3 on the IMS interfaces support in UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect description for the interfaces from 5GC and IMS support and unclear where the IMS and DCH support is on UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.3.2	MMTel Enabler Server

The MMTel Enabler Server provides the server side functionalities corresponding to the MMTel service.

The MMTel Enabler Server supports:

-	the Application Server and Controlling Application Server to discover, to authenticate and to get authorization the MMTel/MMTel service APIs provided by the MMTel Enabler Server;

-	the interaction with Controlling Application Server to configure the DC application profile;

-	the integration of the multiple network layer capabilities provided by the underlying network, including the 3GPP core network and IMS into one service enabler layer API. The MMTel Enabler Server exposes these service enabler layer APIs to the Application Server and Controlling Application Server instead of exposes the network layer capabilities directly. The capabilities provided by the underlying network include:

a) the APIs provided by the NEF via N33 reference point, as defined in 3GPP TS 23.502 [12];

b)	the APIs provided by DCSF via DC4 and MDC3 reference points, as defined in 3GPP TS 23.228 [3]; and

c)	the APIs provided by MF via MDC2 reference point, as defined in 3GPP TS 23.228 [3].

NOTE: in this release, the MMTel Enabler Server interacts with IMS via IMS elements interfaces specified in 3GPP TS 23.228 [3] or implementation specified interfaces;

-	the communication with the corresponding functions in the underlying network on behalf of the Application Server and Controlling Application Server when the capabilities provided by the underlying network are needed to be invoked;

-	the interaction with MMTel Enabler Client for exchanging service configuration related information, e.g. the DC application profile, needed in MMTel service,

-	providing MMTel/MMTel specific value added services which can be used by the Application Server in the mmtel communication between UE and Application Server;

-	the interaction with SEAL functionalities to use the corresponding SEAL services in MMTel service, e.g. to use SEAL notification management service to update of DC application profiles to UE;

-	the storage of the application layer information associated with the MMTel services;

-	the media function to support the media handling/translation, e.g. acts as a media gateway of WebRTC or if supported media type of application is different with the UE;

-	the HTTP Proxy to support the establishes a Bootstrap data channel and application data channel, e.g. if the address type of application is different with the UE, e.g. IPv4 and IPv6, etc;

-	providing call control handling consolidation when receiving call control requests from multiple Application Servers, e.g. to merge same control commands, to reject call control requests from certain Application Server with low priority when conflict happens or to decide the call control request execution sequence, etc; and

-	acting as CAPIF API provider domain functions as specified in 3GPP TS 23.222 [9] and interacts with CAPIF core function via CAPIF-3/3e/4/4e/5/5e reference points.

* * *Second Change * * * *

### 6.3.3	MMTel Enabler Client

The MMTel Enabler Client provide the client side functionality needed in MMTel service. The MMTel enabler client is not IMS enabled and therefore it interacts with DCMTSI Client in the same UE to control the downloading behavior on DCMTSI Client based on the DC application profiles provided by the MMTel Enabler Server.

NOTE 1:	the DCMTSI client is responsible for creating bootstrap channel and actually downloading the data channel applications.

NOTE 2	The interaction between MMTel Enabler Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

* * * END of Changes * * * *