3GPP TSG-SA WG6 Meeting #67	S6-252421

Fukuoka, Japan, 19th – 23rd May 2025	 (revision of S6-251360, S6-252262)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.392 | CR | 0003 | rev | 03 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple call control handling coordination multiple AS alignment with SA2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MMTel_App |  |  |  |  |  | Date: |  |  | 2025-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, there are parts that need to be updated in the figure of the multiple call control handling coordination multiple AS alignment in 8.5.2. In this document, we proposed to update the figure in Clauses 8.5.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the figure in Clauses 8.5.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between description of procedure and its related figure in Clauses 8.5.2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: incorporate comments at SA6#67. Revision2: update the figure in S6-251360. Revision3: update the coversheet in S6-252262. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.5.2	Procedure

Figure 8.5.2-1 illustrates the procedure of multiple call control handling coordination among different Application Servers.

Pre-conditions:

1.	The Application Servers and MMTel Enabler Server have been configured with the necessary credentials to enable authenticating one another.

2.	The call control handling policy (e.g. the priority of each Application Server) is pre-configured in MMTel Enabler Server.

3.	Application Servers have completed call event subscription to MMTel Enabler Server and the MMTel Enabler Server subscribes the call event from  NEF.

4.	The MMTel Enabler Server is authorized to use NEF capability APIs as defined in 3GPP TS 23.502 [12]

Figure 8.5.2-1: Multiple call control handling coordination among different Application Servers

1.

The NEF notifies the MMTel Enabler Server about an event in the IMS session for a user via Nnef_ImsSessionManagement_Notify call or via a Nnef_ImsEE_Notify, depending on the IMS session event that occurred, as described in 3GPP TS 23.502 [12] and based on the events defined in 3GPP TS 23.228 [3] in Table AA.2.4.4.5.

2.	The MMTel Enabler Server checks which Application Server has subscribed this call event notification.

3.	The MMTel Enabler Server sends Call event notification to Application Server(s) which subscribed this call event notification. The Call event notification request includes information elements as specified in Annex AA.2.4.2 of 3GPP TS 23.228 [3].

4-5.Application Servers receive call event notification and sends Call control request (e.g. create new call session, terminate call session or call control, etc.) based on its own service logic. The Call control request includes information elements as specified in Annex AA.2.4.3 or Annex AA.2.4.4 of 3GPP TS 23.228 [3].

6.	The MMTel Enabler Server receives call control requests from different Application Servers.It checks whether these call control requests are contradictory and makes call control handling decision based on service policy and service priority (e.g. if no conflict happens, to decide call control request execution sequence; if conflict happens, to deny call control request from certain AS with lower priority, etc.).

7-8.The MMTel Enabler Server sends call control responses to each Application Server based on the call control handling decision including that whether the call control request is accepted and the failure reason if the call control request is not accepted. The call control response includes information elements as specified in Annex AA.2.4.3 or Annex AA.2.4.4 of 3GPP TS 23.228 [3].

9.	The MMTel Enabler Server sends call control requests decided in step 6 from Application Servers (or permitted Application Servers in conflict scenario) to the NEF by returning Nnef_ImsSessionManagement_notify response and if applicable, to request the addition of DCH media to the ongoing IMS session via a Nnef_ImsSessionManagement_Update request to the NEF as defined in 3GPP TS 23.502 [12].

10. The MMTel Enabler Server sends the call control result notification request to the permitted Application Servers in conflict scenario decided in step 6. The call control result notification includes information elements as specified in clause 8.5.3.1. The Application Server sends the corresponding control result notification response to the MMTel Enabler Server.

* * * END of Change * * * *