3GPP TSG-SA WG6 Meeting #66	S6-251221

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.392 | CR | 0007 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MMTel Enabler Client request DC application profiles based on notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MMTel_App |  |  |  |  |  | Date: |  |  | 2025-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Section 8.3.3.2.1 specifies that the SEAL notification service is used to notify updates of DC application profiles. However, the usage of SEAL and its relationship with the MMTel service are not clearly explained. This CR is proposed to clarify this issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is proposed to clarify the usage of SEAL notification service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The usage of SEAL and its relationship with the MMTel service are not clearly explained. This CR is proposed to clarify this issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.1, 8.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.3.3.2.1	The MMTel Enabler Client request DC application profiles based on notification

The MMTel Enabler Server can notify the MMTel Enabler Client in the UE to send application request messages to the MMTel Enabler Server to request the DC application profiles, the detailed procedure is illustrated in Figure 8.3.3.2.1-1.

NOTE:	The MMTel Enabler Server can instruct the MMTel Enabler Client to request root application and DC application profiles as required. e.g. root application version update, offline of a data channel application, etc.

Pre-condition:

1.	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.3.3.2.1-1: MMTel Enabler Client request DC application profiles based on notification

1.	The notification channel has been established between notification management client and notification management Server as per step 1 to step 6 specified clause 17.3.3 of TS 23.434 [6].

2.	The MMTel Enabler Client subscribes to the MMTel Enabler Server for receiving notifications per step 7 specified clause 17.3.3 of TS 23.434 [6].

If the MMTel Enabler Server provides the notification service, step 1 and  step 2 are skipped.

3.	The MMTel Enabler Server sends a Root application and profile update notification to notify the MMTel Enabler Client in the UE to get the Root application or DC application profile. The Root application and profile update notification includes information elements as specified in Table 8.3.3.3.1-1.

The step 4 to step 8 are same as steps 1 through 5 in clause 8.3.2.2.

* * * Next Change * * * *

8.3.3.3.1	Root application and profile update notification

If the MMTel Enabler Server interacts with the SEAL notification server to use the notification service, the Notification message specified in clause 17.3.2.4 applies to the Root application and profile update notification. The usage of the information elements in MMTel service is clarified as below:

-	The identifier of the MMTel Enabler Client request DC application profiles works as VAL Application ID;

-	The service identifier of MMTel Service in SEAL services works as VAL Service ID;

Besides the IEs specified in clause 17.3.2.4 of 3GPP TS 23.434 [6], the information elements included in Table 8.3.3.3.1-1 is also needed to be included in the Root application and profile update notification from the MMTel Enabler Server to the MMTel Enabler Client.

Table 8.3.3.3.1-1: Information elements in Root application and profile update notification

| Information element | Status | Description |
| --- | --- | --- |
| Urgent | O | Indicates whether the operation is urgent to UE, i.e. whether the Root application and profile need to be updated immediately. E.g. the update of Root application and profile is urgent if the available DC Application for the user is changed |

If the MMTel Enabler Server provides the notification service, only the information elements included in Table 8.3.3.3.1-1 are included in the Root application and profile update notification.