3GPP TSG-SA6 Meeting #67	S6-252418

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025	                                   was S6252072

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.392 | CR | 0009 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections related to Information elements across various procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MMTel_App |  |  |  |  |  | Date: |  |  | 2025-5-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - DC Application list already provides the number of application, hence there is no need for another attribute to indicate the Number of DC Application. - Many of the Information element names are not aligned with the description, which are renamed to more appropriate names. - Reference to tables are corrected. - Unncessary tables are removed (which are duplicate). - Instead of two information elements indicating successful response and failure response, one information element indicating Result is added along with cause (in case of failure) in alignment with other procedures. - Media information was updated from M to O in couple of instances based on description/condition. - Many typo's are corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | All the above indicated changes are done in the clauses included in the clauses affected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect stage-2 requirements and poor quality of the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.3.1, 8.2.3.3.1, 8.2.3.3.2, 8.2.4.3.1, 8.2.4.3.2, 8.2.5.3.1, 8.2.5.3.2, 8.3.2.3.1, 8.3.2.3.2, 8.3.2.3.3, 8.3.2.3.4, 8.3.3.3.1, 8.3.3.3.2, 8.4.3.3.1, 8.4.3.3.2, 8.4.4.2.1, 8.4.4.2.2, 8.4.4.2.3, 8.4.4.2.4, 8.4.5.2.1, 8.4.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.2.2.3.1	DC application and profile configuration request

Table 8.2.2.3.1-1 describes information elements for the DC application and profile configuration request from the Controlling Application Server to the MMTel Enabler Server.

Table 8.2.2.3.1-1: Information elements in DC application and profile configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| DC Application list | M | A list of DC application and configuration parameters included in this request. Each element in this list contains a Data Channel Application package and corresponding configuration parameters. The detailed information elements of DC application list are listed in the Table 8.2.2.3.1-2. |

Table 8.2.2.3.1-2: Information elements in DC application list

| Information element | Status | Description |
| --- | --- | --- |
| Application Identifier | M | Identifier of the Data Channel Application assigned by the Application Provider |
| Application Name (see NOTE 1) | O | Name of the Data Channel Application |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application |
| Application Icon Url | O | Icon url of the Data Channel Application |
| Application Version (see NOTE 1) | O | The latest version of the application. |
| Application Validity | O | The latest validity of the application. |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: Precall only: The Data Channel Application is allowed to be used before the MMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. Incall only: The Data Channel Application is allowed to be used after the MMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. Both precall and Incall: The Data Channel Application is allowed to be used during the entire Precall and incall. |
| Autoload (see NOTE 3) | O | Indicates whether this Data Channel Application is needed to be loaded to the UE automatically. |
| Autolaunch (see NOTE 3) | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and launched automatically. |
| Usage Indication (see NOTE 2) | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Requested Media Type (see NOTE 2) | O | Indicates media type required for this Data Channel Application. The values of this IE include: Voice call only: This Data Channel Application can be used if and only if the corresponding call is a voice call. Video call only: This Data Channel Application can be used if and only if the corresponding call is a video call. Both Voice and video call: This Data Channel Application can be used in both voice call and video call. |
| Application Usage Condition (see NOTE 2) | O | Indicates whether this Data Channel Application can be used based on certain condition. The values of this IE include, but not limited: Service area: the application is allowed to be used in a certain area. Time period: the application is allowed to be used for a certain period of time. |
| 3gpp QoS Hint (see NOTE 2) | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| Personal Data Collection Indication(see NOTE 4) | O | Indicates whether this Data Channel Application will collect the user personal data (along with related consent opt-in/opt-out procedures). |
| Personal Data Collection Info URL (see NOTE 4) | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected, their processing their protection etc. |
| Application Package (see NOTE 1) | O | The Code package of the Data Channel Application. |
| NOTE 1: This IE shall be present when when configuring the DC application. NOTE 2: This IE shall be present when when configuring the DC application profile. NOTE 3: These IEs can be applied only if the specific service using this data channel application is agreed to be used by the user based on the subscription of this specific service or explicitly agreed to be applied by the user on the UE. The relationship with Personal Data Collection should be considered, which is out of 3GPP scope. NOTE 4: These IEs are mandatory when regulatory requirements request that the user has to be informed if the personal data are collected by the application and how they are treated. |  |  |

* * * Next Change * * * *

##### 8.2.3.3.1	DC application and profile update request

Table 8.2.3.3.1-1 describes information elements for the DC application and profile update request from the Controlling Application Server to the MMTel Enabler Server.

Table 8.2.3.3.1-1: Information elements in DC application and profile update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| DC Application list | M | A list of DC application and configuration parameters included in this request. Each element in this list contains a Data Channel Application package and corresponding configuration parameters. The detailed information elements of DC application list are listed in Table 8.2.2.3.1-2. |

* * * Next Change * * * *

##### 8.2.3.3.2	DC application and profile update response

Table 8.2.3.3.2-1 describes information elements for the DC application and profile update response from the MMTel Enabler Server.

Table 8.2.3.3.2-1: Information elements for DC application and profile update response

| Information element | Status | Description |
| --- | --- | --- |
| DC application and profile update response list | M | A list of application and profile update response. This list contains a response for each DC application and profile. The detailed information is listed in Table 8.2.3.3.2-2. |

Table 8.2.3.3.2-2: Information elements in DC application and profile update response list

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the DC application and profile update resquest was successful or failure. |
| > Cause | O | Indicates the cause of DC application and profile update request failure. |

* * * Next Change * * * *

##### 8.2.4.3.1	DC Application delete request

Table 8.2.4.3.1-1 describes information elements for the DC Application delete request from the Controlling Application Server to the MMTel Enabler Server.

Table 8.2.4.3.1-1: Information elements in DC Application delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| List of APPID | M | List of identifiers of the DC application to be deleted. |

* * * Next Change * * * *

##### 8.2.4.3.2	DC Application delete response

Table 8.2.4.3.2-1 describes information elements for the DC Application delete response from the MMTel Enabler Server.

Table 8.2.4.3.2-1: Information elements for DC Application delete response

| Information element | Status | Description |
| --- | --- | --- |
| DC Application delete response list | M | A list of Application deletion response. The detailed information is listed in Table 8.2.4.3.2-2. |

Table 8.2.4.3.2-2: Information elements in DC Application delete response list

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the deletion of the DC Application was successful or failure. |
| > Cause | O | Indicates the cause of failure. |

* * * Next Change * * * *

##### 8.2.5.3.1	Getting DC application profile information request

Table 8.2.5.3.1-1 describes information elements for the Getting DC application profile information request from the Controlling Application Server to the MMTel Enabler Server.

Table 8.2.5.3.1-1: Information elements in Getting DC application profile information request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| APPID list | M | A list of APPID for the request. |

* * * Next Change * * * *

##### 8.2.5.3.2	Getting DC application profile information response

Table 8.2.5.3.2-1 describes information elements for the Getting DC application profile information response from the MMTel Enabler Server.

Table 8.2.5.3.2-1: Information elements in Getting DC application profile information response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the Getting DC application profile information request was successful. |
| > List of DC application profile information | O | List of DC application profile information as defined in Table 8.3.2.3.4-2. |
| Failure response | O | Indicates that the Getting DC application profile information request failed. |
| > Cause | O | Indicates the cause of Getting DC application profile information request failure. |

* * * Next Change * * * *

##### 8.3.2.3.1	Get Root application request

Table 8.3.2.3.1-1 describes information elements for the Get Root application request from the MMTel Enabler Client to the MMTel Enabler Server.

Table 8.3.2.3.1-1: Information elements in Get Root application request

| Information element | Status | Description |
| --- | --- | --- |
| UE Information (see NOTE) | M | The UE related information required to identify the Root application (e.g. device type, device vendor, etc) |
| Root application version | O | The latest version of the Root application stored in the MMTel Enabler Client in the UE. This IE is present if Root application is locally stored in the MMTel Enabler Client. |
| NOTE: The MMTel service provider can provide the MMTel Enabler Client with different Root application based on this IE. |  |  |

* * * Next Change * * * *

##### 8.3.2.3.2	Get Root application response

Table 8.3.2.3.2-1 describes information elements for the Get Root application response from the MMTel Enabler Server to the MMTel Enabler Client.

Table 8.3.2.3.2-1: Information elements for Get Root application response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of Get Root application request. |
| Update needed | M | Indicates whether the Root application is needed to be updated by the MMTel Enabler Client in the UE |
| Root application version | O | The latest version of the Root application. This IE presents only if the Root application is needed to be downloaded or updated by the MMTel Enabler Client in the UE. |
| Root application validity | O | The validity of the Root application. This IE presents only if the Root application is needed to be downloaded or updated by the MMTel Enabler Client in the UE. |
| Root application (see NOTE) | O | The Root application provided by the MMTel service provider, e.g. layout, and/or home page etc of the Data Channel Application list. This IE is present if the update of the Root application in the UE is needed, i.e. the Root application version included in the Get Root application request is not equal to the latest version on the MMTel Enabler Server. |
| NOTE: The detailed information of this IE is implementation specific and out of scope of the present document. |  |  |

* * * Next Change * * * *

##### 8.3.2.3.3	Get DC application profile List request

Table 8.3.2.3.3-1 describes information elements for the Get DC application profile List request from the MMTel Enabler Client to the MMTel Enabler Server.

Table 8.3.2.1-3: Information elements in Get DC application profile List request

| Information element | Status | Description |
| --- | --- | --- |
| MMTel Enabler Client version (see NOTE) | M | The version of MMTel Enabler Client. |
| application type | O | Type of application (e.g. emergency, conference, XR, etc.). |
| Begin index | O | The starting index of the first Data Channel Application required in the Data Channel Application list. This IE is present if the requested Data Channel Application is not the first one in the Data Channel Application list, e.g. when the Data Channel Application list is needed to be shown in multiple pages, this IE is included when requesting the pages other than the first page. The default value of this IE is 0, if this IE is absent. |
| NOTE: The MMTel service provider can provide the MMTel Enabler Client with different DC application profile list based on this IE. |  |  |

* * * Next Change * * * *

##### 8.3.2.3.4	Get DC application profile List response

Table 8.3.2.3.4-1 describes information elements for the Get DC application profile List response from the MMTel Enabler Server to the MMTel Enabler Client.

Table 8.3.2.3.4-1: Information elements in Get DC application profile List response

| Information element | Status | Description |
| --- | --- | --- |
| DC Application profile list | M | A list of DC application profiles available for this user. Each element in this list contains a DC application profile of this Data Channel Application. The detailed information elements of DC application profile are listed in Table 8.3.2.3.4-2. |

Table 8.3.2.3.4-2: Information elements in DC application profile

| Information element | Status | Description |
| --- | --- | --- |
| APPID | M | Identifier of the Data Channel Application. |
| Application Name | O | Name of the Data Channel Application. |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application. |
| Application Icon | O | Icon of the Data Channel Application. |
| Application Version | O | The latest version of the application. |
| Application Validity | O | The latest validity of the application. |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: Precall only: The Data Channel Application is allowed to be used before the MMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. Incall only: The Data Channel Application is allowed to be used after the MMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. |
| Autoload (see NOTE 1) | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch (see NOTE 1) | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and launched automatically. |
| Usage Indication | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Requested Media Type | O | Indicates media type required for this Data Channel Application. The values of this IE include: Voice call only: This Data Channel Application can be used if and only if the corresponding call is a voice call. Video call only: This Data Channel Application can be used if and only if the corresponding call is a video call. Both voice and video call: This Data Channel Application can be used in both voice call and video call. |
| Application Usage Condition | O | Indicates whether this Data Channel Application can be used if under this condition. The values of this IE include but not limited: Service area: The application is allowed to be used in a certain area. Time period: The application is allowed to be used for a certain period of time. |
| 3gpp QoS Hint | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| Personal Data Collection (see NOTE 2) | O | Indicates whether this Data Channel Application will collect the user personal data. |
| Personal Data Collection Info URL (see NOTE 2) | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected (along with related consent opt-in/opt-out procedures), their processing their protection etc. |
| NOTE 1: These IEs can be applied only if the specific service using this data channel application is agreed to be used by the user based on the subscription of this specific service or explicitly agreed to be applied by the user on the UE. NOTE 2: These IEs are mandatory when regulatory requirements request that the user has to be informed if the personal data are collected by the application and how they are treated. |  |  |

* * * Next Change * * * *

##### 8.3.3.3.1	Root application and profile update notification

Table 8.3.3.3.1-1 describes information elements for the Root application and profile update notification from the MMTel Enabler Server to the MMTel Enabler Client.

Table 8.3.2.3.1-1: Information elements in Root application and profile update notification

| Information element | Status | Description |
| --- | --- | --- |
| Immediate Indication | O | Indicates whether the operation is critical to UE, i.e. whether the Root application and profile need to be updated immediately. E.g. the update of Root application and profile is critical, if the available DC Application for the user is changed. |

* * * Next Change * * * *

##### 8.3.3.3.2	Root application notification

Table 8.3.3.3.2-1 describes information elements for the Root application notification from the MMTel Enabler Server to the MMTel Enabler Client.

Table 8.3.3.3.2-1: Information elements for Root application notification

| Information element | Status | Description |
| --- | --- | --- |
| Update Indication | M | Indicates whether the Root application is needed to be updated by the MMTel Enabler Client in the UE. |
| Root application version | O | The latest version of the Root application. This IE presents only if the Root application is needed to be updated by the MMTel Enabler Client in the UE. |
| Root application (see NOTE) | O | The Root application provided by the MMTel service provider, e.g. layout, and/or home page etc. of the Data Channel Application list. This IE is present if the update of the Root application in the UE is needed, i.e. the Root application version included in the Get Root application request is not equal to the latest version on the MMTel Enabler Server. |
| NOTE: The detailed information of this IE is implementation specific and out of scope of the present document. |  |  |

* * * Next Change * * * *

##### 8.4.3.3.1	Third-Party Call with DC capability establishment request

Table 8.4.3.3.1-1 describes information elements for the Third-Party Call with DC capability establishment request from the Application Server to the MMTel Enabler Server.

Table 8.4.3.3.1-1: Third-Party Call with DC capability establishment request

| Information element | Status | Description | Related OMA API Element |
| --- | --- | --- | --- |
| Originating ID (see NOTE) | M | The identifier of the caller. | The first element in the participant list, which considered to denote the “A-Party” specified in OMA Third Party Call API [7]. |
| Terminating ID (see NOTE) | M | The identifier of the callee. | The second element in the participant list, which considered to denote the “B-Party” specified in OMA Third Party Call API [7]. |
| Media information | O | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of MMTel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. | The mediaInfo of the CallSessionInformation specified in OMA Third Party Call API [7]. |
| DC media information | O | Identifier of whether DC media is expected to be used in this call | N/A |
| Application Profile requested | O | The DC application profile expected to be used in this call | N/A |
| Notification information | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. | The callbackReference of the CallSessionInformation specified in OMA Third Party Call API [7]. |
| NOTE: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. |  |  |  |

* * * Next Change * * * *

##### 8.4.3.3.2	Third-Party Call with DC capability establishment response

Table 8.4.3.3.2-1 describes information elements for the Third-Party Call with DC capability establishment response from the Application Server to the MMTel Enabler Server.

Table 8.4.3.3.2-1: Third-Party Call with DC capability establishment response

| Information element | Status | Description | Related OMA API Element |
| --- | --- | --- | --- |
| Call establishment result | M | Indication if the Call establishment is success or failure. | The HTTP response codes of the created call session as specified in OMA REST_NetAPI_Common [11]. |
| Session ID (see NOTE 1) | O | The identifier of the call session. | The callSessionId as specified in OMA Third Party Call API [7]. |
| Failure Cause (see NOTE 2) | O | The reason for failure | The Policy Exception and Service Exception fault as specified in OMA Third Party Call API [7]. |
| Originating ID (see NOTE 3) | M | The identifier of the caller. | The first element in the participant list, which considered to denote the “A-Party” specified in OMA Third Party Call API [7]. |
| Terminating ID (see NOTE 3) | M | The identifier of the callee. | The second element in the participant list, which considered to denote the “B-Party” specified in OMA Third Party Call API [7]. |
| Media information | O | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of MMTel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. | The mediaInfo of the CallSessionInformation specified in OMA Third Party Call API [7]. |
| DC media information | O | Identifier of whether DC media is expected to be used in this call. | N/A |
| Application Profile requested | O | The DC application profile expected to be used in this call. | N/A |
| NOTE 1: This IE shall only be present when the Call establishment result is Success. NOTE 2: This IE shall only be present when the Call establishment result is Failure. NOTE 3: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. |  |  |  |

* * * Next Change * * * *

##### 8.4.4.2.1	Application Add Data Channel media to UE1 session (A2P) request

Table 8.4.4.2.1-1: Application Add Data Channel media to UE1 session (A2P) request

| Information element | Status | Description |
| --- | --- | --- |
| Data Channel APPID | M | The identifier of the DCH application. |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Media resource information | O | The media information to be passed via the Data Channel. It includes information described in 3GPP TS 23.502 [12] clause 5.2.6.40. |
| Application Profile requested | O | The DC application profile expected to be used in this call. |
| Notification address | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. |

* * * Next Change * * * *

##### 8.4.4.2.2	Application Add Data Channel media to UE1 session (A2P) response

Table 8.4.4.2.2-1: Application Add Data Channel media (A2P) response

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Result | M | Indicates the Notificaiton of Application of DCH media info (A2P) request was successful or failure. |
| > Cause | O | Indicates the cause of request failure. |

* * * Next Change * * * *

##### 8.4.4.2.3	Notify Application of DCH media info (A2P) request

Table 8.4.4.2.3-1: Notify Application of DCH media info (A2P) request

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Notification address | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. |
| Media resource information | O | The media information to be passed via the Data Channel. It includes information described in 3GPP TS 23.502 [12] clause 5.2.6.40. |
| Application Profile requested | O | The DC application profile expected to be used in this call. |

* * * Next Change * * * *

##### 8.4.4.2.4	Notify Application of DCH media info (A2P) response

Table 8.4.4.2.4-1: Notify Application of DCH media info (A2P) response

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Result | M | Indicates the Notificaiton of Application of DCH media info (A2P) request was successful or failure. |
| > Cause | O | Indicates the cause of request failure. |

* * * Next Change * * * *

##### 8.4.5.2.1	Notify DCH Application of MDC2 media (P2A) request

Table 8.4.5.2.1-1: Notify DCH Application of MDC2 media (P2A) request

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Media resource information | O | The MDC2 media information requested by the UE1 to be sent over the Data Channel. It includes information described in 3GPP TS 23.502 [12] clause 5.2.6.40. |

* * * Next Change * * * *

##### 8.4.5.2.2	Notify DCH Application of MDC2 media (P2A) response

Table 8.4.5.2.2-1: Notify DCH Application of MDC2 media (P2A) response

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Result | M | Indicates the Notificaiton of Application of DCH media info (A2P) request was successful or failure. |
| > Cause | O | Indicates the cause of request failure. |

* * * End of Changes * * * *