3GPP TSG-SA6 Meeting #67	S6-252074

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.392 | CR | 0011 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update NOTE in the DC application profiles downloading procedure. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MMTel_App |  |  |  |  |  | Date: |  |  | 2025-5-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NOTE in the procedure defined in the clause 8.3.2.2 proposes to include a negative value to indicate UE to delete an existing Root application. The same can be achieved by using a NULL value, which is a general and well known practice. (Instead of using negative value, which might cause mis interpretation during the code implementation.) |  |  |  |  |  |  |  |  |
| Summary of change: |  | NOTE in the clause 8.3.2.2 is updated to indicate NULL value instead of negative value. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect stage-2 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.3.2.2	Procedure

Figure 8.3.2.2-1 illustrates the DC application profiles downloading procedure.

Pre-conditions:

1.	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.3.2.2-1: MMTel Enabler Client in the UE requests the DC application profile list

1.	The MMTel Enabler Client in the UE sends a Get Root application request to MMTel Enabler Server to get the Root application. The request message includes information elements as specified in Table 8.3.2.3.1-1. The application type is used by MMTel Enabler Server to suggest DC applications for download.

NOTE 1:	In a implementation, the DCSF may support the DC application profile functionality of MMTel Enabler Server function, the DCSF may also interact with the MMTel Enabler Server to support this procedure, i.e. The MMTel Enabler Client sends a request to the DCSF, which then sends the request to the MMTel Enabler Server. The response in the opposite direction is also sent to the MMTel Enabler Client through the DCSF.

NOTE 2:	The interaction between DCSF and MMTel Enabler Server is implementation specific in this release.

2.	The MMTel Enabler Server checks whether the Root application version included in the Get Root application request is the newest version of the Root application. If the Root application version IE is not included, the Root application version is not considered as newest.

3.	The MMTel Enabler Server sends a Get Root application response to the MMTel Enabler Client in the UE. The response message includes information elements as specified in Table 8.3.2.3.2-1.

If Root application is locally stored in the MMTel Enabler Client, validity of the application will be updated to the value received in Root application validity IE.

NOTE 3:	When service provider wants the UE to delete the stored Root application, the response message includes NULL value in the Root application validity IE.

NOTE 4:	The root application can also be implementation specific.

4.	The MMTel Enabler Client in the UE send a Get DC application profile List request to MMTel Enabler Server to get the DC application profile list available for this user. The request message includes information elements as specified in Table 8.3.2.3.3-1.

5.	The MMTel Enabler Server sends a Get DC application profile List response to the MMTel Enabler Client in the UE. The response message includes information elements as specified in Table 8.3.2.3.4-1. The response message includes list of Data Channel Application profile.

The MMTel Enabler Client locally stores the downloaded DC application profiles and displays the related application information from the DC application profile on the Root application. Users can select and download corresponding DC applications based on the profile information on the Root application. The MMTel Enabler Client may stores the downloaded DC Application which may be obsoleted in the condition of, e.g. the validity is expired, update available for application, etc.

* * * End of Changes * * * *