3GPP TSG-SA WG6 Meeting #67	S6-252217

Fukuoka, Japan, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.392 | CR | 0017 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to cl. 8.4.4 and cl. 8.4.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MMTel_App |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, in Clauses 8.4.4 and 8.4.5, the data channel is abbreviated as DCH. In this document, we proposed to change the abbreviation of DCH to DC for the unification of the names within the overall document. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the abbreviation of data channel from DCH to DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect description of abbreviation of data channel and it may not be aligned with the SA2 Rel-19 TS 23.228 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.4.1, 8.4.4.2.1, 8.4.4.2.2, 8.4.4.2.3, 8.4.4.2.4, 8.4.5.1.1, 8.4.5.2.1, 8.4.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.4.4.1	Procedure

Figure 8.4.4-1 illustrates the operation to establish an Application Call with Data Channel capability between the Application and the MMTel Enabler Server in order to add an A2P Data Channel to an existing IMS session of the user on UE1 in alignment with the A2P flow specified in 3GPP TS 23.228 [3].

Pre-conditions:

1.	The Application can use DC (supports the media set information for DC) and is authorized to use the service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [3].

3.	The MMTel Enabler Server is authorized to use the NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [12].

4.	The MMTel Enabler Server is subscribed to the AEF (NEF) for IMS session events information e.g., media changes such as requested for the specific DC Application, for a list of users.

NOTE:	How the IMS session ID is provided by the NEF exposure of IMS events in 3GPP TS 23.502 [12] is out of scope of this specification.

5.	The DC Application Server is subscribed to the MMTel Enabler Server for events related to the DC application.

6.	An IMS MMTel session is ongoing for User (UE1) with another user who requests the DC Application to provide some DC media to the user on UE1.

Figure 8.4.4-1: Establishing a Application Call with Data Channel capability

1.	The user sends a request, via the 5GC and IMS, to add DC media for a specific DC Application. The NEF notifies the MMTel Enabler Server of the new IMS session event for the for the media changes related to the DC Application. The request includes parameters such as the session ID, MDC2 endpoint, A2P type.

2.	The MMTel Enabler Server requests an IMS session update to the NEF, providing the Session ID and MDC2 endpoint address. The NEF propagates the request to the IMS core which sets the media operation. The NEF sends the response to the MMTel Enabler to the IMS session Update request.

3.	The MMTel Enabler Server sends the response to the DC Application Server including the Session ID for the Add media request.

4.	Once the IMS system completed the media preparation for the IMS session, the NEF sends an IMS session management notification request to the MMTel Enabler Server with MDC2 resource information for the Session ID. The DC Application Server responds (acknowledges) the MMTel Enabler Server notification.

5.	The MMTel Enabler Server notifies the DC Application Server of the MDC2 resource information for the Session ID.

6.	The MMTel Enabler Server acknowledges the NEF notification.

Post-condition: The UE1 upon getting the updated IMS session information for the DC media proceeds to download the DC Application.

* * * Next Change * * * *

##### 8.4.4.2.1	Application Add Data Chanel media to UE1 session (A2P) request

Table 8.4.4.2.1-1: Application Add Data Chanel media to UE1 session (A2P) request

| Information element | Status | Description |
| --- | --- | --- |
| Data Channel APPID | M | The identifier of the DC application |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Media resource information | O | The media information to be passed via the Data Chanel. It includes information described in 3GPP TS 23.502 [12] clause 5.2.6.40. |
| Application Profile requested | O | The DC application profile expected to be used in this call. |
| Notification address | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. |

##### 8.4.4.2.2	Application Add Data Chanel media to UE1 session (A2P) response

Table 8.4.4.2.2-1: Application Add Data Chanel media (A2P) response

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Successful response | O | Indicates the Notificaiton of Application of DC media info (A2P) request was successful |
| Failure response | O | Indicates the Notificaiton of Application of DC media info (A2P) request has failed |
| > Cause | O | Indicates the cause of request failure |

##### 8.4.4.2.3	Notify Application of DC media info (A2P) request

Table 8.4.4.2.3-1: Notify Application of DC media info (A2P) request

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Notification address | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. |
| Media resource information | O | The media information to be passed via the Data Chanel. It includes information described in 3GPP TS 23.502 [12] clause 5.2.6.40. |
| Application Profile requested | O | The DC application profile expected to be used in this call. |

##### 8.4.4.2.4	Notify Application of DC media info (A2P) response

Table 8.4.4.2.4-1: Notify Application of DC media info (A2P) response

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Successful response | O | Indicates the Notificaiton of Application of DC media info (A2P) request was successful |
| Failure response | O | Indicates the Notificaiton of Application of DC media info (A2P) request has failed |
| > Cause | O | Indicates the cause of request failure |

* * * Next Change * * * *

##### 8.4.5.1.1	Establishing an Application Call with Data Channel capability (P2A)

Figure 8.4.5.1.1-1 illustrates the operation to establish an Application Call with Data Channel capability between a user (on UE1) and a Data Chanel (DC) Application via the MMTel Enabler Server.

Pre-conditions:

1.	The Application can use DC (supports the media set information for DC) and is authorized to use the service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [3].

3.	The MMTel Enabler Server is authorized to use the NEF exposed IMS session management capabilities as specified in [8].

4. The MMTel Enabler Server is subscribed to the AEF (NEF) for IMS session events information for the specific DC Application (using the DC Application Called party ID).

NOTE:	How the IMS session ID is provided by the NEF exposure of IMS events in 3GPP TS 23.502 [12] is out of scope of this specification.

5.	The DC Application Server is subscribed to the MMTel Enabler Server for events related to the DC application.

6.	An IMS MMTel session is ongoing for User (UE).

Figure 8.4.5.1.1-1: Establishing a Application Call with Data Channel capability (P2A)

1.	The user (on UE1) sends a request, via the 5GC and IMS, to add DC media for a specific DC Application.

2.	The P2A session establishment is handled via 5GC and IMS system.

3.	NEF notifies the MMTel Enabler Server of the new IMS session event for the DC Application.

4.	The MMTel Enabler Server notifies the DC Application Server of the new request for DC media.

5.	The DC Application Server accepts the request and provides the MDC2 media resource requested.

6.	The MMTel Enabler Server utilizes the IMS session update capability exposed by the NEF [8] to request addition of the DC media to the existing IMS MMTel Call Session.

7.	The NEF further interacts with the IMS system to request the DC media addition to the ongoing MMTel session and IMS further handles this as specified in Annex AC of 3GPP TS 23.228 [3].

Post-condition: The Application data channel is activated.

* * * Next Change * * * *

##### 8.4.5.2.1	Notify DC Application of MDC2 media (P2A) request

Table 8.4.5.2.1-1: Notify DC Application of MDC2 media (P2A) request

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Media resource information | O | The MDC2 media information requested by the UE1 to be sent over the Data Chanel. It includes information described in 3GPP TS 23.502 [12] clause 5.2.6.40. |

##### 8.4.5.2.2	Notify DC Application of MDC2 media (P2A) response

Table 8.4.5.2.2-1: Notify DC Application of MDC2 media (P2A) response

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Successful response | O | Indicates the Notificaiton of Application of DC media info (A2P) request was successful |
| Failure response | O | Indicates the Notificaiton of Application of DC media info (A2P) request has failed |
| > Cause | O | Indicates the cause of request failure |

* * * END of Change * * * *