3GPP TSG-SA3 Meeting #121	S3-251676

Goteborg, Sweden, 7 – 11 April 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2133 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Roaming and interconnect authorization aspects in indirect communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson, MITRE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2025-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Roaming and interconnect scenarios have not been addressed for indirect communication scenarios. While for target PLMN NF selection a specific WID has been created in CT4, source PLMN NF selection is straightforward, using the exisiting procedures, i.e. no new feature but Cat-F correction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause on roaming and interconnect scenarios for indirect communication scenarios including the related section on source PLMN NF selection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details on roaming and interconnect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.1.X (new), 13.4.1.X.1 (new), 13.4.1.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3-251635 |  |  |  |  |  |  |  |  |

***** START OF CHANGES

#### 13.4.1.X	Service access authorization in interconnect and roaming scenarios with indirect communication

##### 13.4.1.X.1		General

This clause specifies specific requirements to support indirect communication with or without delegated discovery between different PLMNs as specified in clause 6.3.1 of 3GPP TS 23.501 [2] and clause 4.17.10a of 3GPP TS 23.502 [8].

The terms "source PLMN" and "target PLMN" throughout this clause refer respectively to the PLMN originating the request (i.e. the PLMN of the NF Service Consumer) and the PLMN receiving/serving the request (i.e. the PLMN of the NF Service Producer).

The OAuth 2.0 roles for interconnection scenario are described in clause 13.4.1.1A.

#### 13.4.1.X.2	Authorization with NF selection at source network

##### 13.4.1.X.2.0	General

In the scenario where NF Service Consumer and NF Service Producer are in different PLMNs and indirect communication (with or without delegated discovery) is deployed in the source network, the NF selection is performed at the source PLMN regardless of whether indirect communication is deployed in the target network or not. The SCP at the source PLMN sends an access token request to the target PLMN. The architecture is described in clause 4.17.10a of TS 23.502 [8].

##### 13.4.1.X.2.1	Authorization for indirect communication with or without delegated discovery procedure

Figure 13.4.1.X.2-1: Service access authorization in indirect communication (with or without delegated discovery) when NF Service Consumer and NF Service Producer are in different PLMNs and NF selection is at the source PLMN

Steps 1 and 2 of figure 13.4.1.X.2-1 correspond to one of the following:

-	Steps 0 to 4 in clause 13.4.1.3.1.1 (Authorization for indirect communication scenarios without delegated discovery, with mutual authentication between NF Service Consumer and NRF at the transport layer),

-	Steps 0 to 6 in clause 13.4.1.3.1.2 (Authorization for indirect communication scenarios without delegated discovery, without mutual authentication between NF Service Consumer and NRF at the transport layer), or

-	Steps 1 to 5 in clause 13.4.1.3.2 (Authorization for indirect communication with delegated discovery procedure),

with the following differences:

-	The NRF in the source PLMN itself does not perform the authorization of the NF Service Consumer. Instead, the NRF in the source PLMN forwards the access token request to an NRF in the target PLMN, via the SEPPs of the source PLMN and target PLMN.

-	The NRF in the target PLMN authorizes the NF Service Consumer as described for the hNRF in clause 13.4.1.2.2 (Service access authorization in roaming scenarios, service request process).

Steps 3 to 6 of figure 13.4.1.X.2-1 correspond to one of the following:

-	Steps 5 to 8 in clause 13.4.1.3.1.1 (Authorization for indirect communication scenarios without delegated discovery, with mutual authentication between NF Service Consumer and NRF at the transport layer),

-	Steps 7 to 10 in clause 13.4.1.3.1.2 (Authorization for indirect communication scenarios without delegated discovery, without mutual authentication between NF Service Consumer and NRF at the transport layer), or

-	Steps 6 to 9 in clause 13.4.1.3.2 (Authorization for indirect communication with delegated discovery procedure)

with the following differences:

-	The SCP in the source PLMN forwards the service request to the selected NF Service Producer in the target PLMN.

NOTE 1: The NRF in the target PLMN and NF Service Producer cannot authenticate the NF Service Consumer unless the NRF in the target PLMN and the NF Service Producer are provisioned with the required certificates to authenticate the NF Service Consumer.

***** END OF CHANGES