3GPP TSG SA Meeting #108                                                              	SP-250743

Prague, Czech Republic, June 10-13, 2025

Source:	NIST

Title:	Addition to WID on ProSe Phase 4 -- Enhance ProSe to support MANET multicast traffic delivery

Document for:	Discussion

Agenda item:	6.5.2

Work Item / Release:	ProSe_Ph4-ARC / Rel-20

# 1.	Introduction

During SA2 #108 in May 2025, the new WID on ProSe Phase 4, ProSe_Ph4-ARC, was approved after a couple of rounds of revision. Comparing the agreed version S2-2506054 with the original proposal S2-2505674, the key change is the down scope of the objective.

In the original draft WID S2-2505674, WT-1 may cover two bullets:

- WT-1: Enhance ProSe to support efficient MANET multicast for Layer-3 IP Type UE-to-UE multi-hop Relay.

  - Enhance ProSe to support multicast transmission of MANET HELLO and TC messages.

  - Enhance ProSe to support MANET multicast traffic delivery.

However, during the discussion, some companies raised concerns about the second bullet above about the potential amount of efforts required to create and manage the multicast routing tree, and wondered whether TUs will be the increased significantly. Due to the lack of time for online and offline discussion around the end of the meeting week, the WID approved in S2-2506054 removed the second bullet.

After the approval of S2-2506054, the offline discussion regarding the second bullet continued. With further technical clarification given and the justification of the need of the work explained, there seems to be a feasible way forward to add the second bullet back to the scope of the WID without increasing the TU much as outlined in the remaining of this Tdoc.

# 2.	Discussion

The need to enhance ProSe to support MANET multicast traffic delivery is driven by the latency analysis of the MCPTT off-network mode group call operation. As explained in S2-2406197 before, latency of MCPTT group call traffic (both control and voice) delivery is important to the user (e.g., first responders) experience. Therefore, enhancing the ProSe to support MANET multicast traffic delivery would be highly beneficial for optimizing public safety bandwidth usage, and thus great potential to reduce MCPTT group call traffic latency.

Regarding the potential amount of efforts required to create and manage the multicast routing tree, IETF already approved RFC 6621: Simplified Multicast Forwarding (SMF) as a mechanism that provides basic IP multicast for MANET. Other existing multicast protocols may be considered and SA2 doesn’t have to reinvent the wheel. Given that “Enhance ProSe to support multicast transmission of MANET HELLO and TC messages” is already in the scope of this WID, the additional standardization efforts and that groupcast mode is already supported by ProSe, the additional work to support the second bullet “Enhance ProSe to support MANET multicast traffic delivery” is limited, mainly related to Layer-2 ID management, etc.

Therefore, the following NOTE3 has been proposed during the offline discussion to clarify the scope of the second bullet:

NOTE 3: Existing MANET protocols (e.g. Simplified Multicast Forwarding as defined in RFC 6621) will be used for multicast IP traffic routing. The scope of this work will be limited to the mapping between the group ID, IP addresses, and Layer-2 Link IDs.”

# 3.	Conclusion

Based on the above discussion, the following proposal is made:

Proposal: SA is kindly asked to update the objective in the new Rel-20 WID on ProSe Phase 4: ProSe_Ph4-ARC as follows:

- WT-1: Enhance ProSe to support efficient MANET multicast for Layer-3 IP Type UE-to-UE multi-hop Relay.

  - Enhance ProSe to support multicast transmission of MANET HELLO and TC messages.

  - Enhance ProSe to support MANET multicast traffic delivery.

NOTE 1: This WT is subject to coordination with RAN WG2 to determine RAN impact.

NOTE 2: Security aspects will be handled by SA WG3.

NOTE 3: Existing MANET protocols (e.g. Simplified Multicast Forwarding as defined in RFC 6621) will be used for multicast IP traffic routing. The scope of this work will be limited to the mapping between the group ID, IP addresses, and Layer-2 Link IDs.”

The corresponding updated WID is submitted as SP-250744.