3GPP TSG-SA Meeting #108	SP-250754

# Prague, Czech Republic, 10 – 13 June 2025

Source:	Apple

Title:	Proposed revision to SA3 pCR on Update security for data collection for the LMF-based AI/ML positioning

Document for:	Approval

Agenda item:	3.1

Spec:	3GPP TS 33.501

Version:	19.2.0

Work Item:	AIML_CN_SEC

# 1	Discussion

This document proposes a revision to document S3-252409, which is the basis of working agreement WA#67 declared at SA3#122.

There are two aspects considered in S3-252409 related to security for LMF-based AI/ML positioning, namely:

- Data collection for model training and performance monitoring

- Data collection for UE location determination utilizing the trained model (i.e. inference)

For 2), while Apple holds the view that user consent should be also checked for data collection for UE location determination, we respect the majority view that further specification impact for Rel-19 is to be avoided (e.g. with a new user consent purpose). Therefore, we do not intend to make any changes to S3-252409 regarding 2).

For 1), however, S3-252409 specifies that user consent is required, without making any reference to the UE LCS privacy profile, therefore leaving the implication of UE LCS privacy profile up to each company's interpretation. This should be avoided, in our view. The UE LCS privacy profile was specified by SA2 and is documented in TS 23.273 clause 5.4, and therefore Apple's understanding is that the check of the UE LCS privacy profile is in the remit of the SA2. Further, we understand that for this same reason, SA3 did not discuss checking the LCS privacy profile for 1) at SA3#122.

Based on the above, we propose to add a new NOTE clarifying that the check of the UE LCS privacy profile is not precluded, so that SA2 can conclude on this matter, taking SA3's input into account.

# 2   	Proposal

It is proposed to capture the changes to S3-252409 as follows. (The change with respect to S3-252409 is highlighted in yellow.)

Proposed Changes (keeping the revision marks from S3-252409)

* * * Start of Change * * * *

## X.11.2	Security for data collection for the LMF-based AI/ML positioning

User consent for model training as per 6.1.6.3.20 of 29.503 [93] and performance monitoring for LMF-based AI/ML positioning as specified in clause 6.22 of TS 23.273 [86] is required based on regional regulations or operator’s local policy, for which Annex V of the present document applies. For UE location determination utilizing the trained model, LCS privacy profile is sufficient, and Annex V of the present document is not required. The LMF is deemed as enforcement point as specified in clause 6.22.3 of TS 23.273 [86].

NOTE:	The check of LCS privacy profile by LMF for training and performance monitoring for LMF-based AI/ML positioning is not precluded.

* * * End of Change * * * *