# 3GPP TSG-WG SA2 Meeting #169		S2-2505806

# Fukuoka, Japan, May 19 – 23, 2025		(revision of S2-2505582,5354,4086)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0717 | rev | 8 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification and correction on modification of UP connection between UE and LMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the procedure of modification of the User Plane Connection between UE and LMF, there are some aspects to be clarified or corrected. - Since the procedure can also be used to terminate the connection, Step 1a can be used to notify the AMF of the connection termination if the LMF terminates the connection. The AMF releases the context of the UE after receiving the notification. Other steps are not needed in this case. - When performing LMF relocation for the UP connection, the following principles should be applied: 1) The target LMF should be asked first by the source LMF whether it can accept the user plane connection relocation. 2) Based on the target LMF's response, the source LMF notifies the AMF to trigger the target LMF to initiate UP connection establishment. - In SA2 meeting #165, it was agreed to capture the UE identifier for deferred 5GC-MT-LR, where the AMF needs to provide the UE ID to the LMF. The Nlmf_Location_LocationContextTransfer service operation should be updated to cover the UE ID. The message is also used to indicate LMF relocation, the UE Id is also needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Revise the procedure of Modification of User Plane Connection between UE and LMF to make the connection termination logic clearer. - Revise the procedure of Modification of User Plane Connection to support interaction between the source LMF and the target LMF for UP connection relocation negotiation, by using the Nlmf_Location_LocationContextTransfer service operation. - Nlmf_Location_LocationContextTransfer service operation updated to cover UE identifier and additionly support inidiaction of LMF relocation for User Plane Connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Connection termination logic not clear and the principle of modification of User Plane Connection between UE and LMF not align with control plane case. AMF decision would be to force LMF to establish LCS-UPP which is not follow the rule that it shall be UE and LMF decision to establish LCS-UPP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.3, 8.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | SA2 #169: Update to Nlmf_Location_LocationContextTransfer service operation about the conditonal required inputs. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.18.3	Modification of User Plane Connection between UE and LMF

The figure 6.18.3-1 shows how a secure user plane connection between UE and LMF is modified. The flow describes change of LMF but applies also when source and target LMF is the same. The procedure can also be used to terminate the user plane connection to Source LMF not selecting any Target LMF.

2. Trigger LMF relocation

2. Trigger LMF relocation

AMF (Target AMF)

UE

Source LMF

Target LMF

6. Terminate connection with UE

3. Nlmf_Location_UPConfig Request

1a. Nlmf_Location_UPNotify

4.Nlmf_Location_LocationContextTransfer Request

5.Nlmf_Location_LocationContextTransfer Response

4a. Target LMF initiates a User Plane Connection

7. Nlmf_Location_UPConfig Response

0.Trigger LMF UP connection update

AMF (Target AMF)

UE

Source LMF

Target LMF

6. Terminate connection with UE

3. Nlmf_Location_UPConfig Request

1a. Nlmf_Location_UPNotify

4.Nlmf_Location_LocationContextTransfer Request

5.Nlmf_Location_LocationContextTransfer Response

4a. Target LMF initiates a User Plane Connection

7. Nlmf_Location_UPConfig Response

0.Trigger LMF UP connection update

Figure 6.18.3-1: Connection modification between UE and LMFs

0.  [Conditional] The LMF discovers a need to change LMF or decides to terminate the user plane connection, e.g., after receiving an event report via user plane from UE or after receiving an AMF relocation information from a target AMF (target AMF obtains from source AMF the UE LCS-UP context which indicates UE has maintained LCS-UP connection with the source LMF, the target AMF may inform the source LMF about the AMF change using the Nlmf_Locatoion_UPConfig request).

1a.	[Conditional] The LMF sends an Nlmf_Location_UPNotify message that includes an indication of connection change  or termination and if connection change is requested then the message may include a target LMF identification. The address of the AMF was provided to LMF as a "Notification Target Address" in latest Nlmf_Location_UPConfig message or Nlmf_Location_UP Subscribe message. If the LMF is going to terminate the user plane connection, only step 1a and step 6 are performed and other steps are skipped.

If the request is to notify the termination of the user plane connection, the AMF releases the LCS-UP context.

NOTE 1:	The LMF change procedure is independent from the SSC mode of the PDU Session with dedicated DNN used for positioning. For SSC mode 2/3, PSA UPF connection with the LMF can be relocated with UE movement and LMF can discover the need to change LMF to reduce the user plane path latency.

2.	[Conditional] If step 1a includes an indication of connection change, the AMF should use target LMF identification received from source LMF in step 1a or select a target LMF if source LMF does not provide a target LMF identification for user plane connection change.

AMF may perform LMF reselection if a UE established a LCS UP connection with the source LMF moves to a new location (which may be out of serving area of source LMF and in serving area of target LMF) and select the target LMF for the current UE location based on LMF service area, LMF user plane positioning capability information and other information listed in clause 5.1. The target LMF needs to be capable to establish a user plane session for positioning with the UE. After AMF relocation, the target AMF may trigger the LMF reselection and LCS-UP connection modification procedure, e.g., if the source LMF does not perform LMF reselection.

3.	The AMF sends an Nlmf_Location_UPConfig Request towards the source LMF.

If this step is to terminate the LCS-UP connection of the LMF, the message includes an indication of LCS-UP connection termination for the LMF to terminate the user plane connection to the UE.

If this step is to trigger LMF relocation, the message should include the target LMF identifier, which is selected by source LMF in step 0 or by AMF in step 2, to inidicate source LMF to transfer location context(s) to the selected target LMF.

Alternatively, it may include information about AMF reallocation.

NOTE 2:	AMF relocation does not necessarily always cause LMF reselection, the AMF relocation information can keep the LMF informed.

4.   [Conditional] Based on the target LMF identification in the request of step 3, the source LMF should invoke an Nlmf_Location_LocationContextTransfer Request service operation towards the target LMF to provide an indication of user plane connection existing and the current user plane based location context(s) of the UE to the target LMF if there is user plane based periodic and triggered UE location events report context(s).

4a. [Conditional] If target LMF decides to perfom user plane positioning for the UE, Steps 2-8 of figure 6.18.1-1 are performed between AMF (or Target AMF), UE, and Target LMF with addition that UE also stops using the old user plane connection connection to Source LMF.

To guarantee that only LCS-UP connection establishment message from target LMF can be sent to the UE which has already an LCS-UP connection, AMF forwards LCS-UP connection establishment message only from Target LMF to the UE and AMF should reject the user plane connection management message from other LMF with proper failures.

5. The target LMF informs source LMF of the location context transfer operation results.

6.	[Conditional] If the user plane connection to source LMF is still active, the source LMF terminates the connection to the UE.

7.	The Source LMF sends Nlmf_Location_UPConfig Response message to AMF (or Target AMF) to response the request of LCS-UP connection change or confirm connection termination or acknowledge change of AMF. If this step is used for connection termination confirmation, the AMF will release the LCS-UP context after receiving the response message.

# * * * * Second change * * * *

#### 8.3.2.5	Nlmf_Location_LocationContextTransfer service operation

Service operation name: Nlmf_Location_LocationContextTransfer

Description: Transfers user plane connecton exisiting indication and/or location context information for location event reporting for a target UE from the consumer NF.

Input, Required: AMF identity.

Inputs, Conditional Required: UE identifier (SUPI, GPSI), user plane exisiting indication, location context information for location event reporting: Location QoS, Supported GAD shapes, Deferred location type, Deferred location parameters, Notification Target Address, Notification Correlation ID, Embedded event report message.

Input, Optional: Event reporting status, UE location information, LOS/NLOS measurement indication, Indoor/Outdoor indication, UE Positioning Capabilities, Scheduled Location Time, the timestamp of the Location, request for user plane reporting to an LCS Client or AF, cumulative event report timer, maximum number of user plane event reports to an LCS Client or AF.

Output, Required: Success/Failure indication

Output, Optional: None.

See clause 6.4 for an example of usage of this service operation.

# * * * * End of changes * * * *