3GPP SA WG2 Meeting #168	S2-2504028

# Goteborg, Sweden, 07 – 11 April 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5460 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Non-3GPP Device Identifier(s) Information provisioning and policy control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For Non-3GPP Device Identifier(s) Information provisioning procedure defined in clause 4.15.6.15, the following needs further clarification: - "The Target UE identifier(s). A specific UE identified by e.g. GPSI or SUPI." The "identifier(s)" is misleading, and the explanation indicate "A specific UE", there is also no discussion of indicating the Non-3GPP Device Identifier(s) associated with multiple UEs. - It was agreed that the Non-3GPP Device Connection Information is not included in the PDU Session Establishment Request, but there is Non-3GPP Device Connection Information related description in Npcf_SMPolicyControl_Create service operation |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update Target UE in Non-3GPP Device Identifier(s) Information provisioning - Remove Non-3GPP Device Connection Information related description in Npcf_SMPolicyControl_Create service operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missleading description and misalignment of the description for on-3GPP Device Connection Information among description in 23.501, procedure in 23.502 and service operation in 23.502. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.15, 5.2.5.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

4.15.6.15	Provisioning of Non-3GPP Device Identifier Information

This clause describes the procedures to allow an AF to provision QoS information for the non-3GPP device(s) (as defined in clause 5.52 of TS 23.501 [2]) that require differentiated QoS treatment for a UE subscription to the 5G system via NEF. The AF may belong to the operator or to an external party.

To provision QoS information for the non-3GPP device(s) that require differentiated QoS treatment, the steps 1 to 4 of procedure defined in clause 4.15.6.7.2 is performed with the following considerations:

1)	Service Description. Contains an AF-Service-Identifier indicating that the request is for providing Non-3GPP Device Identifier Information.

2)	Service Parameters. Contains the Non-3GPP Device Identifier Information which has the following parameters:

-	Non-3GPP Device Identifier, a generic string, which uniquely identifies a non-3GPP device behind a specific UE subscription.

-	QoS reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

-	Optionally, DNN and S-NSSAI. This may be provided by the AF or may be determined by the NEF based on the AF-Service-Identifier if not provided by the AF.

-	Optionally, Flow description information as specified in clause 6.1.3.6 of TS 23.503 [20]

3)	Target UE. A specific UE identified by e.g. GPSI or SUPI.

4)	The parameters and operation for UE policy delivery is not needed.

The NEF sets the value of Data Subset Identifier to "Non-3GPP Device Identifier Information" in Nudr_DM_Create/Update/Delete service operations when storing/updating/deleting this data. NEF determines to use this data subset based on AF-Service-Identifier.

# *** Next Change ***

5.2.5.4.2	Npcf_SMPolicyControl_Create service operation

Service operation name: Npcf_SMPolicyControl_Create.

Description: The NF Service Consumer can request the creation of a SM Policy Association and provides relevant parameters about the PDU Session to the PCF.

Inputs, Required: SUPI (or PEI in the case of emergency PDU Session without SUPI), PDU Session id, DNN, S-NSSAI and RAT Type.

Inputs, Optional: Information provided by the SMF, such as PDU Session Type, Request Type, Access Type, the IPv4 address and/or IPv6 prefix, PEI, GPSI,  User Location Information, UE Time Zone, Serving Network identifier (PLMN ID, or PLMN ID and NID, see clause 5.34 of TS 23.501 [2]), Charging Characteristics information, Session-AMBR, subscribed default QoS information (5QI, 5QI Priority Level, ARP), UE support of reflective QoS (see TS 23.501 [2], clause 5.7.5.1), Number of supported packet filters for signalled QoS rules for the PDU Session (see TS 23.501 [2], clause 5.7.1.4), UE's capability to support (S)RTP Multiplexed Media Identification Information in IP Packet Filters (see clause 4.3.2.2.1), 3GPP PS Data Off status, Trace Requirements and Internal Group Identifier (see clause 5.9.7 of TS 23.501 [2]), DN Authorization Profile Index, DN authorized Session AMBR, Framed Route information (as defined in Table 5.2.3.3.1-1), MA PDU Request indication, MA PDU Network-Upgrade Allowed indication, ATSSS capabilities of the MA PDU Session, QoS constraints from the VPLMN (as defined in clause 5.7.1.11 of TS 23.501 [2]), Satellite backhaul category, list of NWDAF instance Ids (used by AMF, SMF, UPF) and corresponding Analytics ID(s), PVS IP address(es) and/or PVS FQDN(s) and Onboarding Indication in the case of ON-SNPN (see clause 5.30.2.10.4.2 of TS 23.501 [2]), URSP rule enforcement that including Connection Capability, PCF binding information (address(es) of PCF for UE, instance id of PCF for UE), HR-SBO support indication (see clause 6.2.1.2 of TS 23.503 [20]), Alternative S-NSSAI (see clause 5.15.19 of TS 23.501 [2]), URSP delivery in EPS support indication, Local Offloading Management indication.

NOTE 1:	If SMF receives the DN authorized Session AMBR from the DN-AAA at PDU Session establishment, it includes the DN authorized Session AMBR within the Session-AMBR, instead of the subscribed Session-AMBR received from the UDM, in the request.

NOTE 2:	It is up to stage 3 to determine whether the corresponding supportedFeature in Npcf_SMPolicyControl can be reused as URSP delivery in EPS support indication.

NOTE 3:	When Local Offloading Management is allowed, SMF provides Local Offloading Management indication to PCF.

W-5GAN specific PDU Session information provided by the SMF is specified in TS 23.316 [53].

Outputs, Required: SM Policy Association ID defined in TS 29.512 [57]. Success or Failure.

Outputs, Optional: Policy information for the PDU Session as defined in TS 23.503 [20] and Policy Control Request Trigger(s) of SM Policy Association as defined in clause 6.1.3.5 of TS 23.503 [20].

See clause 5.8.2.2 of TS 23.501 [2] for allocation of IPv4 address and IPv6 prefix. The IPv6 prefix length is /64, or is shorter than /64 when Prefix Delegation applies.

See clause 4.16.4 for the detail usage of this service operation.

See clauses 4.22.2.1 and 4.22.3 for detailed usage of this service operation for ATSSS.

# *** End of Changes ***