3GPP TSG-SA2 Meeting #168	S2-2503000

# Goteborg, Sweden, April 7 - April 11, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1530 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding a Reference to the Home Routed Roaming PDU Session Modification Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc., LG Electronics, Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The clause on Policy control for non-3GPP devices connecting behind a UE refers to clause 4.3.3.2 of TS 23.502. Clause 4.3.3.2 of TS 23.502 is the PDU Session Modification procedure for non-roaming and roaming with local breakout. There should also be a reference to clause 4.3.3.3 to cover the home-routed roaming case, |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a reference to clause 4.3.3.3 of TS 23.502. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description would give the appearance that the UIA_ARC feature will not work in the home routed roaming scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.31 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.1.3.31	Policy control for non-3GPP devices connecting behind a UE

The support of QoS differentiation for non-3GPP devices connecting behind a UE using a Non-3GPP Device Identifier is specified in clause 5.52 of TS 23.501 [2].

The SMF may receive from the UE the Non-3GPP Device Identifier and the corresponding user plane address if the UE initiated the PDU session modification procedure, as specified in clauses 4.3.3.2 and 4.3.3.3 of TS 23.502 [3]. The SMF sends the Non-3GPP Device Identifier and the user plane address of the non-3GPP device to the PCF. At reception of the Non-3GPP Device Identifier and user plane address, the PCF creates PCC rule(s) considering the Non-3GPP Device Identifier. The PCF determines QoS parameters in the PCC rule(s) based on the Non-3GPP Device Identifier Information retrieved from the UDR and/or based on operator policy. The PCF installs the PCC rules at the SMF to enable QoS handling of the traffic to/from the non-3GPP device behind the UE.

If the Non-3GPP Device Identifier is not present within the Non-3GPP Device Identifier Information as specified in clause 5.2.12.2.1 of TS 23.502 [3] for the UE, then the PCF indicates to the SMF that the corresponding Non-3GPP Device Identifier is not available for the UE. In case of PDU Session Modification (i.e. SMF contacted PCF via SM policy association modification), the PCF shall reject SM Policy Association Modification and based on the rejection received, SMF shall reject the PDU Session Modification with a cause code to notify the UE that the Non-3GPP Device Identifier is not available for the UE.

If the Non-3GPP Device Identifier is removed from the UE subscription, the UDR notifies to the PCF via Nudr_DM_Notify service of removal of N3GPP device (as defined in clauses 4.15.6.7.2 and 4.15.6.15 of TS 23.502 [3]). If there is PCC rules associated to the traffic associated to the removed Non-3GPP Device Identifier in a PDU session, the PCF shall remove the PCC rules and start a PCF initiated PDU session modification including a cause indicating that the Non-3GPP Device Identifier is not available.

# * * * * End of changes * * * *