3GPP TSG-WG SA2 Meeting #168	            S2-2504168

Gotenburg, Sweden, April 7-11, 2025                                                    (was S2-2503940)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6209 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU Set based handling without PDU Set QoS parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is a NOTE : Based on the PDU Set Based Handling Support Indication from the target NG-RAN, the SMF can update the QoS profile and, if applicable, Alternative QoS Profiles as defined in clauses 5.7.1.2 and 5.7.1.2a to include the PDU Set QoS parameters respectively. However, it should not apply to PDU Set based handling without provisioned PDU Set QoS paramters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified the NOTE is applied only when the PDU Set QoS paramters have been provisioned. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstood the PDU Set based handling feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

5.37.5.3	Non-homogenous support of PDU set based handling in NG-RAN

The SMF, by sending PDU Set QoS parameters (as described in clause 5.7.7.1) or a DL PDU Set Information Marking Support Indication (as described in clause 5.37.5.1) to the NG-RAN, requests the NG-RAN to activate PDU Set handling for a given QoS Flow and the NG-RAN provides the SMF with a PDU Set Based Handling Support Indication if the PDU Set based handling is supported. Based on this indication, SMF may activate the PDU Set identification and marking in the PSA UPF as described in clause 5.37.5.2.

During mobility procedures that result in the change of NG-RAN, the target NG-RAN provides to the SMF a PDU Set Based Handling Support Indication if it supports PDU Set based handling, as specified in TS 38.413 [34]. Based on the target NG-RAN indication, the SMF may, upon completion of the mobility procedure, initiate the PDU Session modification procedure to provide PDU Set QoS parameters or the DL PDU Set Information Marking Support Indication to NG-RAN and configure the PSA UPF to activate the PDU Set identification and marking. If the PDU Set Based Handling Support Indication is not received from the target NG-RAN and PDU Set identification and marking is active in the PSA UPF, the SMF may deactivate it.

In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header, it shall start doing so from a complete PDU Set.

NOTE:	Based on the PDU Set QoS parameters provisioned in PCC rule and the PDU Set Based Handling Support Indication from the target NG-RAN for the QoS Flow, the SMF can update the QoS profile and, if applicable, Alternative QoS Profiles as defined in clauses 5.7.1.2 and 5.7.1.2a, respectively, to include the PDU Set QoS parameters.

********** End of Changes **********