3GPP TSG-WG SA2 Meeting #168	S2-2504420

# Gothenburg, Sweden, 7th Apr – 11th Apr, 2025	(revision of S2-2504169, 3942, 3525)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1548 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for QoS Notification Control with direction information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The event reporting clauses do not contain a description about the forwarding of the direction information. It is also required to move the PDU Set QoS related statements from clause 6.1.3.22 to clause 6.1.3.27.4. Furthermore, the details of the PCF actions are clarified and appropriate terminology is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description about the forwarding of the direction information is added to the event reporting clauses. The PDU Set QoS related statements are moved from clause 6.1.3.22 to clause 6.1.3.27.4.The details of the PCF actions are clarified and appropriate terminology. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete description for the forwarding of the direction information. Misleading description of PDU Set QoS parameters in the Alternative QoS Parameter sets. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.5, 6.1.3.18, 6.1.3.22, 6.1.3.27.4, 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.1.3.5	Policy Control Request Triggers relevant for SMF

The Policy Control Request Triggers relevant for SMF define the conditions when the SMF shall interact again with PCF after a PDU Session establishment as defined in the Session Management Policy Establishment and Session Management Policy Modification procedure as defined in TS 23.502 [3].

The PCR triggers are not applicable any longer at termination of the SM Policy Association.

The access independent Policy Control Request Triggers relevant for SMF are listed in table 6.1.3.5-1.

The differences with table 6.2 and table A.4.3-2 in TS 23.203 [4] are shown, either "none" means that the parameter applies in 5GS or "removed" meaning that the parameter does not apply in 5GS, this is due to the lack of support in the 5GS for this feature or "modified" meaning that the parameter applies with some modifications defined in the parameter.

Table 6.1.3.5-1: Access independent Policy Control Request Triggers relevant for SMF

| Policy Control Request Trigger | Description | Difference compared with table 6.2 and table A.4.3-2 in TS 23.203 [4] | Conditions for reporting | Motivation |
| --- | --- | --- | --- | --- |
| PLMN change | The UE has moved to another operators' domain. | None | PCF |  |
| QoS change | The QoS parameters of the QoS Flow has changed. | Removed |  | Only applicable when binding of bearers was done in PCRF. |
| QoS change exceeding authorization | The QoS parameters of the QoS Flow has changed and exceeds the authorized QoS. | Removed |  | Only applicable when binding of bearers was done in PCRF. |
| Traffic mapping information change | The traffic mapping information of the QoS profile has changed. | Removed |  | Only applicable when binding of bearers was done in PCRF. |
| Resource modification request | A request for resource modification has been received by the SMF. | None | SMF always reports to PCF |  |
| Routing information change | The IP flow mobility routing information has changed (when IP flow mobility as specified in TS 23.261 [11] applies) or the PCEF has received Routing Rules from the UE (when NBIFOM as specified in TS 23.161 [10] applies). | Removed |  | Not in 5GS yet. |
| Change in Access Type (NOTE 8) (NOTE 11) | The Access Type or RAT Type or both Access Type and RAT Type of the PDU Session changed. | None | PCF |  |
| EPS Fallback | EPS fallback is initiated | Added | PCF |  |
| Loss/recovery of transmission resources | The Access type transmission resources are no longer usable/again usable. | Removed |  | Not in 5GS yet. |
| Location change (serving cell) (NOTE 6) | The serving cell of the UE has changed. | None | PCF |  |
| Location change (serving area) (NOTE 2) | The serving area of the UE has changed. | None | PCF |  |
| Location change (serving CN node) (NOTE 3) | The serving core network node of the UE has changed. | None | PCF |  |
| Change of UE presence in Presence Reporting Area (see NOTE 1) | The UE is entering/leaving a Presence Reporting Area. | None | PCF | Only applicable to PCF |
| Out of credit | Credit is no longer available. | None | PCF |  |
| Reallocation of credit | Credit has been reallocated after the former Out of credit indication. | Added | PCF |  |
| Enforced PCC rule request | SMF is performing a PCC rules request as instructed by the PCF. | None | PCF |  |
| Enforced ADC rule request | TDF is performing an ADC rules request as instructed by the PCRF. | Removed |  | ADC Rules are not applicable. |
| UE IP address change | A UE IP address has been allocated/released. | None | SMF always reports allocated or released UE IP addresses |  |
| UE MAC address change | A new UE MAC address is detected or a used UE MAC address is inactive for a specific period. | New | PCF |  |
| Access Network Charging Correlation Information | Access Network Charging Correlation Information has been assigned. | None | PCF |  |
| Usage report (NOTE 4) | The PDU Session or the Monitoring key specific resources consumed by a UE either reached the threshold or needs to be reported for other reasons. | None | PCF |  |
| Start of application traffic detection and Stop of application traffic detection (NOTE 5) | The start or the stop of application traffic has been detected. | None | PCF |  |
| SRVCC CS to PS handover | A CS to PS handover has been detected. | Removed |  | No support in 5GS yet |
| Access Network Information report | Access information as specified in the Access Network Information Reporting part of a PCC rule. | None | PCF |  |
| Credit management session failure | Transient/Permanent failure as specified by the CHF. | None | PCF |  |
| Addition / removal of an access to an IP-CAN session | The PCEF reports when an access is added or removed. | Removed |  | No support in 5GS yet |
| Change of usability of an access | The PCEF reports that an access becomes unusable or usable again. | Removed |  | No support in 5GS yet |
| 3GPP PS Data Off status change | The SMF reports when the 3GPP PS Data Off status changes. | None | SMF always reports to PCF |  |
| Session AMBR change | The Session-AMBR has changed. | Added | SMF always reports to PCF |  |
| Default QoS change | The subscribed QoS has changed. | Added | SMF always reports to PCF |  |
| Removal of PCC rule | The SMF reports when the PCC rule is removed. | Added | SMF always reports to PCF |  |
| Successful resource allocation | The SMF reports to the PCF that the resources for a PCC rule have been successfully allocated. | Added | PCF |  |
| GFBR of the QoS Flow can no longer (or can again) be guaranteed | The SMF notifies the PCF when receiving notifications from RAN that GFBR of the QoS Flow can no longer (or can again) be guaranteed. | Added |  |  |
| UE resumed from suspend state | The SMF reports to the PCF when it detects that the UE is resumed from suspend state. | None | PCF | Only applicable to EPC IWK |
| Change of DN Authorization Profile Index | The DN Authorization Profile Index received from DN-AAA has changed. | Added | SMF always reports to PCF |  |
| 5GS Bridge/Router information available | SMF has detected new 5GS Bridge/Router information, which may contain, user-plane Node ID, UE-DS-TT residence time and Ethernet port (port number and MAC address) or IP address for the PDU Session, MTU size for IPv4 or MTU size for IPv6 and/or PMIC and/or UMIC. | Added | PCF |  |
| QoS Monitoring | The SMF notifies the PCF of the QoS Monitoring reports (as defined in clause 5.45 of TS 23.501 [2]). | Added | PCF |  |
| DDN Failure event Subscription with Traffic Descriptor | The SMF requests PCF to provide or remove policies if it received an event subscription or cancellation for DDN Failure event including traffic descriptors. The SMF provides the traffic descriptors to the PCF for policy evaluation. | Added | PCF |  |
| DDD Status event Subscription with Traffic Descriptor | The SMF requests PCF to provide or remove policies if it received an event subscription or cancellation for DDD Status event including traffic descriptors. The SMF provides the traffic descriptors and the requested type(s) of notifications (notifications about downlink packets being buffered, and/or discarded) to the PCF for policy evaluation. | Added | PCF |  |
| QoS constraints change | The QoS constraints in the VPLMN have been provided or changed. | Added | SMF always reports to PCF |  |
| Satellite backhaul category change | The backhaul is changed between different types of satellite backhaul, or between satellite backhaul and non-satellite backhaul. | Added | PCF |  |
| NWDAF info change | The NWDAF instance IDs used for the PDU Session or associated Analytics IDs used for the PDU Session and available in the SMF have changed. | Added | PCF |  |
| Request for reporting the PCF binding information (NOTE 9) | The SMF reports the updated PCF binding information of the PCF for the UE. | Added | SMF always reports to PCF |  |
| Notification on BAT offset | The SMF reports the BAT offset and optionally the adjusted periodicity provided by the RAN. | Added | PCF |  |
| UE reporting Connection Capabilities from associated URSP rule (NOTE 10) | The SMF has received from the UE reporting from an associated URSP rule via a PDU Session establishment or PDU Session modification request. | Added | SMF always reports to PCF |  |
| UE Policy Container received or delivery failure for UE Policy Container delivery via EPS | The SMF reports that a UE Policy Container has been received from the UE or a delivery failure for UE Policy Container delivery via EPS. | Added | SMF always reports to PCF |  |
| Change of HR-SBO support indication | The HR-SBO support indication has changed. | Added |  |  |
| Network Slice Replacement | The SMF reports the event of change between S-NSSAI and Alternative S-NSSAI to PCF when the SMF determines that the PDU Session and SM Policy Association can be retained. The SMF provides Alternative S-NSSAI when the PDU Session is transferred from S-NSSAI to Alternative S-NSSAI. | Added | PCF |  |
| ECN marking for L4S can no longer (or can again) be performed | The SMF notifies the PCF when ECN marking for L4S can no longer (or can again) be performed. | Added | PCF |  |
| QoS Monitoring can no longer (or can again) be performed | The SMF notifies the PCF when QoS Monitoring can no longer (or can again) be performed. | Added | PCF |  |
| UE reachability status change | The SMF reports to the PCF when it receives an indication of a change of the UE reachability status. | Added | PCF |  |
| Non-3GPP Device Identifier received | The SMF reports to the PCF when a Non-3GPP Device Identifier is added to the PDU Session. | Added | SMF always reports to PCF |  |
| NOTE 1: The maximum number of PRA(s) per UE per PDU Session is configured in the PCF. The PCF may have independent configuration of the maximum number for Core Network pre-configured PRAs and UE-dedicated PRAs. The exact number(s) should be determined by operator in deployment. NOTE 2: This trigger reports change of Tracking Area in both 5GS and EPC interworking, or reports change of Routing Area for GERAN/UTRAN access (see Annex G of TS 23.502 [3]). NOTE 3: This trigger reports change of AMF in 5GC, change between ePDG and Serving GW in EPC, change between Serving GWs in EPC, change between EPC and 5GC, change between Serving Gateway and SGSN in GERAN/UTRAN from/to E-UTRAN mobility, or change between SGSNs in the case of GERAN/UTRAN access. In HR roaming case, if the AMF change is unknown by the H-SMF, then the AMF change is not reported. NOTE 4: Usage is defined as either volume or time of user plane traffic. NOTE 5: The start and stop of application traffic detection are separate event triggers, but received under the same subscription from the PCF. NOTE 6: Location change of serving cell can increase signalling load on multiple interfaces. Hence it is recommended that any such serving cell changes only applied for a limited number of subscribers avoiding extra signalling load. It also is applicable for GERAN/UTRAN access. NOTE 7: Void. NOTE 8: For 3GPP access the RAT type may refer to NR, E-UTRAN, and, when the SMF+PGW-C enhancements to support GERAN/UTRAN access via Gn/Gp interface as specified in Annex L of TS 23.501 [2] apply, to UTRAN or GERAN. For MA PDU Session this trigger reports the current used Access Type(s) and RAT type(s) upon any change of Access Type and RAT type. NOTE 9: The PCF for the PDU Session knows the change of the PCF for the UE by this Policy Control Request Trigger based on the associated binding information of and notifies the PCF for the UE as described in clause 6.1.3.18. NOTE 10: See clause 6.6.2.4. NOTE 11: Multiple triggers are described in TS 29.512 [44] for this event. |  |  |  |  |

NOTE 1:	In the following description of the access independent Policy Control Request Triggers relevant for SMF, the term trigger is used instead of Policy Control Request Trigger where appropriate.

When the EPS Fallback trigger is armed by the PCF, the SMF shall report the event to the PCF when a QoS Flow with 5QI=1 is rejected due to EPS Fallback.

When the Location change trigger is armed, the SMF shall subscribe to the AMF for reports on changes in location to the level indicated by the trigger. If credit-authorization triggers and Policy Control Request Triggers require different levels of reporting of location change for a single UE, the location to be reported should be changed to the highest level of detail required. However, there should be no request being triggered for PCC rules update to the PCF if the report received is more detailed than requested by the PCF.

NOTE 2:	The access network may be configured to report location changes only when transmission resources are established in the radio access network.

The Resource modification request trigger shall trigger the PCF interaction for all resource modification requests not tied to a specific QoS Flow received by SMF. The resource modification request received by SMF may include request for guaranteed bit rate changes for a traffic aggregate and/or the association/disassociation of the traffic aggregate with a 5QI and/or a modification of the traffic aggregate.

The enforced PCC rule request trigger shall trigger a SMF interaction to request PCC rules from the PCF for an established PDU Session. This SMF interaction shall take place within the Revalidation time limit set by the PCF in the PDU Session related policy information. The SMF reports that the enforced PCC rule request trigger was met and the enforced PCC Rules.

NOTE 3:	The enforced PCC rule request trigger can be used to avoid signalling overload situations e.g. due to time of day based PCC rule changes.

The UE IP address change trigger shall trigger a SMF interaction with the PCF if a UE IP address is allocated or released during the lifetime of the PDU Session. The SMF reports that the UE IP address change trigger was met and the new or released UE IP address.

The UE MAC address change trigger shall trigger a SMF interaction with the PCF if a new UE MAC address is detected or a used UE MAC address is inactive for a specific period during the lifetime of the Ethernet type PDU Session. The SMF reports that the UE MAC address change trigger was met and the new or released UE MAC address.

NOTE 4:	The SMF instructs the UPF to detect new UE MAC addresses or used UE MAC address is inactive for a specific period as described in TS 23.501 [2].

The Access Network Charging Correlation Information trigger shall trigger the SMF to report the assigned access network charging identifier for the PCC rules that are accompanied with a request for this trigger at activation. The SMF reports that the Access Network Charging Correlation Information trigger was met and the Access Network Charging Correlation Information.

If the Usage report trigger is set and the volume or the time thresholds, earlier provided by the PCF, are reached, the SMF shall report this situation to the PCF. If both volume and time thresholds were provided and the thresholds, for one of the measurements, are reached, the SMF shall report this situation to the PCF and the accumulated usage since last report shall be reported for both measurements.

The management of the Presence Reporting Area (PRA) functionality enables the PCF to subscribe to reporting change of UE presence in a particular Presence Reporting Area.

NOTE 5:	PCF decides whether to subscribe to AMF or to SMF for those triggers that are present in both tables 6.1.2.5-2 and 6.1.3.5-1. If the Change of UE presence in Presence Reporting Area trigger is available on both AMF and SMF, PCF should not subscribe to both AMF and SMF simultaneously.

Upon every interaction with the SMF, the PCF may activate / deactivate reporting changes of UE presence in Presence Reporting Area by setting / unsetting the corresponding trigger by providing the PRA Identifier(s) and additionally the list(s) of elements comprising the Presence Reporting Area for UE-dedicated Presence Reporting Area(s).

The SMF shall subscribe to the UE Location Change notification from the AMF by providing an area of interest containing the PRA Identifier(s) and additionally the list(s) of elements provided by the PCF as specified in clause 5.6.11 of TS 23.501 [2] and in clause 5.2.2.3.1 of TS 23.502 [3].

When the Change of UE presence in Presence Reporting Area trigger is armed, i.e. when the PCF subscribes to reporting change of UE presence in a particular Presence Reporting Area and the reporting change of UE presence in this Presence Reporting Area was not activated before, the SMF subscribes to the UE mobility event notification service provided by the AMF for reporting of UE presence in Area of Interest which reports when the UE enters or leaves a Presence Reporting Area (an initial report is received when the PDU Session specific procedure is activated). The SMF reports the PRA Identifier(s) and indication(s) whether the UE is inside or outside the Presence Reporting Area(s), and indication(s) if the corresponding Presence Reporting Area(s) is set to inactive by the AMF to the PCF.

NOTE 6:	The serving node (i.e. AMF in 5GC or MME in EPC/EUTRAN) can activate the reporting for the PRAs which are inactive as described in the TS 23.501 [2].

When PCF modifies the list of PRA id(s) to change of UE presence in Presence Reporting Area for a particular Presence Reporting Area(s), the SMF removes or adds the PRA id(s) provided in the UE mobility event notification service provided by AMF for reporting of UE presence in Area Of Interest. When the PCF unsubscribes to reporting change of UE presence in Presence reporting Area, the SMF unsubscribes to the UE mobility event notification service provided by AMF for reporting of UE presence in Area Of Interest, unless subscriptions to AMF remains due to other triggers.

The SMF stores PCF subscription to reporting for changes of UE presence in Presence Reporting Area and notifies the PCF with the PRA Identifier(s) and indication(s) whether the UE is inside or outside the Presence Reporting Area(s) based on UE location change notification in area of interest received from the serving node according to the corresponding subscription.

NOTE 7:	The SMF can also be triggered by the CHF to subscribe to notification of UE presence in PRA from the AMF, and notifies the CHF when receiving reporting of UE presence in PRA from the AMF, referring to TS 32.291 [20].

If PCF is configured with a PRA identifier referring to the list of PRA Identifier(s) within a Set of Core Network predefined Presence Reporting Areas as defined in TS 23.501 [2], it activates the reporting of UE entering/leaving each individual PRA in the Set of Core Network predefined Presence Reporting Areas, without providing the complete set of individual PRAs.

When a PRA set identified by a PRA Identifier was subscribed to report changes of UE presence in Presence Reporting Area by the PCF, the SMF additionally receives the PRA Identifier of the PRA set from the AMF, along with the individual PRA Identifier(s) belonging to the PRA set and indication(s) of whether the UE is inside or outside the individual Presence Reporting Area(s), as described in TS 23.501 [2].

When the Out of credit detection trigger is set, the SMF shall inform the PCF about the PCC rules for which credit is no longer available together with the applied termination action.

When the Reallocation of credit detection trigger is set, the SMF shall inform the PCF about the PCC rules for which credit has been reallocated after credit was no longer available and the termination action was applied.

The Start of application traffic detection and Stop of application traffic detection triggers shall trigger an interaction with PCF once the requested application traffic is detected (i.e. Start of application traffic detection) or the end of the requested application traffic is detected (i.e. Stop of application traffic detection) unless it is requested within a specific PCC Rule to mute such interaction for solicited application reporting or unconditionally in the case of unsolicited application reporting. The application identifier and service data flow descriptions, if deducible, shall also be included in the report. An application instance identifier shall be included in the report both for Start and for Stop of application traffic detection when service data flow descriptions are deducible. This is done to unambiguously match the Start and the Stop events.

At PCC rule activation, modification and deactivation the SMF shall send, as specified in the PCC rule, the User Location Report and/or UE Timezone Report and/or Satellite Identifier Report to the PCF.

NOTE 8:	At PCC rule deactivation the User Location Report includes information on when the UE was last known to be in that location.

If the trigger for Access Network Information reporting is set, the SMF shall check the need for access network information reporting after successful installation/modification or removal of a PCC rule or upon termination of the PDU Session. The SMF shall check the Access Network Information report parameters (User Location Report, UE Timezone Report, Satellite Identifier Report) of the PCC rules and report the access network information to the PCF. The SMF shall not report any subsequent access network information updates received from the PDU Session without any previous updates of related PCC rule unless the associated QoS Flow or PDU Session has been released.

If the SMF receives a request to install/modify or remove a PCC rule with Access Network Information report parameters (User Location Report, UE Timezone Report, Satellite Identifier Report) set the SMF shall initiate a PDU Session modification to retrieve the current access network information of the UE and forward it to the PCF afterwards.

If the Access Network Information report parameter for the User Location Report is set and the user location (e.g. cell) is not available to the SMF, the SMF shall provide the serving PLMN identifier to the PCF.

The Credit management session failure trigger shall trigger a SMF interaction with the PCF to inform about a credit management session failure and to indicate the failure reason, and the affected PCC rules.

NOTE 9:	As a result, the PCF may decide about e.g. PDU Session termination, perform gating of services, switch to offline charging, change rating group, etc.

NOTE 10:	The Credit management session failure trigger applies to situations wherein the PDU Session is not terminated by the SMF due to the credit management session failure.

The default QoS change triggers shall trigger the PCF interaction for all changes in the default QoS data received in SMF from the UDM.

The Session AMBR change trigger shall trigger the SMF to provide the Session-AMBR to the PCF containing the DN authorised Session AMBR if received from the DN-AAA, or the Subscribed Session-AMBR received from the UDM as described in clause 5.6.6 of TS 23.501 [2].

The default QoS change trigger reports a change in the default 5QI/ARP retrieved by SMF from UDM, as explained in clause 5.7.2.7 of TS 23.501 [2].

If the PCC Rules bound to a QoS Flow are removed when the corresponding QoS Flow is removed or the PCC rules are failed to be enforced, the SMF shall report this situation to the PCF and may provide the reason for failure, if the reason for failure is that the UE is temporarily unreachable, the SMF may also provide the maximum waiting time to the PCF, in this case the PCF does not provide the same or updated PCC Rules for the established PDU Session before the maximum waiting time expires, the PCF may also subscribe to PCRT on change of UE reachability. In other failure scenarios, the PCF may then provide the same or updated PCC rules for the established PDU Session.

NOTE 11:	The PCF can decide to provide PCC Rules when the maximum waiting time expires or send them later depending on implementation.

If the trigger for successful resource allocation is set and the PCF has also provided an indication that a specific PCC rule is subject to this trigger, the SMF shall report to the PCF when the resources associated to this PCC rule have been successfully allocated. The SMF shall report resource allocation failure always to the PCF, independently of this trigger. If the SMF reports resource allocation failure for a PCC rule containing MA PDU Session Control information with Redundant as Steering Mode (see clause 5.32.4 of TS 23.501 [2]), the SMF shall also indicate the respective Access Type.

If the GFBR of the QoS Flow can no longer (or can again) be guaranteed trigger is armed, the SMF shall check the need for reporting to the PCF when the SMF receives an explicit notification from (R)AN indicating that GFBR of the QoS Flow can no longer (or can again) be guaranteed or when the condition described in clause 5.7.2.4 of TS 23.501 [2] is met during the handover. The SMF shall report that GFBR of the QoS Flow can no longer (or can again) be guaranteed accordingly to the PCF for those PCC rules which are bound to the affected QoS Flow and have the QoS Notification Control (QNC) parameter set. If the SMF receives a direction information together with the notification from (R)AN (as described in clause 5.7.2.4.1a of TS 23.501 [2] for the case of a GBR QoS Flow with a QoS profile containing PDU Set QoS parameters), the SMF shall also forward the direction information to the PCF. If other information is received together with the notification from NG-RAN (see clause 5.7.2.4 of TS 23.501 [2]), the SMF shall also provide to the PCF the reference to the Alternative QoS parameter set corresponding to the Alternative QoS Profile referenced by NG-RAN. If NG-RAN has indicated that the lowest priority Alternative QoS Profile cannot be fulfilled, the SMF shall indicate to the PCF that the lowest priority Alternative QoS parameter set cannot be fulfilled.

In an interworking scenario between 5GS and EPC/E-UTRAN, as explained in clause 4.3 of TS 23.501 [2], the PCF may subscribe via the SMF also to the Policy Control Request Triggers described in clause 6.1.2.5 when the UE is served by the EPC/E-UTRAN.

The change of DN Authorization Profile Index shall trigger a SMF interaction to send DN Authorization Profile Index to retrieve a list of PCC Rules (as defined in clause 6.3) and/or PDU Session related policy (as defined in clause 6.4) for an established PDU Session.

If the trigger for 5GS Bridge/Router information available is armed, the SMF shall report the 5GS Bridge/Router information when the SMF has determined or updated the 5GS Bridge/Router information, e.g. when SMF has detected an Ethernet port which supports exchange of Ethernet Port Management Information Containers or received User plane node Management Information Container or Port Management Information Container. If a new manageable Ethernet DS-TT port is detected, the SMF provides User plane node ID, the port number and optionally MAC address of the related port of the related PDU Session to the PCF. If the SMF has received UE-DS-TT Residence Time then the SMF also provides UE-DS-TT Residence Time to the PCF. If the SMF has received the User plane node Management Information Container from NW-TT or Port Management Information Container from NW-TT or DS-TT, the SMF also provides User plane node Management Information Container or Port Management Information Container and related port number to the PCF. In the case of Deterministic Networking, the SMF may also provide the MTU size for IPv4 or the MTU size for IPv6.

When the QoS Monitoring trigger is set, the SMF shall, upon receiving the QoS Monitoring report from the UPF, send the measurement report to the PCF.

If the Policy Control Request Trigger "DDN Failure event subscription with Traffic Descriptor" or "DDD Status event subscription with Traffic Descriptor" is set, the SMF shall request policies if it received a subscription or cancellation of notifications for availability after DDN Failure event with traffic descriptor or DDD Status event with traffic descriptor, respectively. The SMF indicates whether it is a subscription or cancellation event and provides the received Traffic Descriptor as well as the requested type(s) of notifications (notifications about downlink packets being buffered, and/or discarded) to the PCF. When the SMF indicates a subscription event, the PCF checks whether an installed PCC rule exists for the received Traffic Descriptor and if so, the PCF sets the Downlink Data Notification Control information of that PCC rule according to the requested type(s) of notifications. Otherwise, the PCF provides a new PCC Rule with the received Traffic Descriptor in the SDF Template, the Downlink Data Notification Control information set according to the requested type(s) of notifications and other PCC Rule information set to the same values as in the existing PCC rule that previously matched the traffic. When the new PCC rule has to be bound to the QoS Flow associated with the default QoS rules, the PCF sets the "Bind to QoS Flow associated with the default QoS rule" parameter. From now on, the PCF needs to keep the PCC rule for the DDD event detection fully synchronized with the existing PCC rule that previously matched the traffic for all other policy and charging control settings to ensure the same user experience and traffic treatment according to the operator policy. When the SMF indicates a cancellation event, the PCF removes the Downlink Data Notification Control information in the installed PCC Rule or removes the PCC Rule if a new PCC rule has been provided during the subscription event and this PCC rule is no longer necessary for any other policy enforcement.

NOTE 12:	Downlink Data Delivery (DDD) status event and DDN Failure event are specified in clause 4.15.3 of TS 23.502 [3].

The QoS constraints change trigger shall trigger a SMF interaction with the PCF if QoS constraints are received by the SMF during the lifetime of the PDU Session. The SMF reports that the QoS constraints change trigger was met and the new QoS constraints.

When the Satellite backhaul category change trigger is armed, the SMF reports to the PCF that the Satellite backhaul category change was met and the new Satellite backhaul category (including satellite backhaul is no longer used) when it becomes aware that there is a change of the backhaul which is used for the PDU Session between different types of satellite backhaul, or between satellite backhaul and a non-satellite backhaul. The SMF determines whether or not a satellite backhaul is used and whether there is a change of backhaul based on signalling from the AMF as specified in TS 23.501 [2].

NOTE 13:	As specified in clause 5.43.4 of TS 23.501 [2], Satellite backhaul category refers to the type of the satellite (or non-satellite) used in the backhaul. Only a single backhaul category can be indicated.

The NWDAF info change trigger shall trigger the SMF to interact with the PCF when the list of NWDAF Instance IDs used for the PDU Session or associated Analytics IDs used for the PDU Session are changed in the SMF.

The Request for reporting the PCF binding information indicates to the SMF to report to the PCF both the updated PCF binding information (of the PCF for the UE), if it is received from the AMF together with Request for notification of SM Policy Association establishment and termination if both are received from the AMF.

When the Notification on BAT offset trigger is set, the SMF shall, upon receiving a BAT offset and optionally an adjusted periodicity from the RAN (in a notification that GFBR of the QoS Flow can no longer be guaranteed as defined in clause 5.27.2.5.3 of TS 23.501 [2]), report the BAT offset and optionally the adjusted periodicity to the PCF for the PCC rule which is bound to the QoS Flow for which the notification from RAN was received.

The UE reporting Connection Capabilities from associated URSP rule trigger indicates to the SMF that when a UE includes Connection Capabilities in the PDU Session Establishment Request or PDU Session Modification Request, the SMF shall forward this information to the PCF as described in clause 6.6.2.4.

The UE Policy Container received or delivery failure for UE Policy Container delivery via EPS trigger shall trigger a SMF interaction with the PCF, if a UE Policy Container is received from the UE via EPS or in case of a delivery failure for UE Policy Container delivery via EPS (with appropriate reason, e.g. UE is not reachable), as described in clause 4.11.0a.2a.10 of TS 23.502 [3].

NOTE 14:	The UE Policy Container can include a list of provisioned PSIs and/or UE capabilities (e.g. indication of supporting URSP rules over EPS) or the result of the delivery of the UE Policy Container as well as the result of processing the content of the UE Policy Container by the UE.

When the Change of HR-SBO support indication trigger is armed, the H-SMF reports to the H-PCF that the HR-SBO support indication change was met. The H-SMF determines whether there is a change of HR-SBO support indication based on HR-SBO Request Indication from the V-SMF and/or the SM subscription data from UDM as described in clause 6.7.2.2 of TS 23.548 [33].

The Network Slice Replacement trigger shall trigger a SMF interaction with the PCF to notify change between S-NSSAI and Alternative S-NSSAI when the SMF determines that the existing PDU Session and existing SM Policy Association can be retained as described in clause 5.15.19 of TS 23.501 [2]. The SMF provides Alternative S-NSSAI if the PDU Session is transferred from a S-NSSAI to its Alternative S-NSSAI. The SMF indicates to the PCF that the PDU Session is transferred from the Alternative S-NSSAI to the replaced S-NSSAI, when the replaced S-NSSAI is available again and the PDU Session is transferred to the replaced S-NSSAI.

NOTE 15:	The SMF reports to the PCF a PDU session transfer anytime when the PDU Session is transferred from one S-NSSAI to another S-NSSA.

If the "ECN marking for L4S can no longer (or can again) be performed" trigger is armed, the SMF shall report to the PCF for those PCC rules which have enabled ECN marking for L4S (explicitly or implicitly as described in clause 6.1.3.22) if neither 5G-AN nor UPF PSA ECN marking for L4S can be enabled on the affected QoS Flows, and when ECN marking for L4S can be enabled on the affected QoS Flows (again).

If the "QoS Monitoring can no longer (or can again) be performed trigger" is armed, the SMF shall report to the PCF for those PCC rules with QoS Monitoring Policy when the support for QoS monitoring is not possible or has changed for the respective QoS Monitoring parameter, i.e. from QoS monitoring is possible to QoS monitoring is not possible or vice versa. The SMF determines whether QoS monitoring is possible or not as described in clause 5.45.1 of TS 23.501 [2] and indicates the affected QoS Monitoring parameter(s) to the PCF.

When the UE reachability status change is armed, the SMF subscribes to event of "UE reachability status" by using the Namf_EventExposure_Subscribe defined in clause 5.2.2.3.1 of TS 23.502 [3]. The SMF reports a change of the UE reachability status to the PCF.

When SMF receives a Non-3GPP Device Identifier, the SMF reports to PCF Non-3GPP device Identifier and corresponding user plane address as described in clause 5.52 of TS 23.501 [2] and Annex C of TS 23.316 [27].

# * * * * Next change * * * *

#### 6.1.3.18	Event reporting from the PCF

The AF may subscribe/unsubscribe to notifications of events from the PCF for the PDU Session to which the AF session is bound. The AF can either subscribe/unsubscribe directly at the PCF or indirectly via an NEF or a TSCTSF.

The PCF for the UE may subscribe/unsubscribe to notifications of events from the PCF for the PDU Session. Other NFs may subscribe/unsubscribe to notifications of events from the PCF for the PDU Session or from the PCF for the UE.

The events that can be subscribed by the AF and by other NFs are listed in Table 6.1.3.18-1.

Table 6.1.3.18-1: Events relevant for reporting from the PCF

| Event | Description | NF that can subscribe for reporting | Availability for Rx PDU Session (NOTE 2) | Availability for N5 per PDU Session | Availability for Bulk Subscription (NOTE 1) | Availability for N43 per SUPI, DNN, S-NSSAI | Availability for N5 per UE (NOTE 6) | Availability for N24 per UE (NOTE 6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PLMN Identifier Notification (NOTE 5) | The PLMN identifier or SNPN identifier where the UE is currently located. | AF, PCF | Yes | Yes | Yes | No | No | Yes |
| Change of Access Type | The Access Type and, if applicable, the RAT Type of the PDU Session has changed. | AF | Yes | Yes | Yes | No | No | No |
| EPS fallback | EPS fallback is initiated | AF | Yes | Yes | No | No | No | No |
| Signalling path status | The status of the resources related to the signalling traffic of the AF session. | AF | Yes | Yes | No | No | No | No |
| Access Network Charging Correlation Information | The Access Network Charging Correlation Information of the resources allocated for the AF session. | AF | Yes | Yes | No | No | No | No |
| Access Network Information Notification | The user location and/or timezone and/or identifier of the serving satellite of the UE when the PDU Session has changed in relation to the AF session. | AF | Yes | Yes | No | No | No | No |
| Reporting Usage for Sponsored Data Connectivity | The usage threshold provided by the AF has been reached; or the AF session is terminated. | AF | Yes | Yes | No | No | No | No |
| Service Data Flow deactivation | The resources related to the AF session are released. | AF, TSCTSF | Yes | Yes | No | No | No | No |
| Resource allocation outcome | The outcome of the resource allocation related to the AF session. | AF, TSCTSF | Yes | Yes | No | No | No | No |
| QoS targets can no longer (or can again) be fulfilled | The QoS targets can no longer (or can again) be fulfilled by the network for (a part of) the AF session. | AF | No | Yes | No | No | No | No |
| QoS Monitoring parameters | The QoS Monitoring parameter(s) (as defined in clause 5.45 of TS 23.501 [2]) are reported to the AF according to the subscription based on QoS Monitoring reports received from the SMF. | AF | No | Yes | No | No | No | No |
| Network support for QoS Monitoring | The QoS Monitoring can no longer (or can again) be performed by the network for the service data flow. | AF | No | Yes | No | No | No | No |
| Packet Delay Variation | Monitoring and reporting of 5GS Packet Delay Variation based on packet delay measured between UE and PSA UPF. | AF | No | Yes | No | No | No | No |
| Round-trip delay measurement over two service data flows | Measurements of round-trip delay considering the UL direction of a service data flow and the DL direction of another service data flow. It is derived from measurements of packet delay between UE and PSA UPF. | AF | No | Yes | No | No | No | No |
| Network support for ECN marking for L4S (NOTE 8) | The ECN marking for L4S can no longer (or can again) be performed by the network for the service data flow. | AF | No | Yes | No | No | No | No |
| Out of credit | Credit is no longer available. | AF | Yes | Yes | No | No | No | No |
| Reallocation of credit | Credit has been reallocated after the former Out of credit indication. | AF | Yes | Yes | No | No | No | No |
| 5GS Bridge/Router information Notification (NOTE 3) | 5GS Bridge/Router information that the PCF has received from the SMF. | TSN AF, TSCTSF | No | Yes | No | No | No | No |
| Notification on outcome of service area coverage change | The outcome of the request of service area coverage change. | AF | No | No | Yes | No | Yes | No |
| Notification on outcome of UE Policies delivery | The outcome of the request for UE policies delivery due to service specific parameter provisioning procedure. | AF | No | No | No | No | No | Yes |
| Start of application traffic detection and Stop of application traffic detection | The start or the stop of application traffic has been detected. | PCF, AF | No | No | Yes | Yes (NOTE 4) | No | No |
| UE reporting Connection Capabilities from associated URSP rule | The Connection Capability received from the UE during PDU Session Establishment or Modification, see clause 6.6.2.4. | PCF | No | No | No | Yes | No | Yes |
| Satellite backhaul category change | The backhaul has changed between different types of satellite backhaul, or the backhaul has changed between satellite backhaul and non-satellite backhaul. | AF | No | Yes | Yes | No | No | No |
| Change of PDUID | The PDUID assigned to a UE has changed. | 5G DDNMF | No | No | No | No | Yes | No |
| SM Policy Association established or terminated | The establishment or termination of a SM Policy Association is reported. | PCF | No | No | No | Yes (NOTE 7) | No | No |
| Reporting of extra UE addresses | Reporting of the extra IP addresses or address ranges allocated for the given PDU Session resulting from framed routes or IPv6 prefix delegation. | TSCTSF | No | Yes | No | No | No |  |
| Notification on BAT offset | The PCF reports the BAT offset and optionally the adjusted periodicity that has been received from the SMF. | TSCTSF | No | Yes | No | No | No |  |
| UE reachability status change | The PCF reports when it receives an indication of a change of the UE reachability status. | AF | No | Yes | No | No | No | No |
| Result of UE Policy Container delivery via EPS | The PCF reports the result of UE policies delivery via EPS. | PCF | No | No | No | Yes (NOTE 9) | No | No |
| Notification on outcome Network Slice Replacement | The PCF reports the outcome of Network Slice Replacement. | AF | No | No | No | No | Yes | No |
| Data Rate Limitation Information for the non-GBR service data flow | The PCF reports the uplink and downlink maximum bitrate authorized for the non-GBR service data flow. | AF | No | Yes | No | No | No | No |
| Data Rate Limitation Information for the PDU Session | The PCF reports the uplink and downlink Authorized Session-AMBR of the PDU Session. | AF | No | No | Yes | No | No | No |
| NOTE 1: Additional parameters for the subscription as well as reporting related to these events are described in TS 23.502 [3]. NOTE 2: Applicability of Rx is described in Annex C. NOTE 3: 5GS Bridge/Router information is described in clause 6.1.3.5. NOTE 4: Bulk subscription is implicit. NOTE 1 does not apply. NOTE 5: For a PDU Session established over a SNPN, the combination of the PLMN id and the NID identifies the SNPN. NOTE 6: This column contains also UE context related events that are reported to other consumers such as 5G DDNMF via other reference points than N5. The Conditions for reporting column indicates the respective consumer. NOTE 7: The PCF for the UE subscribes to this Event indirectly via AMF and SMF. NOTE 8: Subscription to this event is performed implicitly when AF provides the ECN marking for L4S support indication. NOTE 9: The PCF for the UE subscribes to this Event to PCF for the PDU Session. |  |  |  |  |  |  |  |  |

If an AF requests the PCF to report the PLMN identifier where the UE is currently located, then the PCF shall provide the PLMN identifier or the SNPN identifier to the AF if available. Otherwise, the PCF shall provision the corresponding PCC rules, and the Policy Control Request Trigger to report PLMN change to the SMF. The PCF shall, upon receiving the PLMN identifier or the SNPN identifier from the SMF forward this information to the AF, including the PLMN Id and if available the NID. If the H-PCF requests to report the PLMN identifier where the UE is currently located, the V-PCF provisions the PCRT on "PLMN change" to the AMF as described in clause 6.1.2.5 and then forwards the PLMN ID received from the AMF to the H-PCF.

If an AF requests the PCF to report on the change of Access Type, the PCF shall provide the corresponding Policy Control Request Trigger to the SMF to enable the report of the Change in Access Type to the PCF. The PCF shall, upon reception of information about the Access Type the user is currently using and upon indication of change of Access Type, notify the AF on changes of the Access Type and forward the information received from the SMF to the AF. The change of the RAT Type shall also be reported to the AF, even if the Access Type is unchanged. For MA PDU Session the Access Type information may include two Access Type information that the user is currently using.

If an AF requests the PCF to report on the signalling path status, for the AF session, the PCF shall, upon indication of removal of PCC Rules identifying signalling traffic from the SMF report it to the AF.

If an AF requests the PCF to report Access Network Charging Correlation Information, the PCF shall provide to the AF the Access Network Charging Correlation Information, which allows to identify the usage reports that include measurements for the Service Data Flow(s), once the Access Network Charging Correlation Information is known at the PCF.

If an AF requests the PCF to report Access Network Information (i.e. the User Location Report and/or the UE Timezone Report and/or the Satellite Identifier Report) at AF session establishment, modification or termination, the PCF shall set the Access Network Information report parameters in the corresponding PCC rule(s) and provision them together with the corresponding Policy Control Request Trigger to the SMF. For those PCC rule(s) based on preliminary service information the PCF may assign the 5QI and ARP of the QoS Flow associated with the default QoS rule to avoid signalling to the UE.

NOTE 1:	The PCF can also use the dynamic or pre-defined PCC Rules related to the IMS signalling to request Access Network Information reporting. This can be used to support e.g. regulatory requirements for SMS over IP, where the IMS network (i.e. P-CSCF) needs to retrieve the user location and/or UE Time Zone information. Note that due to regulatory requirements, the Access Network Information can be requested for SMS over IP, impacting a large number of PDU Sessions, that can lead to significant increase in signalling load when the Access Network Information is requested from AMF.

The PCF shall, upon receiving an Access Network Information report corresponding to the AF session from the SMF, forward the Access Network Information as requested by the AF (if the SMF only reported the serving PLMN identifier or the SNPN identifier to the PCF, as described in clause 6.1.3.5, the PCF shall forward it to the AF). For AF session termination the communication between the AF and the PCF shall be kept alive until the PCF report is received.

If an AF requests the PCF to report the Usage for Sponsored Data Connectivity, the PCF shall provision the corresponding PCC rules, and the Policy Control Request Trigger to the SMF. If the usage threshold provided by the AF has been reached or the AF session is terminated, the PCF forwards such information to the AF.

If an AF or TSCTSF requests the PCF to report the Service Data Flow deactivation, the PCF shall report the release of resources corresponding to the AF session. The PCF shall, upon being notified of the removal of PCC Rules corresponding to the AF session from the SMF, forward this information to the AF. The PCF shall also forward, if available, the reason why the resources are released, the user location information and the UE Timezone.

If an AF or TSCTSF requests the PCF to report the Resource allocation outcome, the PCF shall report the outcome of the resource allocation of the Service Data Flow(s) related to the AF session. The AF or TSCTSF may request to be notified about successful or failed resource allocation. In this case, the PCF shall instruct the SMF to report the successful resource allocation trigger (see clause 6.1.3.5). If the SMF has notified the PCF that the resource allocation of a Service Data Flow is successful and the currently fulfilled QoS matches an Alternative QoS parameter set (as described in clause 6.2.2.1), the PCF shall also provide to the AF the QoS Reference parameter or the Requested Alternative QoS Parameter Set which corresponds to the Alternative QoS parameter set referenced by the SMF. If the SMF has notified the PCF about resource allocation failure together with an Access Type (as described in clause 6.1.3.5), the PCF shall only notify the AF when the PCC rule is removed and without forwarding any Access Type information. If the SMF has notified the PCF about resource allocation failure due to UE temporary unreachable together with a maximum waiting time, if available, (as described in clause 6.1.3.5), the PCF shall notify the AF on resource allocation failure and provide the UE temporary unreachable and the maximum waiting time, if available.

If an AF requests the PCF to report when the QoS targets can no longer (or can again) be fulfilled for a particular media flow, the PCF shall set the QNC indication in the corresponding PCC rule(s) that includes a GBR or delay critical GBR 5QI value and provision them together with the corresponding Policy Control Request Trigger to the SMF. At the time, the SMF notifies that GFBR can no longer (or can again) be guaranteed for a QoS Flow to which those PCC Rule(s) are bound, the PCF shall report to the AF the affected media flow and provides the indication that QoS targets can no longer (or can again) be fulfilled. If the PCF receives a direction information together with the notification from the SMF, the PCF shall also forward the direction information to the AF. If other information is received together with the notification from SMF (see clause 5.7.2.4 of TS 23.501 [2]), the PCF shall also provide to the AF the QoS Reference parameter or the Requested Alternative QoS Parameter Set which corresponds to the Alternative QoS parameter set referenced by the SMF. If the SMF has indicated that the lowest priority Alternative QoS parameter set cannot be fulfilled, the PCF shall indicate to the AF that the lowest priority QoS Reference or the lowest priority set of Requested Alternative QoS Parameters of the Alternative Service Requirements cannot be fulfilled.

If the AF subscribes to be notified of the QoS Monitoring reports, the PCF decides about the path for the QoS Monitoring reports and sets the QoS Monitoring Policy Control Request Trigger accordingly, as described in clause 6.1.3.21. The PCF sends the QoS Monitoring reports to AF based on the QoS Monitoring reports that it receives from the SMF, according to AF subscription and PCF selected notification path e.g. PCF does not report to AF if AF will receive the QoS Monitoring reports directly from the UPF.

NOTE 2:	The QoS Monitoring report received by the PCF and the information sent to the AF can be different. The QoS Monitoring report (e.g. packer delay) may be used by PCF to calculate the requested QoS parameter (e.g. packet delay variation).

NOTE 3:	This event can only be subscribed as part of an AF session with required QoS (described in clause 6.1.3.22) and as part of AF requested QoS for a UE or group of UEs not identified by a UE address (described in clause 6.1.3.28).

NOTE 4:	If the service data flow is mapped to two QoS Flows (i.e. the UL traffic and DL traffic of the service data flow are separated into two QoS Flows respectively) in the same PDU Session, the PCF triggers QoS Monitoring for each direction packet delay of the individual QoS Flows respectively and generates the QoS Monitoring reports for the AF based on the packet delay monitored on the QoS Flow for each direction (as described in clause 5.37.4 of TS 23.501 [2]).

If the AF subscribes to be notified of Packet Delay Variation reports (the variation of UL/DL packet delay between UE and PSA UPF), the PCF triggers the QoS monitoring procedure, derives the 5GS Packet Delay Variation and reports the value to the AF, as described in clause 6.1.3.26.

NOTE 5:	This event can only be subscribed as part of an AF session with required QoS (described in clause 6.1.3.22).

If the AF subscribes to Round-trip delay measurement over two service data flows considering the UL direction of a service data flows and the DL direction of another service data flow, PCF triggers the QoS monitoring procedure to derive the Round-Trip delay measurement for delay measurements on the individual QoS Flows respectively (as described in clause 6.1.3.27.1 and in clause 5.37.4 of TS 23.501 [2]. The PCF derives the Round-Trip delay based on the packet delay measurement reports of the QoS Flows of each direction and reports the results to the AF. PCF sets QoS Monitoring Policies for each of the individual service data flows and QoS Monitoring Policy Control Request Trigger as described in clause 6.1.3.21.

NOTE 6:	This event can only be subscribed as part of an AF session with required QoS (described in clause 6.1.3.22).

If the AF subscribes to the event Network support for QoS Monitoring, the PCF sets the QoS Monitoring can no longer (or can again) be performed Policy Control Request Trigger in the SMF, if not done before. The PCF shall notify the AF that QoS Monitoring can no longer (or can again) be performed by the network based on the last notification from the SMF that QoS Monitoring can no longer (or can again) be performed but only if the QoS monitoring parameter for which the AF subscription relates to is affected.

NOTE 7:	AF QoS monitoring subscription can require implicit support of QoS monitoring parameters not directly associated with the functionality requested by the AF, e.g. PDV monitoring requires the support of packet delay monitoring. In that case the PCF will also report whether QoS monitoring can no longer (or can again) be performed.

If the AF indicates ECN marking for L4S support by the application, PCF authorizes the request and sets the ECN marking for L4S can no longer (or can again) be performed trigger accordingly. PCF shall further send the notification it receives from the SMF to AF on whether the network can not (or can again) perform ECN marking for L4S, for example, if due to user mobility neither target 5G-AN nor UPF PSA support ECN marking for L4S.

If an AF requests the PCF to report on the Out of credit event for the associated service data flow(s), the PCF shall inform the AF (when it gets informed by the SMF) that credit is no longer available for the services data flow(s) related to the AF session together with the applied termination action.

If an AF requests the PCF to report on the Reallocation of credit event for the associated service data flow(s), the PCF shall inform the AF (when it gets informed by the SMF) that credit has been reallocated after credit was no longer available and the termination action was applied for the service data flow(s) related to the AF session.

The PCF can arm the trigger of 5GS Bridge/Router information available to SMF based on local policy (i.e. without an AF request) or based on subscription request from TSCTSF. The PCF shall, upon reception of the 5GS Bridge/Router information (refer to clauses 6.1.3.23, 6.1.3.23a, 6.1.3.23b) from the SMF, forward this information to the TSN AF or the TSCTSF. When the PCF has received the User plane node Management Information Container or Port Management Information Container and related port number from SMF, the PCF also provides User plane node Management Information Container or Port Management Information Container and related port number to the TSN AF or TSCTSF. When SMF has reported the 5GS Bridge/Router information and no AF session exists, the PCF forward this information to a pre-configured TSN AF, or to a pre-configured TSCTSF or a TSCTSF discovered and selected via NRF. In the case of private IPv4 address being used for IP type PDU Session, the PCF shall additionally report DNN and S-NSSAI of the PDU Session to TSCTSF.

If the AF requests the PCF to report on the outcome of the service area coverage change, the PCF reports the outcome of the service area coverage change to the AF and notifies the current service area coverage to the AF. The outcome is the result of the execution of the request of service coverage change at the PCF; the outcome is successful if the request was executed, and includes the current service area coverage that may be the same or different from the service area coverage provided by the AF. The subscription may also be implicit. In this case there may be bulk subscription, either for an Internal-Group-Id or for any UE. In order to prevent massive notifications to the AF, the request for any UE is associated to a specific Application Identifier or DNN, S-NSSAI. For bulk subscription, when the AF request includes an expiration time, the PCF stops reporting to the AF when the expiration time is reached.

If the AF requests the (H-)PCF, via V-PCF when roaming, to report on the outcome of the UE Policies delivery due to service specific parameter provisioning procedure, the (H-)PCF reports the outcome of the related UE Policies provisioning procedure for the related traffic descriptor for the UE to the AF, via V-PCF when roaming. The outcome of the UE Policies provisioning procedure includes the success, the failure with an appropriate cause, including failure reported by the UE as described in TS 24.501 [22] or failure detected by the network e.g. service parameters provided by the AF are not authorized for a SUPI, or the interim status report such as the UE is temporarily unreachable or that URSP Rules have not yet been delivered by the H-PCF (see clauses 4.15.6.7 and 5.2.5.7 of TS 23.502 [3]). The PCF reports the outcome of the UE Policy provisioning procedure for each of the UEs that were included as Target UEs in the service specific information Data Subset in UDR. When the AF requested the PCF for the UE to report on the outcome of the UE Policies delivery due to service specific parameter provisioning procedure targeting a single UE, the Result of UE Policy Container delivery via EPS event trigger shall be subscribed.

NOTE 8:	An example reason for sending an interim status report that indicates that "URSP Rules have not yet been delivered by the H-PCF" may be that the UE does not support the VPLMN Specific URSP Rules feature and is not registered in the PLMN where the service parameters apply.

A request to report Start of application traffic detection and Stop of application traffic detection triggers the reporting when the PCF receives start of application traffic detection event or stop of application traffic detection event from SMF. The reception of a subscription to this event triggers the setting of the corresponding Policy Control Request Trigger to SMF, if not already subscribed.

A request to forward UE reporting Connection Capabilities from an associated URSP rule triggers the reporting when the PCF receives UE reporting of URSP rule enforcement information from the SMF matching specific Connection Capabilities (see clause 6.6.2.4). The request may include SUPI(s), DNN(s) and/or S-NSSAI(s) to which the request applies. The PCF reports the received Connection Capabilities and PDU Session information including the SUPI, UE requested DNN, Selected DNN, S-NSSAI, SSC Mode, PDU Session Type.

If an AF requests the PCF to report Start of application traffic detection and Stop of application traffic detection via bulk subscription, the AF shall provide the application identifier together with the S-NSSAI and DNN. The PCF provides a PCC rule for the application identifier together with the corresponding Policy Control Request Trigger to the SMF for every PDU Session to this S-NSSAI and DNN. When the PCF receives start of application traffic detection event or stop of application traffic detection event for the PCC rule in a PDU Session, the PCF forwards the event to the AF together with the UE identifier and optionally the IP address of the PDU Session corresponding to this PCC rule. When the AF removes bulk subscription for this application identifier, then the PCF removes the Policy Control Request Trigger from the SMF for every PDU Session to this S-NSSAN and DNN, if it is not used for other purpose.

NOTE 9:	The restriction of the bulk subscription to a specific combination of S-NSSAI and DNN avoids excessive signalling load.

If an AF requests the PCF to report on the change between different types of satellite backhaul or the change between satellite backhaul and non-satellite backhaul (as specified in clause 5.43.4 of TS 23.501 [2]), the PCF shall provide the corresponding Policy Control Request Trigger to the SMF to enable the report of satellite backhaul category change (see clause 6.1.3.5) to the PCF. The PCF shall, upon reception of information about the change of Satellite backhaul category, notify the AF on the Satellite backhaul category change event was met and forward the current Satellite backhaul category received from the SMF to the AF. When the satellite backhaul is no longer used, the Satellite backhaul category indicates that a non-satellite backhaul is used.

If 5G DDNMF requests the PCF to report on the Change of PDUID, the PCF shall notify whenever a new PDUID is allocated. Further details on how the 5G DDNMF retrieves and subscribes to notifications on Change of PDUID are defined in TS 23.304 [34].

A request to report SM Policy Association established or terminated triggers an implicit subscription and the reporting to the PCF for the UE when the PCF for the PDU Session receives PCF binding information together with Request for notification of SM Policy Association establishment and termination from the SMF. The PCF for the PDU Session derives the PCF for the UE from the PCF binding information and then notifies the PCF for the UE on this event, including the PCF binding information of the PCF for the PDU Session.

If the TSCTSF requests the PCF notifications for reporting of extra UE addresses, the PCF shall provide the extra UE addresses allocated to the PDU Session due to Framed Routes or IPv6 prefix delegation. The report shall include the actual list of IPv4 address masks or a list of IPv6 prefixes as currently allocated.

If the AF provides the Capability for BAT adaptation or BAT Window and subscribes to PCF for Notification on BAT offset, the PCF will trigger the subscription to SMF for Notification on BAT offset defined in clause 6.1.3.5. When the Notification on BAT offset trigger is set and the PCF receives a BAT offset and optionally an adjusted periodicity from the SMF, the PCF identifies the affected AF session (based on the PCC rule indicated by the SMF) and forwards the BAT offset and optionally the adjusted periodicity for this AF session to the TSCTSF.

A request to report Result of UE Policy Container delivery via EPS triggers the reporting when the PCF for the PDU Session receives the UE Policy Container from the UE during UE Policy Container delivery via EPS, or a delivery failure result for UE Policy Container delivery via EPS with appropriate reason from the SMF. The reception of a subscription to this event triggers the setting of the corresponding Policy Control Request Trigger to SMF, if not already subscribed.

If an AF requests the PCF to report on the UE reachability status change, the PCF shall provide the corresponding Policy Control Request Trigger to the SMF to enable the report of the UE reachability status change to the PCF, if not already subscribed. The PCF shall, upon indication of change of reachability status, notify the AF and forward the information received from the SMF to the AF.

If an AF requests the PCF to report on the change between S-NSSAI and Alternative S-NSSAI, the PCF reports the outcome of the network slice replacement to the AF.

If an AF requests the PCF to report on the Data Rate Limitation information (see clause 5.37.4 of TS 23.501 [2]) report for the non-GBR service data flow, the PCF reports the uplink and downlink maximum bitrate authorized for the service data flow to the AF. Further details are described in clause 6.1.3.27.1.

If an AF requests the PCF to report on Data Rate Limitation information report for the PDU Session, the PCF reports the Authorized Session-AMBR for the PDU Session to the AF. Further details are described in clause 6.1.3.27.1.

# * * * * Next change * * * *

#### 6.1.3.22	AF session with required QoS

The AF may request that a data session to a UE is set up with a specific QoS (e.g. low latency or PDV) and priority handling. The AF can request the network to provide QoS for the AF session based on the service requirements with the help of a QoS Reference parameter that refers to pre-defined QoS information. Instead of the QoS Reference, the AF may provide individual QoS parameters associated to the Flow Description.

a)	When the AF provides only a QoS Reference to determine the QoS parameters but no individual QoS parameters:

-	When the PCF authorizes the service information from the AF, it derives the QoS parameters of the PCC rule based on the service information and the indicated QoS Reference.

NOTE 1:	An SLA has to be in place between the operator and the ASP defining the possible QoS levels and their charging rates. For each of the possible pre-defined QoS information sets, the PCF needs to be configured with the corresponding QoS parameters and their values as well as the appropriate Charging key (or receive this information from the UDR).

-	The AF may change the QoS by providing a different QoS Reference while the AF session is ongoing. If this happens, the PCF shall update the related QoS parameter sets in the PCC rule accordingly.

b)	When the AF provides individual QoS parameters instead of a QoS Reference:

-	The AF provides one or more of the following individual QoS parameters, i.e. Requested Priority, Maximum Burst Size, Requested 5GS Delay, Requested Maximum Bitrate, Requested Guaranteed Bitrate and Requested Packet Error Rate.

NOTE 2:	Different combinations of individual QoS parameters with specific parameter names exist and they are described in TS 23.501 [2] (for Time Sensitive Communication), in clause 6.1.3.23 (for integration with Time Sensitive Networking) and in TS 29.514 [36].

-	If the AF request for QoS is sent via the TSCTSF and the request contains a Requested 5GS Delay, the TSCTSF determines a Requested PDB considering the UE-DS-TT Residence Time (either provided by the PCF or pre-configured).

-	When the PCF authorizes the service information from the AF, it derives the QoS parameters of the PCC rule based on the service information and the individual QoS parameters received from the AF and TSCTSF. The PCF should select a standardized, pre-configured or existing dynamically assigned 5QI that matches the individual QoS parameters. If no 5QI exists that matches the individual QoS parameters, the PCF generates a new dynamically assigned 5QI based on the individual QoS parameters.

-	The AF may change the QoS by providing different values for the individual QoS parameters while the AF session is ongoing. If this happens, the PCF shall update the related QoS parameter sets in the PCC rule accordingly.

-	The PCF may reject the individual QoS parameters received from the AF based on operator policy or impossibility to support the requested values of the individual QoS parameters. If this happens, the PCF may provide in the response to the AF one or more combinations of individual QoS parameters that can be supported. The PCF may indicate in the response to the AF that the request is not authorized because the applicable QoS parameters are not supported in the PLMN where the UE is registered, then the AF may retry the request when the UE moves to a different PLMN.

In addition to the QoS Reference or the individual QoS parameters described above, the AF may provide further parameters associated with the Flow Description, e.g. parameters that describe traffic characteristics as described in clause 6.1.3.23 or 6.1.3.23a and Indication of ECN marking for L4S.

The PCF generates a PCC Rule with service data flow filter (including IP Packet Filter set as in clause 5.7.6.2 of TS 23.501 [2]) or Ethernet Packet Filter set as in clause 5.7.6.3 of TS 23.501 [2]) derived from the Flow Descriptions provided by the AF, the derived PCC rule QoS parameters such a 5QI, ARP, GBR and MBR (see clause 6.3.1 for all possible PCC rule QoS parameters) and the associated TSC Assistance Container as received from the TSN AF or TSCTSF.

For TSC QoS, the PCF derives the 5QI value as defined in clause 5.27.3 of TS 23.501 [2], the PCF derives the MBR using the Requested Maximum Bitrate provided by the AF and sets the GBR equal to the MBR unless the AF provides a Requested Guaranteed Bitrate, in which case the MBR and GBR are set separately.

If the PCF gets informed about Policy Control Request Triggers relevant for the AF session, the PCF shall inform the AF about it as defined in clause 6.1.3.18.

If an AF session can adjust to different QoS parameter combinations, the AF may provide Alternative Service Requirements in a prioritized order (indicating the preference of the QoS requirements with which the service can operate) in addition to the QoS Reference or individual QoS parameters. Alternative Service Requirements contain:

-	When the AF requests the network to provide QoS with a QoS Reference, one or more QoS Reference parameters in a prioritized order.

-	When the AF requests the network to provide QoS with individual QoS parameters, one or more Requested Alternative QoS Parameter Set(s) in a prioritized order. Each Requested Alternative QoS Parameter Set is comprised of the following individual parameters: Requested 5GS Delay, Requested Guaranteed Flow Bitrate, Requested Packet Error Rate, and optionally, a Requested Averaging Window. Each requested Alternative QoS Parameter Set may also include a Maximum Burst Size parameter.

If the AF request is sent via the TSCTSF, the TSCTSF determines a Requested PDB considering the Requested 5GS Delay and the UE-DS-TT Residence Time.

An AF that provides Alternative Service Requirements shall also subscribe to receive notifications from the PCF for successful resource allocation and when the QoS targets can no longer (or can again) be fulfilled as described in clause 6.1.3.18.

When the PCF authorizes the service information from the AF and generates a PCC rule, it shall also derive Alternative QoS Parameter Sets for this PCC rule based on the QoS Reference parameters or the Requested Alternative QoS Parameter Sets in the Alternative Service Requirements. If the AF provided Requested Alternative QoS Parameter Sets in the request, the PCF may reject any of the Requested Alternative QoS Parameter Sets it has received based on operator policy or impossibility to support the requested values of the individual parameters. If this happens, the PCF may provide in the response to the AF one or more Requested Alternative QoS Parameters Sets that can be supported.

The PCF shall enable QoS Notification Control and include the derived Alternative QoS parameter sets (in the same prioritized order indicated by the AF) in the PCC rule sent to the SMF. When the PCF notifies the AF that QoS targets can no longer be fulfilled, the PCF shall include the QoS Reference parameter or the set of Requested Alternative QoS Parameters corresponding to the Alternative QoS parameter set referenced by the SMF, or an indication that the lowest priority QoS Reference or the lowest priority set of Requested Alternative QoS Parameters of the Alternative Service Requirements cannot be fulfilled (as described in clause 6.1.3.18).

NOTE 3:	The AF behaviour is out of the scope of this TS but can include adaptation to the change of QoS (e.g. rate adaptation) as well as application layer signalling with the UE.

The AF may change the Alternative Service Requirements while the AF session is ongoing. If this happens, the PCF shall update the Alternative QoS parameter sets in the PCC rule accordingly.

The AF may indicate to the PCF that the UE does not need to be informed about changes related to Alternative QoS Profiles. With this indication received from the AF, the PCF decides whether to disable the notifications to the UE when changes related to the Alternative QoS Profiles occur and sets the Disable UE notifications at changes related to Alternative QoS Profiles parameter in the PCC rule accordingly.

The AF may also provide the PCF with QoS duration and QoS inactivity interval. The requested QoS is applied to each QoS duration interval. Once the PCF receives the request from the AF, the PCF provides a PCC Rule with the QoS parameters to SMF to allocate resources. The PCF may allocate resources at the beginning of each QoS duration interval and release the resources at the end of the corresponding QoS duration interval. This process is repeated until the AF session is revoked. If the AF has subscribed to the PCF and resource allocation for any of the QoS duration interval fails, the PCF informs the AF of the resource allocation failure.

NOTE 4:	When leveraging the QoS duration and the QoS inactivity interval, both are expected to be in the order of minutes to avoid too frequent signalling between RAN, AF and 5GC/PCF.

If the AF provides an explicit indication (i.e. Indication of ECN marking for L4S support) that the UL and/or DL of the service data flow supports ECN marking for L4S or the PCF decides, based on local configuration, that the service data flow supports ECN marking for L4S, then the PCF may explicitly, or implicitly (based on PCF/SMF local configuration), indicate to the SMF to enable for ECN marking for L4S. The PCF decision may be taken, based on local configuration in PCF and SMF and L4S traffic detection result. If L4S support is detected on the UL and/or DL traffic of the service data flow, the QoS Flow is enabled with ECN marking for L4S, see clause 5.37.3 of TS 23.501 [2].

The PCF may generate policies to request to monitor the Traffic Parameter (i.e. N6 jitter range associated with DL Periodicity) and include it into a PCC rule based local policy. Based on the received PCC rule or local configuration, the SMF indicates UPF to monitor and report the requested traffic characteristics as described in clause 5.37.8.2 of TS 23.501 [2] and in clause 6.1.3.27.6.

The AF may also request measurements of QoS Monitoring parameters for the AF session (as described in clause 6.1.3.21).

# * * * * Next change * * * *

##### 6.1.3.27.4	PDU Set based handling

PDU Set based handling support is defined in clause 5.37.5 of TS 23.501 [2]. The PCF may, based on local configuration or on information received from the AF, generate PDU Set Control Information and the PCF includes the UL and/or DL Protocol Description (see clause 5.37.5 of TS 23.501 [2] and TS 26.522 [40]) and/or On-path N6 Signalling Information (see clause 5.37.9 of TS 23.501 [2]) received from AF and/or the PDU Set Control Information in the PCC Rule.

The PDU Set Control Information includes the PDU Set QoS Parameters (see clause 5.7.7 of TS 23.501 [2]). The PCF may in addition, based on local configuration or on information received from the AF, generate Alternative QoS Parameter sets containing PDU Set QoS parameters. Based on the PDU Set QoS parameters received in the PCC rule or a pre-configured PCC rule, the SMF provides 5G-AN with the PDU Set QoS Parameters and optionally Alternative PDU Set QoS parameters (as described in clause 5.37.5 of TS 23.501 [2]). If the PCC rule includes the Protocol Description and/or On-path N6 Signalling Information, it is used for PDU Set identification (as described in clauses 5.37.5 and 5.37.9 of TS 23.501 [2]).

When the AF requests that a data session to a UE (including the case of 5G-RG) is set up with a specific QoS (as described in clause 6.1.3.22), the AF may also provide Requested PDU Set Delay Budget (UL and/or DL), Requested PDU Set Error Rate (UL and/or DL) and/or Requested PDU Set Integrated Handling Information (UL and/or DL) to describe the requirements for the PDU Set based QoS handling (both the Requested PDU Set Delay Budget and Requested PDU Set Error Rate in one direction shall be provided together when they are provided by the AF) and the PCF generates the corresponding PDU Set QoS parameters (described in clause of 5.7.7 of TS 23.501 [2]) in the PCC rule. If Alternative Service Requirements are provided by the AF (described in clause 6.1.3.22), the AF is expected to also provide Requested PDU Set Delay Budget (UL and/or DL) and Requested PDU Set Error Rate (UL and/or DL) in the Alternative Service Requirements and the PCF generates the corresponding PDU Set QoS parameters in the Alternative QoS Parameter Sets in the PCC rule.

The AF may provide the Protocol Description (UL and/or DL) and/or On-path N6 Signalling Information as described in TS 23.501 [2].

# * * * * Next change * * * *

### 6.3.1	General

The Policy and charging control rule (PCC rule) comprises the information that is required to enable the user plane detection of, the policy control and proper charging for a service data flow. The packets detected by applying the service data flow template of a PCC rule form a service data flow.

Two different types of PCC rules exist: Dynamic rules and predefined rules. The dynamic PCC rules are provisioned by the PCF to the SMF, while the predefined PCC rules are configured into the SMF, as described in TS 23.501 [2], and only referenced by the PCF.

NOTE 1:	The procedure for provisioning predefined PCC rules is out of scope for this specification.

The operator defines the PCC rules.

Table 6.3.1 lists the information contained in a PCC rule, including the information name, the description and whether the PCF may modify this information in a dynamic PCC rule which is active in the SMF. The Category field indicates if a certain piece of information is mandatory or not for the construction of a PCC rule, i.e. if it is possible to construct a PCC rule without it.

The differences with table 6.3 in TS 23.203 [4] are shown, either "none" means that the IE applies in 5GS or "removed" meaning that the IE does not apply in 5GS, this is due to the lack of support in the 5GS for this feature or "modified" meaning that the IE applies with some modifications defined in the IE.

Table 6.3.1: The PCC rule information in 5GC

| Information name | Description | Category | PCF permitted to modify for a dynamic PCC rule in the SMF | Differences compared with table 6.3. in TS 23.203 [4] |
| --- | --- | --- | --- | --- |
| Rule identifier | Uniquely identifies the PCC rule, within a PDU Session. It is used between PCF and SMF for referencing PCC rules. | Mandatory | No | None |
| Service data flow detection | This part defines the method for detecting packets belonging to a service data flow. |  |  |  |
| Precedence | Determines the order, in which the service data flow templates are applied at service data flow detection, enforcement and charging. (NOTE 1). | Conditional (NOTE 2) | Yes | None |
| Service data flow template | For IP PDU traffic: Either a list of service data flow filters or an application identifier that references the corresponding application detection filter for the detection of the service data flow. For Ethernet PDU traffic: Combination of traffic patterns of the Ethernet PDU traffic. It is defined in clause 5.7.6.3 of TS 23.501 [2]. | Mandatory (NOTE 3) | Conditional (NOTE 4) | Modified (packet filters for Ethernet PDU traffic added) |
| Mute for notification | Defines whether application's start or stop notification is to be muted. | Conditional (NOTE 5) | No | None |
| Expedited Transfer Indication | Defines if Expedite Data Transfer of larger payload for XR application is set or not. |  | No | Added |
| Charging | This part defines identities and instructions for charging and accounting that is required for an access point where flow based charging is configured |  |  |  |
| Charging key (NOTE 22) | The charging system (CHF) uses the charging key to determine the tariff to apply to the service data flow. |  | Yes | None |
| Service identifier | The identity of the service or service component the service data flow in a rule relates to. |  | Yes | None |
| Sponsor Identifier | An identifier, provided from the AF which identifies the Sponsor, used for sponsored flows to correlate measurements from different users for accounting purposes. | Conditional (NOTE 6) | Yes | None |
| Application Service Provider Identifier | An identifier, provided from the AF which identifies the Application Service Provider, used for sponsored flows to correlate measurements from different users for accounting purposes. | Conditional (NOTE 6) | Yes | None |
| Charging method | Indicates the required charging method for the PCC rule. Values: online or offline or neither. | Conditional (NOTE 7) | No | None |
| Service Data flow handling while requesting credit | Indicates whether the service data flow is allowed to start while the SMF is waiting for the response to the credit request. Only applicable for charging method online. Values: blocking or non-blocking |  | No | New |
| Measurement method | Indicates whether the service data flow data volume, duration, combined volume/duration or event shall be measured. This is applicable to reporting, if the charging method is online or offline. Note: Event based charging is only applicable to predefined PCC rules and PCC rules used for application detection filter (i.e. with an application identifier). |  | Yes | None |
| Application Function Record Information | An identifier, provided from the AF, correlating the measurement for the Charging key/Service identifier values in this PCC rule with application level reports. |  | No | None |
| Service Identifier Level Reporting | Indicates that separate usage reports shall be generated for this Service Identifier. Values: mandated or not required |  | Yes | None |
| Policy control | This part defines how to apply policy control for the service data flow. |  |  |  |
| Gate status | The gate status indicates whether the service data flow, detected by the service data flow template, may pass (Gate is open) or shall be discarded (Gate is closed). |  | Yes | None |
| 5G QoS Identifier (5QI) | The 5QI authorized for the service data flow. | Conditional (NOTE 10) | Yes | Modified (corresponds to QCI in TS 23.203 [4]) |
| QoS Notification Control (QNC) | Indicates whether notifications are requested from 3GPP RAN when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. | Conditional (NOTE 15) | Yes | Added |
| Reflective QoS Control | Indicates to apply reflective QoS for the SDF. |  | Yes | Added |
| UL-maximum bitrate | The uplink maximum bitrate authorized for the service data flow |  | Yes | None |
| DL-maximum bitrate | The downlink maximum bitrate authorized for the service data flow |  | Yes | None |
| UL-guaranteed bitrate | The uplink guaranteed bitrate authorized for the service data flow |  | Yes | None |
| DL-guaranteed bitrate | The downlink guaranteed bitrate authorized for the service data flow |  | Yes | None |
| UL sharing indication | Indicates resource sharing in uplink direction with service data flows having the same value in their PCC rule |  | No | None |
| DL sharing indication | Indicates resource sharing in downlink direction with service data flows having the same value in their PCC rule |  | No | None |
| Redirect | Redirect state of the service data flow (enabled/disabled) | Conditional (NOTE 8) | Yes | None |
| Redirect Destination | Controlled Address to which the service data flow is redirected when redirect is enabled | Conditional (NOTE 9) | Yes | None |
| ARP | The Allocation and Retention Priority for the service data flow consisting of the priority level, the pre-emption capability and the pre-emption vulnerability | Conditional (NOTE 10) | Yes | None |
| Bind to QoS Flow associated with the default QoS rule | Indicates that the dynamic PCC rule shall always have its binding with the QoS Flow associated with the default QoS rule (NOTE 11). |  | Yes | Modified (corresponds to bind to the default bearer in TS 23.203 [4]) |
| Bind to QoS Flow associated with the default QoS rule and apply PCC rule parameters | Indicates that the dynamic PCC rule shall always have its binding with the QoS Flow associated with the default QoS rule. It also indicates that the that the QoS related attributes of the PCC rule shall be applied to derive the QoS parameters of the QoS Flow associated with the default QoS rule instead of the PDU Session related parameters Authorized default 5QI/ARP. | Conditional (NOTE 17) | Yes | Added |
| PS to CS session continuity | Indicates whether the service data flow is a candidate for vSRVCC. |  |  | Removed |
| Priority Level | Indicates a priority in scheduling resources among QoS Flows (NOTE 14). |  | Yes | Added |
| Averaging Window | Represents the duration over which the guaranteed and maximum bitrate shall be calculated (NOTE 14). |  | Yes | Added |
| Maximum Data Burst Volume (MDBV) | Denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB (NOTE 14). |  | Yes | Added |
| Disable UE notifications at changes related to Alternative QoS Profiles | Indicates to disable QoS Flow parameters signalling to the UE when the SMF is notified by the NG-RAN of changes in the fulfilled QoS situation. The fulfilled situation is either the QoS profile or an Alternative QoS Profile. | Conditional (NOTE 25) | Yes | Added |
| Precedence for TFT packet filter allocation | Determines the order of TFT packet filter allocation for PCC rules | Conditional (NOTE 28) | Yes | Added |
| ECN marking for L4S (NOTE 32) | The ECN marking for L4S indicates the UL and/or DL of the service data flow, detected by the service data flow template, supports ECN marking for L4S and to enable ECN marking for L4S for the service data flow. | Conditional | Yes | Added |
| Multi-modal Service ID | The Multi-modal Service ID indicates the multi-modal service that the service data flow is related to. | Conditional | Yes | Added |
| Access Network Information Reporting | This part describes access network information to be reported for the PCC rule when the corresponding QoS Flow is established, modified or terminated. |  |  |  |
| User Location Report | The serving cell of the UE is to be reported. When the corresponding QoS Flow is deactivated, and if available, information on when the UE was last known to be in that location is also to be reported. |  | Yes | None |
| UE Timezone Report | The time zone of the UE is to be reported. |  | Yes | None |
| Satellite Identifier Report | The identifier of the serving satellite of the UE is to be reported if the UE access is over a gNB onboard the satellite. |  | Yes | New |
| Usage Monitoring Control | This part describes identities required for Usage Monitoring Control. |  |  | None |
| Monitoring key (NOTE 23) | The PCF uses the monitoring key to group services that share a common allowed usage. |  | Yes | None |
| Indication of exclusion from session level monitoring | Indicates that the service data flow shall be excluded from PDU Session usage monitoring |  | Yes | None |
| N6-LAN Traffic Steering Enforcement Control (NOTE 18) | This part describes information required for N6-LAN Traffic Steering. |  |  |  |
| Traffic steering policy identifier(s) | Reference to a pre-configured traffic steering policy at the SMF (NOTE 12). |  | Yes | None |
| Metadata | Data provided by AF and included by UPF when forwarding traffic to N6-LAN. |  | Yes | Added |
| Application Function influence on traffic routing Enforcement Control (NOTE 18) | This part describes information required for Application Function influence on traffic routing. |  |  |  |
| Data Network Access Identifier | Identifier(s) of the target Data Network Access (DNAI). It is defined in clause 5.6.7 of TS 23.501 [2]. |  | Yes | Added |
| Per DNAI: Traffic steering policy identifier | Reference to a pre-configured traffic steering policy at the SMF (NOTE 19). |  | Yes | Added |
| Per DNAI: N6 traffic routing information | Describes the information necessary for traffic steering to the DNAI. It is described in clause 5.6.7 of TS 23.501 [2] (NOTE 19). |  | Yes | Added |
| Information on AF subscription to UP change events | Indicates whether notifications in the case of change of UP path are requested and optionally indicates whether acknowledgment to the notifications shall be expected (as defined in clause 5.6.7 of TS 23.501 [2]). |  | Yes | Added |
| Indication of UE IP address preservation | Indicates UE IP address should be preserved. It is defined in clause 5.6.7 of TS 23.501 [2]. |  | Yes | Added |
| Indication of traffic correlation (NOTE 29) | Indicates that the target PDU Sessions should be correlated via a common DNAI in the user plane. It is described in clause 5.6.7 of TS 23.501 [2]. |  | Yes | Added |
| Information on User Plane Latency requirements | Indicates the user plane latency requirements. It is defined in clause 6.3.6 of TS 23.548 [33]. |  | Yes | Added |
| Indication for Simultaneous Connectivity at Edge Relocation | Indicates request for simultaneous connectivity over source and target PSA from the AF (see clause 5.6.7 of TS 23.501 [2]). |  | Yes | Added |
| Information for EAS IP Replacement in 5GC | Indicates the Source EAS identifier and Target EAS identifier, (i.e. IP addresses and port numbers of the source and target EAS). (see clause 5.6.7 of TS 23.501 [2]). |  | Yes | Added |
| EAS Correlation indication | Indicates selecting a common EAS for the application identified by Service data flow template accessed by the UEs with the same Traffic Correlation ID. |  | Yes | Added |
| Traffic Correlation ID | Identification of a set of UEs accessing the application identified by the Service data flow template (see clause 5.6.7 of TS 23.501 [2]). |  | Yes | Added |
| Common EAS IP address | IP address of the common EAS for the application identified by the Traffic Description for the UEs the AF request aims at (as defined in clause 5.6.7 of TS 23.501 [2]). |  | Yes | Added |
| Common DNAI | Common DNAI applicable to the set of UEs identified by a traffic correlation ID. |  | Yes | Added |
| FQDN(s) | FQDN(s) for the application indicated in the PCC rule (see clause 5.6.7 of TS 23.501 [2]). |  | Yes | Added |
| Indication of considering N6 delay | Indicates to consider the N6 delay measurement. |  | Yes | Added |
| NEF information | Notification Endpoint of NEF subscription to be notified with information related to UE members of the set of UEs identified by traffic correlation ID. |  | Yes | Added |
| Handling of Payload Headers Control (NOTE 18) | This part describes information required for Handling of Payload Headers Control. |  |  |  |
| Header Handling Control information | Information required for Handling of Payload Headers (see clause 5.6.17 of TS 23.501 [2]) |  | Yes | Added |
| NBIFOM related control Information | This part describes PCC rule information related with NBIFOM. |  |  |  |
| Allowed Access Type | The access to be used for traffic identified by the PCC rule. |  |  | Removed |
| RAN support information | This part defines information supporting the RAN for e.g. handover threshold decision. |  |  |  |
| UL Maximum Packet Loss Rate | The maximum rate for lost packets that can be tolerated in the uplink direction for the service data flow. It is defined in clause 5.7.2.8 of TS 23.501 [2]. | Conditional (NOTE 13) | Yes | None |
| DL Maximum Packet Loss Rate | The maximum rate for lost packets that can be tolerated in the downlink direction for the service data flow. It is defined in clause 5.7.2.8 of TS 23.501 [2]. | Conditional (NOTE 13) | Yes | None |
| MA PDU Session Control (NOTE 20) | This part defines information supporting control of MA PDU Sessions |  | Yes | New |
| Application descriptors | Identifies the application traffic for which MA PDU Session control is required based on the Steering Functionality, the Steering Mode, Steering Mode Indicator and Threshold Values. It is described in clause 5.32.8 of TS 23.501 [2]. | Conditional (NOTE 27) | Yes | New |
| Steering Functionality | Indicates the applicable traffic steering functionality. | Conditional (NOTE 21, NOTE 31) | Yes | New |
| Steering Mode | Indicates the rule for distributing traffic between accesses together with associated steering parameters (if any). | Conditional (NOTE 21, NOTE 31) | Yes | New |
| Steering Mode Indicator | Indicates either autonomous load-balance operation or UE-assistance operation, if the Steering Mode is set to "Load Balancing", as defined in TS 23.501 [2]. |  | Yes | New |
| Threshold Values (NOTE 30) | A Maximum RTT or a Maximum Packet Loss Rate or both. |  | Yes | New |
| Transport Mode (NOTE 33) | Indicates the transport mode that should be used for the matching traffic, as defined in TS 23.501 [2]. |  | Yes | New |
| Charging key for Non-3GPP access (NOTE 22) | Indicates the Charging key used for charging packets carried via Non-3GPP access for a MA PDU Session. |  | Yes | New |
| Monitoring key for Non-3GPP access (NOTE 23) | Indicates the Monitoring key used to monitor usage of the packets carried via Non-3GPP access for a MA PDU Session. |  | Yes | New |
| QoS Monitoring | This part describes PCC rule information related with QoS Monitoring. |  |  |  |
| QoS Monitoring parameter(s) | Indicates the QoS Monitoring parameter(s) for which QoS Monitoring can be enabled as defined in clause 5.45 of TS 23.501 [2]. |  | Yes | Added |
| Reporting frequency | Defines the frequency for the reporting, such as event triggered, periodic. |  | Yes | Added |
| Target of reporting | Defines the target of the QoS Monitoring reports, it can be the NEF, the AF or the Local NEF. |  | Yes | Added |
| Indication of direct event notification | Indicates that the QoS Monitoring event shall be reported by the UPF directly to the NF indicated by the Target of reporting. |  | Yes | Added |
| DataCollection_ApplicationIdentifier | Identifier used in SMF to decide whether this PCC Rule corresponds to an event exposure subscription (see clause 4.15.4.5.1 of TS 23.502 [3]). |  | No | Added |
| Alternative QoS Parameter Sets (NOTE 24) (NOTE 26) | This part defines Alternative QoS Parameter Sets for the service data flow. |  |  |  |
| Packet Delay Budget | The Packet Delay Budget in this Alternative QoS Parameter Set. |  | Yes | Added |
| Packet Error Rate | The Packet Error Rate in this Alternative QoS Parameter Set. |  | Yes | Added |
| UL-guaranteed bitrate | The uplink guaranteed bitrate in this Alternative QoS Parameter Set. |  | Yes | Added |
| DL-guaranteed bitrate | The downlink guaranteed bitrate in this Alternative QoS Parameter Set. |  | Yes | Added |
| Maximum Data Burst Volume (MDBV) | The Maximum Data Burst Volume (MDBV) in this Alternative QoS Parameter Set. |  | Yes | Added |
| Averaging Window | The Averaging Window in this Alternative QoS Parameter Set |  | Yes | Added |
| UL-PDU Set Delay Budget | The uplink PDU Set Delay Budget (PSDB) in this Alternative QoS Parameter Set (see clause 5.7.7.2 of TS 23.501 [2]). |  | Yes | Added |
| DL-PDU Set Delay Budget | The downlink PDU Set Delay Budget (PSDB) in this Alternative QoS Parameter Set (see clause 5.7.7.2 of TS 23.501 [2]). |  | Yes | Added |
| UL-PDU Set Error Rate | The uplink PDU Set Error Rate (PSER) in this Alternative QoS Parameter Set (see clause 5.7.7.3 of TS 23.501 [2]). |  | Yes | Added |
| DL-PDU Set Error Rate | The downlink PDU Set Error Rate (PSER) in this Alternative QoS Parameter Set (see clause 5.7.7.3 of TS 23.501 [2]). |  | Yes | Added |
| TSC Assistance Container | This part defines parameters provided by TSN AF or TSCTSF. The parameters are defined in clause 5.27.2 of TS 23.501 [2]. |  | No | Added |
| Traffic Parameter Information | This part describes PCC rule information related with Traffic Parameter Information for power saving as specified in clause 5.37.8 of TS 23.501 [2]. |  |  |  |
| Periodicity | Indicates the time period between start of two data bursts in UL and/or DL direction. |  | Yes | Added |
| Traffic Parameter Measurement | This part describes PCC rule information related with Traffic Parameter Measurement for power saving as specified in clause 5.37.8 of TS 23.501 [2]. |  |  |  |
| Traffic Parameter to be measured | Indicates to measure the N6 jitter range associated with DL Periodicity and optionally, the UL/DL periodicity. |  | Yes | Added |
| Reporting condition | Defines the condition for the reporting, such as event triggered or periodic, frequency. |  | Yes | Added |
| Downlink Data Notification Control | This part describes information required for controlling the sending of Downlink data delivery status event and DDN Failure event notifications as specified in clause 4.15.3 of TS 23.502 [3]. |  |  |  |
| Notification control for DDD status | Indicates that notifications of downlink data delivery status are required and the requested type of such notifications. |  | Yes | Added |
| Notification Control for DDN Failure | Indicates that notifications of DDN Failure are required. |  | Yes | Added |
| PDU Set Control Information | Information needed to support the delivery of PDU Sets of a service data flow. |  |  |  |
| PDU Set QoS Parameters (UL/DL) | See clause 5.7.7 of TS 23.501 [2]. |  | Yes | Added |
| Data Burst Handling Information | This part describes Data Burst Handling Information |  |  |  |
| End of Data Burst Marking Indication | Indicates to detect last PDU of the data burst and to mark End of Data Burst Indication (See clause 5.37.8 of TS 23.501 [2]) on the last PDU. |  | Yes | Added |
| Data Burst Size Marking Indication | Indicates to identify the size of Data Burst and mark Data Burst Size (See clause 5.37.10.1 of TS 23.501 [2]). |  | Yes | Added |
| Time to Next Burst Marking Indication | Indicates to identify the time to next Data Burst and mark Time to Next Burst (See clause 5.37.10.2 of TS 23.501 [2]). |  | Yes | Added |
| Protocol Description Information | Information needed to support identifying PDU Set Information for packets, size of a Data Burst, the time to next Data Burst and/or last packet of a Data Burst. |  |  |  |
| Protocol Description (UL/DL) | Indicates the protocol, e.g. which is used to detect PDU Set Information of packets, size of a Data Burst, the time to next Data Burst and/or last packet of the Data Burst (see clauses5.37.5, 5.37.8, 5.37.10.1 and 5.37.10.2 of TS 23.501 [2] and TS 26.522 [40]), or the use of UDP-option for retrieving media related information (see clause 5.37.9 of TS 23.501 [2]). |  | No | Added |
| On-path N6 Signalling | Information needed to use On-path signalling for retrieving media related information. |  |  |  |
| On-path N6 Signalling Information | Indicates the method and provides the AS proxy address for On-path N6 signalling (see TS 23.501 [2] clause 5.37.9) |  |  |  |
| NOTE 1: For PCC rules based on an application detection filter, the precedence is only relevant for the enforcement, i.e. when multiple PCC rules overlap, only the enforcement, reporting of application starts and stops, monitoring, and charging actions of the PCC rule with the highest precedence shall be applied. NOTE 2: The Precedence is mandatory for PCC rules with SDF template containing SDF filter(s). For dynamic PCC rules with SDF template containing an application identifier, the precedence is either preconfigured in SMF or provided in the PCC rule from PCF. NOTE 3: Either service data flow filter(s) or application identifier shall be defined per each rule. NOTE 4: YES, if the service data flow template consists of a set of service data flow filters. NO if the service data flow template consists of an application identifier NOTE 5: Optional and applicable only if application identifier exists within the rule. NOTE 6: Applicable to sponsored data connectivity. NOTE 7: Mandatory if there is no default charging method for the PDU Session. NOTE 8: Optional and applicable only if application identifier exists within the rule. NOTE 9: If Redirect is enabled. NOTE 10: Mandatory when Bind to QoS Flow associated with the default QoS rule is not present. NOTE 11: The presence of this attribute causes the 5QI/ARP/QNC/Priority Level/Averaging Window/Maximum Data Burst Volume of the rule to be ignored for the QoS Flow binding. NOTE 12: The Traffic steering policy identifier can be different for uplink and downlink direction. If two Traffic steering policy identifiers are provided, then one is for uplink direction, while the other one is for downlink direction. NOTE 13: Optional and applicable only for voice service data flow in this release. NOTE 14: Optional and applicable only when a value different from the standardized value for this 5QI in Table 5.7.4-1 TS 23.501 [2] is required. NOTE 15: Optional and applicable only for GBR service data flow. NOTE 16: Usage of the charging information in described in TS 32.255 [21]. NOTE 17: Only one PCC rule can contain this attribute and this PCC rule shall not contain the attribute Bind to QoS Flow associated with the default QoS rule. NOTE 18: None, one, multiple or all of these control information may be present in a PCC rule. NOTE 19: Per DNAI, a Traffic steering policy identifier and/or N6 traffic routing information can be provided. If the pre-configured traffic steering policy (that is referenced by the Traffic steering policy identifier) contains information that is overlapping with the N6 traffic routing information, the N6 traffic routing information shall take precedence. NOTE 20: Only applicable to a PCC Rules provided to a MA PDU Session. NOTE 21: Mandatory when MA PDU Session Control information is provided. NOTE 22: When a Charging key for Non-3GPP access is provided, the parameters in the Charging Section (other than the Charging key) apply to both accesses and the Charging key (in the Charging Section) shall be used for charging packets carried via the 3GPP access. NOTE 23: When a Monitoring key for Non-3GPP access is provided, the Monitoring key (in the Usage Monitoring Control Section) shall be used to monitor usage of the packets carried via the 3GPP access. NOTE 24: Optional and applicable only for GBR service data flow with QoS Notification Control enabled. NOTE 25: Optional and applicable only for GBR service data flow for which Alternative QoS Parameter Set(s) are provided. NOTE 26: One or more Alternative QoS Parameter Sets can be provided in a prioritized order starting with the Alternative QoS Parameter Set that has the highest priority. NOTE 27: Mandatory in MA PDU Session Control information only when there is application identifier in the service data flow template. NOTE 28: If this parameter is used, it has to be present in every PCC rule of the PDU Session. NOTE 29: The use of traffic correlation is defined in TS 23.501 [2], clauses 5.6.7.1 and 5.29. NOTE 30: If Steering Mode is set to "Redundant", either a Maximum RTT or a Maximum Packet Loss Rate may be provided, but not both. NOTE 31: The Steering functionality "ATSSS-LL" shall not be provided together with Steering Mode "Redundant". NOTE 32: This parameter is only provided when the PCF is configured to provide an explicit indicator to the SMF to enable ECN marking for L4S for the traffic identified by the SDF template. NOTE 33: The Transport Mode may be included when the Steering Functionality is the MPQUIC-IP functionality,, the MPQUIC-UDP functionality,, or the MPQUIC-E functionality. |  |  |  |  |

The Rule identifier shall be unique for a PCC rule within a PDU Session. A dynamically provided PCC rule that has the same Rule identifier value as a predefined PCC rule shall replace the predefined rule within the same PDU Session.

The Precedence defines in what order the activated PCC rules within the same PDU Session shall be applied at the UPF for service data flow detection. When a dynamic PCC rule and a predefined PCC rule have the same precedence, the dynamic PCC rule takes precedence.

NOTE 2:	The operator shall ensure that overlap between the predefined PCC rules can be resolved based on precedence of each predefined PCC rule in the SMF. The PCF shall ensure that overlap between the dynamically allocated PCC rules can be resolved based on precedence of each dynamically allocated PCC rule.

For downlink packets all the service data flow templates, activated for the PDU Session shall be applied for service data flow detection and for the mapping to the correct QoS Flow. For uplink packets the service data flow templates activated on their QoS Flow shall be applied for service data flow detection (further details are provided in clause 6.2.2.2).

The Service data flow template may comprise any number of Service data flow filters or an application identifier as is defined in table 6.3.1.

NOTE 3:	Predefined PCC rules may include service data flow templates, which support extended capabilities, including enhanced capabilities to identify events associated with application protocols.

A Service data flow filter contains information for matching user plane packets for IP PDU traffic or Ethernet PDU traffic. All Service data flow filters of a Service data flow template shall be of the same type, i.e. either Packet Filters for IP or Ethernet PDU traffic (defined in clause 5.7.6 of TS 23.501 [2]). The Service data flow template information within an activated PCC rule is applied by the SMF to instruct the UPF to identify the packets belonging to a particular service data flow.

For the IP PDU Session type only, the Service data flow template may consist of an application identifier that references an application detection filter that is used for matching user plane packets. The application identifier is also identifying the application, for which the rule applies. The same application identifier value can occur in a dynamic PCC rule and one or multiple predefined PCC rules. If so, the PCF shall ensure that there is at most one PCC rule active per application identifier value at any time.

The Mute for notification defines whether notification to the PCF of application's starts or stops shall be muted. Absence of this parameter means that start/stop notifications shall be sent.

The Expedited Transfer Indication defines whether Expedite Data Transfer of larger payload for XR application is set or not. It takes values "TRUE" or "FALSE".

The Charging key is the reference to the tariff for the service data flow. Any number of PCC Rules may share the same charging key value. The Charging key values for each service shall be operator configurable.

NOTE 4:	Assigning the same Charging key for several service data flows implies that the charging does not require the credit management to be handled separately.

The Service identifier identifies the service. PCC Rules may share the same service identifier value. The service identifier provides the most detailed identification, specified for flow-based charging, of a service data flow.

NOTE 5:	The PCC rule service identifier need not have any relationship to service identifiers used on the AF level, i.e. is an operator policy option.

The Sponsor Identifier indicates the (3rd) party organization willing to pay for the operator's charge for connectivity required to deliver a service to the end user.

The Application Service Provider Identifier indicates the (3rd) party organization delivering a service to the end user.

The Charging method indicates whether online charging or offline charging is required, or the service data flow is not subject to any end user charging. If the charging method identifies that the service data flow is not subject to any end user charging, a Charging key shall not be included in the PCC rule for that service data flow, along with other charging related parameters. If the charging method is omitted the SMF shall apply the default charging method provided within the PDU Session related policy information (see clause 6.4). The Charging method is mandatory if there is no default charging method for the PDU Session.

NOTE 6:	With converged charging architecture for 5GC, online charging method also includes usage reporting from the SMF to the CHF. Hence, setting the charging method to online will also result in usage reports and thus allow for offline charging being performed by the CHF.

The Service Data Flow handling while requesting credit indicates either "blocking" if a credit for the Charging Key needs to be granted as a condition for the PCC Rule to be active or "non-blocking" if a credit for the Charging Key has been requested as a condition for the PCC Rule to be active.

The Measurement method indicates what measurements apply to charging for a PCC rule.

The Service Identifier Level Reporting indicates whether the SMF shall generate reports per Service Identifier. The SMF shall accumulate the measurements from all PCC rules with the same combination of Charging key/Service Identifier values in a single report.

The Application Function Record Information identifies an instance of service usage. A subsequently generated usage report (i.e. CDR), generated as a result of the PCC rule by the SMF, may include the Application Function Record Information, if available. The Application Function Record Information may contain the AF Charging Identifier and/or the Flow identifiers. If exclusive charging information related to the Application function record information is required, the PCF shall provide a service identifier, not used by any other PCC rule of the PDU Session at this point in time, for the AF session.

NOTE 7:	For example, the PCF may be configured to maintain a range of service identifier values for each service which require exclusive per instance charging information. Whenever a separate counting or credit management for an AF session is required, the PCF shall select a value, which is not used at this point in time, within that range. The uniqueness of the service identifier in the SMF ensures a separate accounting/credit management while the AF record information identifies the instance of the service.

The Gate indicates whether the SMF shall instruct the UPF to let a packet identified by the PCC rule pass through (gate is open) to discard the packet (gate is closed).

NOTE 8:	A packet, matching a PCC Rule with an open gate, may be discarded due to credit management reasons.

The 5G QoS Identifier, 5QI, represents the QoS parameters for the service data flow. The 5G QoS identifier is scalar and accommodates the need for differentiating QoS in both 3GPP and non-3GPP access type.

The bitrates indicate the authorized bitrates at the IP packet level of the SDF, i.e. the bitrates of the IP packets before any access specific compression or encapsulation.

The UL maximum-bitrate indicates the authorized maximum bitrate for the uplink component of the service data flow.

The DL maximum-bitrate indicates the authorized maximum bitrate for the downlink component of the service data flow.

The UL guaranteed-bitrate indicates the authorized guaranteed bitrate for the uplink component of the service data flow.

The DL guaranteed-bitrate indicates the authorized guaranteed bitrate for the downlink component of the service data flow.

The 'Maximum bitrate' is used for enforcement of the maximum bit rate that the SDF may consume, while the 'Guaranteed bitrate' is used by the SMF to determine resource allocation demands.

The UL sharing indication indicates that resource sharing in uplink direction for service data flows with the same value in their PCC rule shall be applied by the SMF as described in clause 6.2.2.4.

The DL sharing indication indicates that resource sharing in downlink direction for service data flows with the same value in their PCC rule shall be applied by the SMF as described in clause 6.2.2.4.

The Allocation and Retention Priority indicates the allocation, retention and priority of the service data flow. The ARP contains information about the priority level, the pre-emption capability and the pre-emption vulnerability. The Allocation and Retention Priority resolves conflicts of demands for network resources.

The Priority Level is signalled together with the 5QI to the (R)AN and UPF, only when a value different from the standardized value in the QoS characteristics Table 5.7.4-1 in TS 23.501 [2] is required.

The Averaging Window is signalled together with the 5QI to the (R)AN and UPF, only when a value different from the standardized value in the QoS characteristics Table 5.7.4-1 in TS 23.501 [2] is required.

The Maximum Data Burst Volume, MDBV, is signalled together with the 5QI to the (R)AN, only when a value different from the standardized value in the QoS characteristics Table 5.7.4-1 in TS 23.501 [2] is required.

The Bind to QoS Flow associated with the default QoS rule indicates that the SDF shall be bound to the QoS Flow associated with the default QoS rule. The presence of this parameter attribute causes the 5QI/ARP of the rule to be ignored by the SMF during the QoS Flow binding.

The Bind to QoS Flow associated with the default QoS rule and apply PCC rule parameters indicates that the SDF shall be bound to the QoS Flow associated with the default QoS rule and that the QoS related attributes of the PCC rule shall be applied by the SMF to derive the QoS parameters of the QoS Flow associated with the default QoS rule instead of the PDU Session related information Authorized default 5QI/ARP.

NOTE 9:	The Bind to QoS Flow associated with the default QoS rule and apply PCC rule parameters Indication has to be used whenever the PDU Session related information Authorized default 5QI/ARP (as described in clause 6.3.1) cannot be directly used as the QoS parameters of the QoS Flow associated with the default QoS rule, for example when a GBR 5QI is used or the 5QI priority level has to be changed.

The QoS Notification Control, QNC, indicates whether notifications are requested from the access network (i.e. 3GPP RAN) when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. If it is set and the GFBR can no longer (or can again) be guaranteed, the access network (i.e. 3GPP RAN) sends a notification towards the SMF, which then notifies the PCF.

The Disable UE notifications at changes related to Alternative QoS Profiles parameter indicates to disable QoS Flow parameters signalling to the UE when the SMF is notified by the NG-RAN of changes in the fulfilled QoS situation. The fulfilled situation is either the QoS profile or an Alternative QoS Profile.

The Precedence for TFT packet filter allocation parameter determines the order in which TFT packet filters are allocated for PCC rules. The PCF may include this parameter if there is a possibility to run into a restriction regarding the number of TFT packet filters that can be allocated for the PDU Session and interworking with EPS with N26 deployment is supported (see also clause 4.11.1 of TS 23.502 [3]).

NOTE 10:	PCF can know that interworking with EPS with N26 is supported based on DNN and S-NSSAI of the PDU Session.

The Reflective QoS Control indicates to apply reflective QoS for the service data flow. The indication is used to control the RQI marking in the DL packets of the service data flow and may trigger the sending of the RQA parameter for the QoS Flow the service data flow is bound to. Reflective QoS is defined in clause 5.7.5 of TS 23.501 [2].

NOTE 11:	While the UE applies a standardized value for the precedence of all UE derived QoS rules, PCC rules require different precedence values and PCF configuration has to ensure that there is a large enough value range for the precedence of PCC rules corresponding to UE derived QoS rules. To avoid that the precedence of network provided QoS rules need to be changed when Reflective QoS is activated and filters are overlapping, the PCF will take the standardized value for the precedence of UE derived QoS rules into account when setting the precedence value of PCC rules subject to Reflective QoS.

The Reflective QoS Control parameter shall not be used for the PCC rule with match-all SDF template. If PCC rule with match-all SDF template is present, the Reflective QoS Control parameter shall not be used for PCC rules which contain the Bind to QoS Flow of the default QoS rule parameter, either.

The N6-LAN Traffic Steering Enforcement Control contains Traffic steering policy identifier(s) for steering traffic onto N6-LAN to the appropriate N6 service functions deployed by the operator or a third party, and optional Metadata if received from AF due to Application Function influence on Service Function Chaining as described in clause 5.6.16 of TS 23.501 [2].

The content of the Header Handling Control information is defined in clause 5.6.17 of TS 23.501 [2].

The ECN marking for L4S indicates that the service data flow supports ECN marking for L4S to be performed.

The Multi-modal Service ID indicates the multi-modal service that the service data flow is related to.

The access network information reporting parameters (User Location Report, UE Timezone Report, Satellite Identifier Report) instruct the SMF about what information to forward to the PCF when the PCC rule is activated, modified or removed.

The Monitoring Key is the reference to a resource threshold. Any number of PCC Rules may share the same monitoring key value. The monitoring key values for each service shall be operator configurable.

The Indication of exclusion from session level monitoring indicates that the service data flow shall be excluded from the PDU Session usage monitoring.

The Application Function influence on traffic routing Enforcement Control may contain:

-	a set of DNAI(s) (i.e. a reference to the DNAI(s) the SMF needs to consider for UPF selection/reselection), an optional Indication of traffic correlation and, per DNAI, a corresponding Traffic steering policy identifier (i.e. a reference to a pre-configured traffic steering policy at the SMF), and/or a corresponding N6 traffic routing information (when the N6 traffic routing information is provided explicitly as part of the AF request, as described in clause 5.6.7 of TS 23.501 [2]), or;

-	an AF subscription to UP change events parameter which contains subscription information defined in clause 5.2.8.3 of TS 23.502 [3] for the change of UP path Event Id i.e. an Indication of early and/or late notification and information on where to provide the corresponding notifications (Notification Target Address + Notification Correlation ID as specified in clause 4.15.1 of TS 23.502 [3]) and optionally an indication of "AF acknowledgment to be expected" to the corresponding notifications as described in clause 5.6.7 of TS 23.501 [2].

-	a user plane latency requirements parameter which contains AF requested information on the requirements for user plane latency defined in TS 23.548 [33].

-	an indication for Simultaneous Connectivity at Edge Relocation, which includes Keep existing PSA indication to provide guidance to the network on whether to provide simultaneous connectivity over source and target PSA at edge relocation; also, optionally a related Keep existing PSA timer that indicates the minimum time interval to be considered for inactivity for the traffic described before the connectivity over the source PSA may be removed, as defined in TS 23.548 [33].

-	an EAS Correlation indication, which indicates selecting a common EAS for a set of UEs identified by Traffic Correlation ID and accessing the application identified by Service data flow template or FQDN(s).

-	a Traffic Correlation ID, identifying a set of UEs accessing the application identified by the Service data flow template or FQDN(s). See more in clause 5.6.7 of TS 23.501 [2].

-	a Common EAS IP address, IP address of the common EAS accessed by the UEs with the same Traffic Correlation ID, for the application identified by the Service data flow template.

-	FQDN(s), FQDN(s) for the application traffic identified by the Service data flow template, and used for influencing EASDF-based DNS query procedure as defined in TS 23.548 [33].

-	NEF Information, Notification Endpoint of the NEF responsible of the set of UEs associated with the Traffic correlation ID.

-	Indication of considering N6 delay which indicates to consider the N6 delay measurement between UPF(s) and EAS(es). See more in clause 6.9 of TS 23.548 [33].

The Traffic Steering Enforcement Control may contain Indication of UE IP address preservation. The SMF takes this indication into account when determining whether to reselect PSA UPF, as specified in clause 5.6.7 of TS 23.501 [2].

The Redirect indicates whether the uplink part of the service data flow should be redirected to a controlled address.

The Redirect Destination indicates the target redirect address when Redirect is enabled.

The UL Maximum Packet Loss Rate indicates the maximum rate for lost packets that can be tolerated in the uplink direction.

The DL Maximum Packet Loss Rate indicates the maximum rate for lost packets that can be tolerated in the downlink direction.

The Application descriptors provides one or several instances of the OSId and OSAppId combination. It is used by the UE to identify the application traffic for which steering is required based on the Steering Functionality, the Steering Mode, the Steering Mode Indicator and the Threshold Values.

The Steering Functionality indicates the method for how traffic matching the SDF template in the UPF or a Traffic descriptor in the UE is sent over the MA PDU Session. The method ATSSS_LL indicates that the traffic matching the SDF template is sent over the MA PDU Session without additional tunnelling, e.g. with IP flow switching. The method MPTCP indicates that the traffic matching the SDF template is sent over the MA PDU Session using MPTCP. The methods MPQUIC-UDP, MPQUIC-IP, or MPQUIC-E indicate that traffic matching the SDF template is sent over the MA PDU Session using MPQUIC based on the CONNECT-UDP, CONNECT-IP, or CONNECT-Ethernet proxy methods, respectively.

The Steering Mode indicates the rule for distributing downlink SDFs in the UPF or uplink traffic in the UE between accesses, together with the associated steering parameters. The PCF may indicate separate values for uplink and downlink directions. The available Steering Modes are defined in TS 23.501 [2].

The Steering Mode Indicator indicates that the UE or the UPF or both may change the steering parameters provided in the Steering Mode and may adjust the traffic steering based on their own decisions, as further defined in TS 23.501 [2]. When the PCF selects the Load-Balancing Steering Mode for both the uplink and the downlink, and the PCF provides a Steering Mode Indicator for the uplink equal to UE-assistance operation, then the PCF shall provide the same Steering Mode Indicator for the downlink.

The Threshold Values indicate the authorized RTT or Packet Loss Rate for a SDF. Depending on the Steering Mode value, the PCF may include, per SDF, one threshold value for RTT or one threshold value for Packet Loss Rate, or both. The PCF may indicate separate values for uplink and downlink directions. The Steering Modes that may use the threshold values and how the UE and UPF enforces them are defined in TS 23.501 [2].

The Transport Mode indicates one of the Transport Modes as defined in TS 23.501 [2] that should be applied for transmitting a UDP flow between UE and UPF. The Transport Mode may be included when the MPQUIC-UDP functionality, the MPQUIC-IP functionality, or the MPQUIC-E functionality is selected as the Steering Functionality.

The Charging key for Non-3GPP access indicates the Charging key that shall be used for charging the detected service data flow traffic carried via Non-3GPP access. The other charging related parameters apply for both accesses.

The Monitoring key for Non-3GPP access indicates the Monitoring key that shall be used for monitoring the usage of the detected service data flow traffic carried via Non-3GPP access.

The QoS Monitoring parameter(s) indicates the QoS Monitoring parameter(s) for which QoS Monitoring can be enabled for a service data flow as defined in clause 5.45 of TS 23.501 [2], e.g. the UL packet delay, DL packet delay or round trip packet delay between the UE and the UPF.

The Reporting frequency indicates the frequency for the reporting, such as event triggered, periodic. The following applies:

-	If the Reporting frequency indicates "periodic", the reporting period shall also be included in the PCC rule.

-	The reporting period shall also be used for reporting measurement failure in any of the Reporting frequency modes "periodic" or "event triggered".

NOTE 12:	The indication of a measurement failure is not possible or not supported for some QoS Monitoring parameter(s) (as described in the respective clauses of clause 5.45 of TS 23.501 [2]).

-	If the Reporting frequency indicates "event triggered", the reporting period, Reporting threshold(s) and the minimum waiting time shall also be included in the PCC rule. The Reporting threshold(s) indicates the measurement threshold for each of the included QoS Monitoring parameter(s). The minimum waiting time indicates the minimum time interval between subsequent reports. In the case of monitoring of the UL and/or DL Available Bitrate for the service data flow, a list of Reporting threshold(s) shall be included for the respective direction(s) as described in clause 6.1.3.27.1 and clause 5.45.6 of TS 23.501 [2].

The Target of reporting indicates the target for the QoS Monitoring reports sent as notifications. It can be either the NEF, the AF or the Local NEF, indicated as Notification Target Address + Notification Correlation ID.

The Indication of direct event notification indicates that the QoS Monitoring reports shall be sent by the UPF directly to the NF indicated by the Target of reporting (i.e. to the Local NEF or the AF as described in clause 5.8.2.18 of TS 23.501 [2]).

The DataCollection_ApplicationIdentifier is provided to assist the SMF when it needs to decide whether this PCC Rule corresponds to an event exposure subscription (see clause 4.15.4.5.1 of TS 23.502 [3]).

The Alternative QoS Parameter Set(s) define alternative set(s) of QoS parameters for the service data flow. Every set consists of a PER, a PDB, an UL and a DL guaranteed bitrate QoS parameter, optionally, an Averaging Window and optionally, an PSDB and PSER for UL and/or a DL direction. For delay-critical service data flow, every Alternative QoS Parameter Set may also include a Maximum Data Burst Volume (MDBV).

The content of the TSC Assistance Container is defined in clause 5.27.2 of TS 23.501 [2].

The Traffic Parameter Information applies to the UE power saving as specified in clause 5.37.8 of TS 23.501 [2]. The following parameters are included:

-	Periodicity:

-	indicates the time period between start of two data bursts in UL and/or DL direction.

-	this parameter is only included when PCF receives the periodicity information from AF.

The Traffic Parameter Measurement applies to the UE power saving as specified in clause 5.37.8 of TS 23.501 [2]. The following parameters are included:

-	Traffic Parameter to be measured:

-	UL and/or DL periodicity. This parameter is only included when PCF does not receive the periodicity information from AF.

-	N6 jitter range associated with DL Periodicity.

-	reporting condition can be optionally included to define the condition for the reporting, such as event triggered or periodic, frequency.

The Downlink Data Notification Control applies to the control of subscription to Downlink Data Delivery status event notifications and DDN Failure event notifications as specified in clause 4.15.3 of TS 23.502 [3]. The following parameters are included:

-	The Notification control for DDD status applies as described in clause 4.15.3.2.8 of TS 23.502 [3] and contains the following parameters:

-	indication that notifications of Downlink Data Delivery status are required; and

-	the requested type of such notifications (notifications about downlink packets being buffered, and/or discarded).

-	The Notification Control for DDN Failure applies as described in clause 4.15.3.2.9 of TS 23.502 [3] and contains the following parameters:

-	indication that notifications of DDN Failure are required.

NOTE 12:	Downlink Data Notification Control information is provided to assist the SMF in the generation/update of N4 information. The PCF will not be notified about the Downlink data delivery status events or the DDN Failure events.

The PDU Set Control Information is needed to support the delivery of PDU Sets for a service data flow. The parameter values for UL and DL may be different (see clause 5.7.7 of TS 23.501 [2]).

The Protocol Description Information is needed to support identifying PDU Set Information for DL/UL packets, size of a DL Data Burst, time to next Data Burst and/or last packet of a DL Data Burst. The Protocol Descriptions for UL and DL may be different (see clause 5.37.5 of TS 23.501 [2]). It may also indicate whether Media over QUIC or UDP-option is used to carry media related information, see clause 5.37.9 of TS 23.501 [2].

The On-path N6 Signalling Information is used to receive media related information using On-path N6 signalling, see clause 5.37.9.3 and clause 5.37.9.4 of TS 23.501 [2].

The Data Burst Handling Information is needed for the following:

-	to support detecting last PDU of the Data Burst and marking End of Data Burst Indication on the last packet of the Data Burst (see clause 5.37.8 of TS 23.501 [2]);

-	to support identifying the size of a DL Data Burst and marking Data Burst Size as defined in clause 5.37.10.1 of TS 23.501 [2];

-	to support identifying the time to next Data Burst and marking Time to Next Burst as defined in clause 5.37.10.2 of TS 23.501 [2].

# * * * * End of changes * * * *