3GPP TSG-SA #108	SP-250574

Prague, Czech Republic, 10-13 June 2025	(was SP-250xxx)

Source:	Qualcomm Incorporated

Title:	Way Forward on AIoT device credentials storage

Document for:	Discussion/Approval

Agenda Item:	3.3

Work Item / Release:	Ambient_IoT-SEC/ Rel-19

Abstract of the contribution: Proposes a way forward for AIoT device credentials storage.

# 1.	Discussion

Protection requirements of authentication credentials stored in the Ambient IoT (AIoT) device (short: “AIoT device credentials storage”) has been a controversial topic discussed in 3GPP for almost a year. There were LSs exchanged with SA1 (examples [1], [2], [3]). SA3 has also been liaising externally to 3GPP (examples [4], [5]). The issue stems from the fact that there is an inherent trade off between the requirements of RAN for low power consumption (e.g. ~1 µW peak power consumption, a few hundred µW peak power consumption) and low complexity cost of the device and the requirement that until now all 3GPP UEs using 3GPP RATs that connect to a PLMN is mandated to store the credentials in UICC. It is obvious from both ETSI 102 221 [6] and TR 31.970 [7] that UICC needs orders of magnitude more power during UICC initialization stage. SP-240879 [8] calculated the power requirement to be 11mW (or 11000 times the requirement of peak power consumption from RAN). Put another way, no convincing argurment has been shown that a UICC is a currently viable choice for secure credential at the power levels that some AIoT devices will support.  Hence mandating UICC for this use case may make the use not achievable or prohibitally expensive.

On the other hand, the security requirements for AIoT cannot be disregarded. While the main driver for rel.19 is “indoor warehouse” use case, it cannot be prevented that a deployment uses AIoT devices and system for other not yet known use cases. Also there is evidence already (see [9]) that the AIoT functionality is expected to expand to include outdoor scenarios, and device originated procedures.

The good news is that the 3GPP security framework that was defined in rel.15 (for NR/5GC) provides some flexibility already. For NR/5GC as defined in TS 33.501 [10] is already possible to have UE-to-NW authentication procedures without requiring storage of credentials in UICC as long as it is for a “private network”. The more precise definition of private network was provided in rel.16 onwards through the definition of Standalone Non-Public Network (SNPN). For Rel-19, as there is no expectation of connectivity between PLMNs, e.g. AIoTF of operator A and ADM of operator B, then in many ways all deployments are in effects SNPNs. It is therefore proposed that for AIoT device credentials storage the similar principles are  followed by 3GPP SA3.

We believe that the following is a good outline of a compromise way forward and strongly encourage feedback and discussion on such an approach:

- when the AIoT system is defined as SNPN the AIoT device credentials storage follows 3GPP defined requirements e.g. t the exact mechanism to achieve these requirements is out of scope of 3GPP;

- when the AIoT system is defined as PLMN the AIoT device credentials storage should/shall use UICC.

NOTE 0: The above PLMN requirement is based on no connectivity between PLMNs and needs re-consideration before applying to more general cases. Revisting the above PLMN requirement is also dependent on prgress on understanding the power usage of various forsm of UICC.

NOTE 1: In case UICC is used, what is the exact form factor and whether it is removable or integrated is out of scope of 3GPP.

NOTE 2: It is up to the operator to define the AIoT system as SNPN or PLMN.

# 2.	Proposal

It is proposed that SA#108 endorses the following guidance for AIoT device credentials storage:

- when the AIoT system is defined as SNPN, the AIoT device credentials storage follows 3GPP defined requirements, the exact mechanism is out of scope of 3GPP (similar to Annex I.2.2 of TS 33.501 [10]);

- when the AIoT system is defined as PLMN, the AIoT device credentials storage should/shall use UICC.

NOTE 0: The above PLMN requirement is based on no connectivity between PLMNs and needs re-consideration before applying to more general cases. Revisting the above PLMN requirement is also dependent on progress on understanding the power usage of various forsm of UICC.

NOTE 1: In case UICC is used, the exact form factor and whether it is removable or integrated is out of scope of 3GPP.

NOTE 2: It is up to the operator to define the AIoT system as SNPN or PLMN.

The above guidance, if endorsed, can be sent to SA3 and other involved WGs with LS from SA#108, or documented in the meeting minutes of SA#108.

# 3.	References

[1]	“LS on Further Clarification for Ambient IoT Security”, SP-241419

[2] “LS on UICC support for Ambient IoT Service”, SP-241016

[3] “Reply to LS on Further Clarification for Ambient IoT Security”, S1-242490

[4]	“LS on secure storage and processing of credentials for AIoT”, S3-250966

[5] “Reply LS on secure storage and processing of credentials for AIoT”, S3-251818

[6] ETSI 102 221: “UICC-Terminal interface; Physical and logical characteristics”

[7] TR 31.970: “UICC power optimisation for Machine-Type Communication (MTC)”

[8] SP-240879: “UICC support for Ambient IoT Service”, Oppo

[9] S2-2506092: “Study item on Architecture support of Ambient power-enabled Internet of Things-Phase 2”

[10] TS 33.501: “Security architecture and procedures for 5G System”