3GPP TSG-SA WG6 Meeting #67	S6-252476

# Fukuoka, Japan, 19-23 May 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0041 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Deregistration of an FL Member |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 23.482, there is a capability for registering an FL member (in clause 8.4); however this mechanism to de-register the FL member is missing. This CR attempts to fix this issue and complete the capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The missing procedure for the de-registration is added in clause 8.4, as well as the information flows related to the de-registration, and the APIs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no mechanism in AIMLE to support the FL member de-registering from the ML repository. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3, 8.4, 8.4.1, 8.4.2, 8.4.3, 8.4.3A (new), 8.4.4.6 (new), 8.4.4.7 (new), 9.3.1.1, 9.3.1.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

## 6.3	Support for FL member registration

This functionality covers the registration,registration update and de-registration of the candidate FL member to the ML repository which is keeping the FL member registrations. Such candidate member can be a VAL server functionality or an enabler layer functionality (e.g. AIMLE server) which is registering to the ML repository/registry to act as FL member for a given application event (analytics event or event triggered by a VAL layer application server).

* * * * Second Change * * * *

## 8.4	Support for FL member registration

### 8.4.1	General

This clause provides the procedures for the registration, registration update, and deregistration of candidate FL members in the ML repository which serves as service registry for the FL members undertaking a task related to the ML model lifecycle, such as ML model local training. Candidate FL members can be application layer entities at the server side (e.g., VAL server, AIMLE server), which can potentially be selected as FL clients or FL server for a particular ASP /vertical requirement, or at the client side (e.g. AIMLE client), which can potentially be selected as FL clients. If VAL server or the AIMLE client is the candidate FL member, it registers indirectly via AIMLE server to the ML repository.

### 8.4.2	Procedure on FL member registration

Figure 8.4.2-1 illustrates the procedure where the registration of a candidate FL member happens via the ML repository, serving as AIML service registry.

Figure 8.4.2-1: Procedure for registration on FL member registry

1.	The candidate FL member (e.g., VAL server via AIMLE server or AIMLE server) sends an FL member registration request to the ML repository for registering to the ML repository which acts as the AIML service registry.

2.	The ML repository validates the received request and generates the identity and other security related information for all the FL members listed in the registration request.

3.	The ML repository sends the generated information in the FL member registration response message to the candidate FL member.

The procedure for AIMLE client to register to ML repository as FL member is introduced in clause 8.7.2.2.

### 8.4.3	Procedure on FL member registration update

Figure 8.4.3-1 illustrates the procedure where the registration update of a candidate FL member happens via the ML repository.

Figure 8.4.3-1: Procedure for registration update

1.	The candidate FL member (VAL server via AIMLE server or AIMLE Server) sends an FL member registration update request to the ML repository acting as AIML service registry for updating the registration of the FL member.

2.	The ML repository validates the received update request and generates the identity and other security related information for the FL member.

3.	The ML repository sends the generated information in the FL member registration update response message to the candidate FL member.

The procedure for AIMLE client to update its registration to ML repository as FL member is introduced in clause 8.7.2.3.

### 8.4.3A	Procedure on FL member deregistration

Figure 8.4.3A-1 illustrates the procedure where the deregistration of a candidate registered FL member happens via the ML repository, serving as AIML service registry.

The deregistration may be triggered by the VAL server or AIMLE server in case of an expected or predicted unavailability of the candidate FL member e.g. due to load / energy, access limitation in different VAL service area based on the vertical/ASP requirement.

Such deregistration may be provided by the AIMLE server on behalf of a VAL server e.g. after offboarding of the VAL server from the SEAL platform, or upon termination of the AIMLE/VAL service (FL task).

Figure 8.4.3A-1: Procedure for deregistration on FL member registry

1.	The candidate registered FL member (e.g., VAL server via AIMLE server or AIMLE server) sends an FL member deregistration request to the ML repository for registering to the ML repository which acts as the AIML service registry.

2.	The ML repository validates the received request for all the registered FL members listed in the deregistration request.

3.	The ML repository sends the generated information in the FL member deregistration response message to the candidate registered FL member.

The procedure for AIMLE client to deregister to ML repository as FL member is introduced in clause 8.7.2.4.

### 8.4.4	Information flows

#### 8.4.4.1	General

The following information flows are specified for supporting the FL member registration and registration update based on clause 8.4.2 and clause 8.4.3.

#### 8.4.4.2	FL member registration request

Table 8.4.4.2-1 describes information elements for the FL member registration request from the candidate FL member (VAL server, AIMLE server) to the ML repository/registry.

Table 8.4.4.2-1: FL member registration request

| Information element | Status | Description |
| --- | --- | --- |
| Security information | M | Information for ML repository to validate the registration request. |
| FL member ID | M | The identifier of the candidate FL member. This can be the VAL server ID, the EAS ID, the AIMLE server ID based on which entity is the candidate FL member. |
| FL member supported AI/ML role | M | The supported AI/ML role of FL member which can be used as FL server or FL client. |
| FL member capabilities | O (NOTE) | FL member capability information (e.g. ML application type, allowed resource usage level). |
| FL member velocity | O | Indicates the FL member velocity (e.g., mobile or static). |
| FL member Location information | O | Indicates the location information of the UE (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses). |
| FL member Availability schedule | O (NOTE) | Availability schedule of the FL member for a certain FL member. |
| Supported ML model ID list | O | The list of ML model IDs for which the FL member can be used. |
| Area of Interest | O | The area of interest (i.e. location co-ordinates) for which the registration applies. |
| Time validity | O | The time validity for the registration of the FL member. |
| NOTE: At least one of these shall be present. |  |  |

#### 8.4.4.3	FL member registration response

Table 8.4.4.3-1 describes information elements for the FL member registration response to the candidate FL member (VAL server, AIMLE server) from the ML repository/registry.

Table 8.4.4.3-1: FL member registration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the registration request (positive or negative acknowledgement). |
| Registration ID | M | The generated ID for the registration. |

#### 8.4.4.4	FL member registration update request

Table 8.4.4.4-1 describes information elements for the FL member registration update request from the candidate FL member (VAL server, AIMLE server) to the ML repository/registry.

Table 8.4.4.4-1: FL member registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | Identifier of the existing registration for which the update request applies. |
| Security information | M | Information for ML repository to validate the registration update request |
| FL member ID | M | The identifier of the candidate FL member for which the update applies. This can be the VAL server ID, the EAS ID, the AIMLE server ID based on which entity is the candidate FL member. |
| Updated registration parameters | M | The updates of the registration parameters. |
| > FL member supported AI/ML role | O (see NOTE) | The updated supported AI/ML role of FL member which can be FL server, FL client. |
| > FL member Availability schedule | O (see NOTE) | Updated Availability schedule of the FL member. |
| > supported ML model ID list | O (see NOTE) | The updated list of ML model IDs for which the FL member can be used |
| > area of interest | O (see NOTE) | The updated area of interest for the registration update. |
| > time validity | O (see NOTE) | The update time validity for the FL member registration update. |
| NOTE: At least one of these information elements shall be present. |  |  |

#### 8.4.4.5	FL member registration update response

Table 8.4.4.5-1 describes information elements for the FL member registration update response to the candidate FL member (VAL server, AIMLE server) from the ML repository/registry.

Table 8.4.4.5-1: FL member registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the registration update request (positive or negative acknowledgement). |
| Registration ID | O | The generated ID for the new registration based on the update (if needed). |

#### 8.4.4.6	FL member deregistration request

Table 8.4.4.6-1 describes information elements for the FL member deregistration request from the candidate FL member (VAL server, AIMLE server) to the ML repository/registry.

Table 8.4.4.2-1: FL member deregistration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | The identity of the registered FL member |

#### 8.4.4.7	FL member deregistration response

Table 8.4.4.7-1 describes information elements for the FL member deregistration response to the candidate registered FL member (VAL server, AIMLE server) from the ML repository/registry.

Table 8.4.4.3-1: FL member deregistration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the deregistration request (positive or negative acknowledgement). |

* * * * Second Change * * * *

### 9.3.1	MLR FL member Registration API

#### 9.3.1.1	General

Table 9.3.1.1-1 illustrates the API for FL member registration. This API enables the VAL server (candidate FL member) to communicate with the ML repository for registering as candidate FL member.

Table 9.3.1.1-1: MLR_FLMemberRegister API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| MLR_FLMemberRegistration | Request | Request/Response | VAL Server |
| MLR_FLMemberRegistrationUpdate | Request | Request/Response | VAL Server |
| MLR_FLMemberDeregistration | Request | Request/Response | VAL Server |

* * * Third Change * * * *

#### 9.3.1.X MLR_FLMemberDeregistration_Request operation

API operation name: MLR_FLMemberDeregistration_Request

Description: The consumer requests for deregistering a candidate registered FL member.

Inputs: See clause 8.4.4.6.

Outputs: See clause 8.4.4.7.

See clause 8.3A for details of usage of this operation.