ao3GPP TSG-SA WG6 Meeting #67	S6-252404

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252231)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0043 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on ML model information storage and discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There are several unambiguous constructions used in the specification, e.g. can - In ML model informaiton discovery procedure, the order of the paragrahgs is wrong: The AIMLE server cannot determine whether the received ML model satisfy the ML model requirement before ML repository send the response message to the AIMLE server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Fix the unambiguous constructions. - Fix the wrong order fo the paragrahgs in ML model informaiton discovery procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unclear description will remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.2.1, 8.11.3, 8.11.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.11.2.1	AIMLE server-initiated ML model information storage

Figure 8.11.2.1-1 illustrates the procedure of the AIMLE server-initiated ML model information storage.

Pre-condition:

1.	The AIMLE server has either received application specific model details from AIML consumer or produced analytics model.

Figure 8.11.2.1-1: AIMLE server-initiated ML model information storage

1.	The AIMLE server sends a ML model information storage request to the ML repository to store an ML model. The ML model included in the request is one trained by the ML repository consumer or its information is  provisioned to the ML repository consumer. The request contains information elements as described in Table 8.11.4.1-1. The request message may contain Indication of continuous training and continuous training model parameter.

NOTE 1:	The security requirements within the ML model storage requirements are out of scope.

2.	Upon receiving the ML model information storage request, the ML repository verifies if the AIMLE server is authorized to store the ML model identified by ADAE Analytics ID and/or list of the allowed vendors provided within the ML model information attribute. If the AIMLE server is authorized, the ML repository processes the request and records information of the ML model (e.g., by creating a ML model profile). The ML repository sends an ML model information storage response to the AIMLE server with an identifier of the created ML model profile.

### 8.11.3	ML model information discovery

Figure 8.11.3-1 illustrates the procedure of ML model information discovery.

Pre-condition:

1.	Either AIMLE consumer has requested to discover ML model or AIMLE server decides to discover model for training.

Figure 8.11.3-1: ML model information discovery

1.	The AIMLE server sends an ML model information discovery request to the ML repository. The request contains information elements as described in Table 8.11.4.3-1.

2.	Upon receiving the ML model information discovery request, the ML repository verifies if the requestor is authorized to discover the ML model(s) identified by the ML model ID. Further, it verifies whether the requestor is present in the list of allowed vendors or not. If the ML repository consumer is authorized, the ML repository processes the request and discovers the model based on ML model ID, ADAE Analytics ID, Base Model ID, and/or ML model interoperability information.

If the discovery request is for transfer of learning, the ML repository identifies all candidate models matching the domain and required accuracy level.

After the AIMLE server receives the response from the ML repository, the AIMLE server determines whether the received ML model satisfy the ML model requirement of the AIMLE server and whether the AIMLE server needs to continuously train the model. The determination for continuous training is based on the AIMLE server capability for continuous training, its own requirements for the ML model and ML model profile (Indication of continuous model training, continuous training model parameter).

The ML repository sends the response message to the ML repository consumer. The response includes information of the discovered ML model.

NOTE:	When the ML model is within training or re-training phase, the ML model is not available for discovery. The details are indicated within the Result IE in Table 8.11.4.4-1. The training and re-training phase is used during federated learning so that AIML enablement consumer can rely on model repository to get the model details.

After the AIMLE server receives the response from the ML repository, the AIMLE server determines whether the received ML model satisfy the ML model requirement of the AIMLE server and whether the AIMLE server needs to continuously train the model. The determination for continuous training is based on the AIMLE server capability for continuous training, its own requirements for the ML model and ML model profile (Indication of continuous model training, continuous training model parameter).

* * * Next Change * * * *

#### 8.11.4.1	ML model information storage request

Table 8.11.4.1-1 describes the information flow from the AIMLE server to the ML repository or from the AIMLE client/VAL server to the AIMLE server as a request for the ML model information storage.

Table 8.11.4.1-1: ML model information storage request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identity | M | The identity of the requestor performing the request. |
| Security Credentials | O (NOTE 1) | Security credentials of the requestor performing the request. |
| ML model | O (NOTE 2) | The ML model to be stored in the ML repository. |
| ML model address | O (NOTE 2) | The address (e.g., a URL or an FQDN) of the ML model file. |
| ML model information | M | Provides information of the ML model, as described in Table 8.11.4.1-2. |
| NOTE 1: This information is needed if the requestor is at the VAL service provider domain. NOTE 2: At least one of these information elements shall be provided. |  |  |

* * * End of Change * * * *