3GPP TSG-SA WG6 Meeting #67	S6-252441

Fukuoku, Japan, 17th – 21st May 2025	(revision of S6-252087)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0740 | rev | 1 | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Eees_EASDiscovery API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Misalignments exist in the specification in the service description clauses and the procedures: - In clauses 8.8.1.1, 8.8.2.4, 8.8.3.2, and 8.8.4.2, it described that EAS sends requests to EES for EAS discovery, reuse the mechanism introduced in clause 8.5. Thus, EAS is also consumer of the Eees_EASDiscovery API. - Duplications exist in the EES services on EAS Discovery and Target EAS Discovery. This paper proposes to align the consumer of the Eees_EASDiscovery API, in the service description clauses and the procedures, and remove duplications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Add EAS to the consumer list of Eees_EASDiscovery API. - Add EES as consumer of the Eees_EASDiscovery service, to align with the description in clauses 8.8.3.2 and 8.8.4.3. - Remove Eees_TargetEASDiscovery from the table 6.7.2-2. - Remove "Target" before the "EASDiscovery" in table 6.7.2-5. - Remove Eees_TargetEASDiscovery API from the ACR API table. - Remove API clauses for Eees_TargetEASDiscovery. - Add IEs to Table 8.15.3.2-1 EAS information provisioning request, to align the description in clause 8.18.2.4.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignments exist in the specification in the service description calsues and the procedures for the Eees_EASDiscovery API. Duplications exist in the EES services on EAS Discovery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2, 8.5.1, 8.8.4.3, 8.8.5.1, 8.8.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.7.2	APIs provided by the Edge Enabler Layer

Table 6.7.2-1 summarizes the APIs exposed by the ECS.

Table 6.7.2-1: APIs provided by the ECS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eecs_ServiceProvisioning | EEC | 8.3 |
| Eecs_EESRegistration | EES | 8.4.4 |
| Eecs_TargetEESDiscovery | EES | 8.8.3.3 |

Table 6.7.2-2 summarizes the APIs exposed the EES.

Table 6.7.2-2: APIs provided by the EES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EECRegistration | EEC | 8.4.2 |
| Eees_EASRegistration | EAS | 8.4.3 |
| Eees_EASDiscovery | EEC, EAS, EES | 8.5 |
| Eees_UELocation | EAS | 8.6.2 |
| Eees_ACRManagementEvent | EAS | 8.6.3 |
| Eees_AppClientInformation | EAS | 8.6.4 |
| Eees_UEIdentifier | EAS | 8.6.5 |
| Eees_SessionWithQoS | EAS | 8.6.6 |
| Eees_AppContextRelocation | EEC, EAS | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_EELManagedACR | EAS | 8.8.3.6 |
| Eees_SelectedTargetEAS | EAS | 8.8.3.7 |
| Eees_ACRStatusUpdate | EAS | 8.8.3.8 |
| Eees_EECContextPull | EES | 8.9.4.2 |
| Eees_EECContextPush | EES | 8.9.4.3 |

NOTE:	The event exposure related APIs (e.g. Eees_EASDiscovery and Eees_ACREvents) can be realized as single event subscription API.

* * * Next Change * * * *

### 8.5.1	General

Discovery procedures enable entities in an edge deployment to obtain information about EAS and their available services, based on specified criteria of interest.

EAS discovery enables the EEC, EAS, and EES to obtain information about available EASs of interest. The discovery of the EASs is based on matching EAS discovery filters provided in the request.

When multiple EASs are discovered for a specific AC, the EEC may select one or more EASs to enable AC communication with one of the selected EASs. The selection algorithm is outside the scope of this specification. Once the EAS is selected, the EEC may subscribe for the ACR event notifications at the EES of the selected EAS, as described in clause 8.8.3.5.2. The EDN configuration information received from ECS may be used for establishing a connection to EAS(s).

EAS discovery may be initiated by the EEC when a certain trigger condition at the UE is met. Some examples are as follows:

-	AC related updates available at the EEC (e.g. due to AC installation/re-installation/activation), AC requesting application server access;

-	Lifetime received via EAS discovery response specified in clause 8.5.3 is expired; or

-	EEC detects the need of application context relocation as in clause 8.8.

NOTE:	When the EEC decides to perform EAS discovery is up to EEC implementation.

EAS discovery may also be initiated by the EAS or EES when a certain trigger condition is met, as described in clause 8.8.3.2.

* * * Next Change * * * *

#### 8.8.4.3	EAS discovery response

The information elements specified in the Table 8.5.3.3-1 is used for the EAS discovery response sent from the EES to the EAS

.

* * * Next Change * * * *

#### 8.8.5.1	General

Table 8.8.5.1-1 illustrates the APIs for ACR.

Table 8.8.5.1-1: ACR APIs

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_EASDiscovery | Request | Request/Response | EAS, EES |
| Eees_SelectedTargetEAS | Declare | Request/Response | EAS |
| Eecs_TargetEESDiscovery | Request | Request/Response | EES |
| Eees_AppContextRelocation | Request | Request/Response | EEC, EAS |
| Eees_ACREvents | Subscribe | Subscribe/Notify | EEC |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eees_EELManagedACR | Request | Request/Response | EAS |
|  | Subscribe | Subscribe/Notify | EAS |
|  | Notify |  |  |
| Eees_ACRStatusUpdate | Request | Request/Response | EAS |

* * * Next Change * * * *

#### 8.8.5.2	Eees_EASDiscovery API

See clause 8.5.4.2 for details of the API operations for T-EAS discovery.

##### 8.8.5.2.1	Void

##### 8.8.5.2.2	Void

See clause 8.8.3.2 for details of usage of the T-EAS discovery operation.

* * * END of Changes * * * *