3GPP TSG-SA WG6 Meeting #67	S6-252411

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252168)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0046 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial anchor group management operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Sometimes several spatial anchors at a specific location illustrates services which are of similar characteristics and in these cases they can be better managed as groups. Managing them as groups offers more flexibility to the spatial anchor creators and users. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the procedures for managing spatial anchors as group. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functionality of managing spatial anchors as groups will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All new – 8.3.x.2, 8.3.x.2.1, 8.3.x.2.2, 8.3.x.3, 8.3.x.3.1, 8.3.x.3.2, 8.3.x.3.3, 8.3.x.3.4, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Implement this CR after implementing CR#29 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.3.x.2	Procedures

##### 8.3.x.2.1	Spatial anchor group creation

Figure 8.3.x.2.1-1 illustrates the spatial anchor group creation procedure through which the requestor SAn client or the VAL server can request the SAn server to create a spatial anchor group associating or linking a list of spatial anchors with the group. SAn server creates and stores the spatial anchor group and assigns a unique identifier (Spatial anchor group ID) to enable the requestor to perform spatial anchor group related operations.

Editor's note:	Whether and how SEAL GMS capability can be extended to support SAn server in grouping the spatial anchors is FFS.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

Figure 8.3.x.2.1-1: Spatial anchor group creation

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor group create request to the SAn server. The request includes a requestor identifier, security credentials, list of spatial anchors identified by the spatial anchor IDs to be associated or linked as part of the spatial anchor group, human readable description about the spatial anchor group and service area where the spatial anchor group is accessible as described in Table 8.3.x.3.1-1.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the creation of spatial anchor group. If the requestor is authorized, the SAn server creates the spatial anchor group and assigns a unique identifier to it and stores the spatial anchor group related information provided in the step 1. The spatial anchor group related information contains spatial anchor group ID, list of spatial anchor IDs associated with the group, service area etc.

3.	The SAn server sends a spatial anchor group create response as described in Table 8.3.x.3.2-1 to the requestor. If the SAn server created spatial anchor group, the response includes an indication of success and spatial anchor group identifier of newly created spatial anchor group and list of spatial anchor IDs associated with the created group. Otherwise, the response includes an indication of failure and can include a reason for failure.

##### 8.3.x.2.2	Spatial anchor group deletion

Figure 8.3.x.2.2-1 illustrates the spatial anchor group deletion procedure through which the requestor SAn client or the VAL server can request the SAn server to delete a spatial anchor group.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.x.2.2-1: Spatial anchor group deletion

1.	 The requestor (e.g., SAn client or VAL server) sends a spatial anchor group delete request to the SAn server. The request as includes a requestor identifier, security credentials, identifier of the spatial anchor group. If the requestor wants to delete more than one group, the request can carry the list of spatial anchor group IDs. If the requestor wants to delete the spatial anchors and its information also, the request can include an indication to delete the spatial anchors.

2.	Upon receiving the request, the SAn server validates if the spatial anchor group exists and if the requestor is authorized to delete the spatial anchor group. If the requestor is authorized, the SAn server deletes the spatial anchor group and its associated information. If the request indicates to delete the spatial anchors associated with the group, then the SAn server deletes the spatial anchors associated with the group and the related information.

3.	The SAn server sends a spatial anchor group delete response to the requestor. If its success response, it includes the list of spatial anchor group IDs that are deleted. It may also include the list of spatial anchors that are deleted if the request includes an indication to delete the spatial anchors. In case of failure, the response includes the reason for failure.

#### 8.3.x.3	Information flows

##### 8.3.x.3.1	Spatial anchor group create request

Table 8.3.x.3.1-1 describes information elements for the spatial anchor group create request from the SAn client or VAL server to the SAn server.

Table 8.3.x.3.1-1: Spatial anchor group create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| Group description | O | Human readable description about the spatial anchor group (e.g. List of restaurants) |
| Service area | O | Service area where the spatial anchor group is visible or accessible. |
| VAL service information | M | Information about the application service associated with the spatial anchor group. |
| > Service ID | M | The identifier of the application service. |
| Access control rules | O | Access control rules defining which entities are permitted to discover the spatial anchor group. |
| List of spatial anchors IDs | M | List of spatial anchors IDs to be associated with the group. |

##### 8.3.x.3.2	Spatial anchor group create response

Table 8.3.x.3.2-1 describes information elements for the spatial anchor group create response from the SAn client or VAL server to the SAn server.

Table 8.3.x.3.2-1: Spatial anchor group create response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the spatial anchor group create request was successful. |
| > Spatial anchor group ID | M | The unique identifier of the created spatial anchor group. |
| > List of Spatial anchor IDs | M | List of spatial anchor IDs associated with the created spatial anchor group |
| Failure response (NOTE) | O | Indicates that the create spatial anchor request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: One of the IEs shall be present. |  |  |

##### 8.3.x.3.3	Spatial anchor group delete request

Table 8.3.x.3.3-1 describes information elements for the spatial anchor group delete request from the SAn client or VAL server to the SAn server.

Table 8.3.x.3.3-1: Spatial anchor group delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| List of Spatial anchor group | M | Information related to the spatial anchor groups to be deleted |
| > Spatial anchor group ID | M | Identify of the spatial anchor group |
| > Delete spatial anchors indication | O | Indicates whether the SAn server need to delete the spatial anchors associated with the spatial anchor group |

##### 8.3.x.3.4	Spatial anchor group delete response

Table 8.3.x.3.4-1 describes information elements for the spatial anchor group delete response from the SAn server to SAn client or VAL server.

Table 8.3.x.3.4-1: Spatial anchor group delete response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| > List of Spatial anchor group information | M | Information related to the spatial anchor groups |
| >> Spatial anchor group ID | M | Identify of the spatial anchor group deleted successfully |
| >> List of Spatial anchor ID | O | Identify of the spatial anchors associated with the spatial anchor group that are deleted. This IE shall be present if the request includes an indication to delete the spatial anchors also. |
| Failure response (NOTE) | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: Only one of these IE shall be present. |  |  |