3GPP TSG-SA WG6 Meeting #66	S6-251283

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251120)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0147 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The support to enable 5G ProSe multi-hop UE-to-network relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MultiHop-MC |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR introduces the support of 5G ProSe multi-hop UE-to-network relay in alignment with 3GPP TS 23.304. The CR is aligned with the concepts defined in TS 23.304, 5G ProSe single hop UE2NW relay is not explicitly mentioned in the relevant architecture. Please refer to clause 4.2.7 with the two figures below: figure 4.2.7.1-1 and 4.2.7.1A-1 Figure 4.2.7.1-1: Reference architecture for 5G ProSe Layer-3 UE-to-Network Relay Figure 4.2.7.1A-1: Reference architecture for 5G ProSe multi-hop Layer-3 UE-to-Network Relay 5G ProSe single hop is equivalent to the concept of having a 5G ProSe UE2NW relay between the remote UE and the network, which is what is already specified in Rel-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Adding a new clause describing the main parameters to enable 5G ProSe UE-to-network relay, either Layer-2 or Layer-3 techniques. 2. Updating the general clause (clause 4.8.1) to explain the possibility of utilizing 5G ProSe intermediate UE2NW relay UE(s) between the remote UE and the 5G ProSe UE2NW relay UE. Adding the abbreviation of RSC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe multi-hop UE-to-network relay is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.8.1, 4.8.2 and 4.8.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ECN	Explicit Congestion Notification

L4S	Low Latency, Low Loss and Scalable Throughput

NPN	Non-Public Network

NTN	Non-Terrestrial Network

PNI-NPN	Public Network Integrated Non-Public Network

RSC	Relay Service Code

SNPN	Stand-alone Non-Public Network

NID	Network Identifier

# * * * * First change * * * *

## 4.8	Use of 5G ProSe UE-to-network relay

### 4.8.1	General

The MC service shall support the capabilities for 5G ProSe UE-to-network relay. For this matter, 5G ProSe Layer-2 and 5G ProSe Layer-3 UE-to-network relaying techniques can be utilized, as described in 3GPP TS 23.304 [17]. The 5G ProSe Layer-3 UE-to-Network relaying technique may be done with or without the support of N3IWF, as described in 3GPP TS 23.304 [17].

A 5G ProSe UE-to-network relay supporting MC service UE provides means of connectivity and relaying of MC traffic to remote MC service UE(s). For this matter, the 5G ProSe UE-to-network Relay Discovery service allows the MC service remote UE to discover a potential UE-to-network relay UE supporting MC service in its proximity as described in 3GPP TS 23.304 [17]. Upon its discovery, the 5G ProSe Direct UE-to-network Relay Communication functionality is utilized to achieve communication to provide the MC service remote UE access to 5GS, and relay MC traffic via the UE-to-network relay UE over the NR PC5 reference point.

5G ProSe UE-to-network relay service is either using one 5G ProSe UE-to-network relay MC service UE (as described in clause 4.8.2), or one 5G ProSe UE-to-network relay MC service UE together with 5G ProSe intermediate UE-to-network relay MC service UE(s) (as described in clause 4.8.x).

### 4.8.2	5G ProSe UE-to-network relay service requirements

In order to enable 5G ProSe UE-to-network relaying capabilities – whether based on Layer-3 or Layer-2 UE-to-network relaying techniques, the MC system provides the appropriate parameters and configurations to the MC service UE(s).

As defined in 3GPP TS 23.304 [17], among these parameters are: Relay Service Code(s) (RSCs) which can be associated to a certain MC service group, User Info ID, ProSe Layer-2 Group ID and ProSe Group IP multicast address. Moreover, the MC service group ID is resolved to the ProSe Layer-2 Group ID and ProSe Group IP multicast address, which are utilized within the 5G ProSe Relay Discovery and 5G ProSe Direct Communication procedures, as described in 3GPP TS 23.304 [17]. Furthermore, the RSCs are utilized to restrict the necessary UE-to-network relay service and related procedures within members of a certain MC service group.

Moreover, in case of 5G ProSe Layer-3 UE-to-network relay with the support of N3IWF, the UE-to-network relay is provisioned with policies and parameters, among others suitable RSC(s), in order to support N3IWF access, as defined in 3GPP TS 23.304 [17].

# * * * * Third change * * * *

### 4.8.x	5G ProSe multi-hop UE-to-network relay service requirements

In order to enable 5G ProSe multi-hop UE-to-network relaying capabilities (either Layer-2 or Layer-3), the MC system provides the appropriate parameters and configurations to the MC service UE(s). As defined in 3GPP TS 23.304 [17], the parameters associated with 5G ProSe Layer-3 multi-hop UE-to-network relay are among others: User Info ID, ProSe Layer-2 Group ID and ProSe Group IP multicast address, RSC(s), UE-to-network relay indicator per RSC (to indicate whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-network relay service), multi-hop indicator per RSC, and maximum number of hops supported per RSC.

NOTE:	The RSC used for multi-hop service is different from the RSC used in clause 4.8.2.

# * * * * End of changes * * * *