3GPP TSG-SA3 Meeting #122	S3-251853

Fukuoka, Japan, 19 – 23 April 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.517 | CR | 0013 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test name and clean up of 33.517 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-03-25 |
|  | D |  |  |  |  |  |  |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Missing test name and some format glitches and typos |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced test name, Format glitches and typos corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing test name. non-standard formats and typos remain and test automation may be affected |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1, 4.2.2.5, 4.2.2.6, 4.2.2.7, 4.2.2.9, 4.2.2.10, 4.2.5, 4.4.1,4.4.2,4.4.3, 4.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

#### 4.2.2.1	Security functional requirements on the SEPP deriving from 3GPP specifications – general approach

In addition to the requirements and test cases in TS 33.117 [2], clause 4.2.2.1, a SEPP shall satisfy the following:

-	It is assumed for the purpose of the present SCAS that a SEPP conforms to all mandatory security-related provisions pertaining to a SEPP in:

-	3GPP TS 33.501 [3]: "Security architecture and procedures for 5G system";

-	other 3GPP specifications that make reference to TS 33.50 [3]1 or are referred to from TS 33.501 [3] Security procedures pertaining to a SEPP are typically embedded in NF/NF service status discovery/subscribe/notify procedures across PLMNs and are hence assumed to be tested together with them.

********** END OF 1st CHANGE *********

********** START OF 2nd CHANGE **********

#### 4.2.2.5	Confidential IEs replacement handling in original N32-f message

Requirement Name: Confidential IEs replacement handling in original N32-f message

Requirement Reference: TS 29.573 [6], clause 5.3.2.3

Requirement Description:

Based on the protection policy exchanged between the SEPPs, the sending SEPP prepares an input for the JWE ciphering and integrity protection as an array of free form JSON objects in the "DataToIntegrityProtectAndCipher" block with each entry containing either a HTTP header value or the value of a JSON payload IE of the API message being reformatted. The index value "encBlockIdx" in the payload part of DataToIntegrityProtectBlock points to the index of a header value or IE value in this input array.

Threat References: TR 33.926 [4], clause G.2.4.2, Exposure of confidential IEs in N32-f message

Test Case:

Test Name: TC_SEPP_CONFIDENTIAL_IE_REPLACEMENT_N32F

Purpose:

Verify that the SEPP under test correctly replaces information elements requiring encryption with the value " encBlockIdx ".

Procedure and execution steps:

Pre-Conditions:

-	System documentation of the SEPP under test, which details how raw public keys/certificates of peer SEPPs are to be configured and how internal log files can be accessed.

-	A second SEPP instance for N32 communication with the SEPP under test, which allows for the creation of custom N32-f messages. This system may be simulated.

-	Both SEPPs are to be configured with a raw public key/certificate of their communication peer to be able to establish a N32-c connection.

-	An arbitrary Data-type encryption policy which includes at least one information element requiring encryption on N32-f. The SEPP under test is to be configured with this policy.

Execution Steps

1.	Both SEPPs establish a mutual N32-c connection.

2.	Via the PLMN-internal interface, the tester provides the SEPP under test with a message to be forwarded to the peer SEPP on N32. This message needs to contain at least one information element that requires encryption according to the locally configured Data-type encryption policy.

3.	The tester captures the related N32-f message after transformation by the SEPP under test.

Expected Results:

Information elements in the original message that require encryption according to the Data-type encryption policy are replaced with the value " encBlockIdx ".

Expected format of evidence:

Evidence suitable for the interface, e.g. text representation of the captured N32-f message.

********** END OF 2nd CHANGE *********

********** START OF 3rd CHANGE **********

#### 4.2.2.6	Correct handling of protection policy mismatch

Requirement Name:	Correct handling of protection policy mismatch

Requirement Reference: TS 33.501 [3], clause 13.2.3.6

Requirement Description:

"When a SEPP receives a data-type encryption or modification policy on N32-c as specified in clause 13.2.2.2, it compares it to the one that has been manually configured for this specific roaming partner and IPX provider. If a mismatch occurs for one of the two policies, the SEPP performs one of the following actions, according to operator policy:

-	Send the error message as specified in TS 29.573 [6], clause 6.1.4.3.2, to the peer SEPP

-	Create a local warning"

Threat References: TR 33.926 [4], clause G.2.3.2, Incorrect handling for protection policy mismatch

Test case:

Test Name: TC_SEPP_POLICY_MISMATCH

Purpose:

Verify that the SEPP under test is able to identify the mismatch between the protection policies manually configured for a specific roaming partner and IPX provider and the protection policies received on N32-c connection, and take action accordingly.

Procedure and execution steps:

Pre-Conditions:

-	Test environment with a peer SEPP instance (as cSEPP), which may be simulated.

-	The SEPP under test and the peer SEPP have mutually authenticated and already established N32-c connection.

-	Exchanging of Data-type encryption policies and Modification policies is required to be performed between the SEPP under test and the peer SEPP.

-	The tester shall have access to the interfaces of the SEPP under test and the peer SEPP.

-	The tester has configured on the SEPP under test the policies for receiving messages, i.e. the Data-type encryption policy d of the peer SEPP and the Modification policy m for the peer SEPP and an IPX provider I used for the peer SEPP.

-	The tester has configured on the peer SEPP the policies for sending, i.e. the peer SEPP's Data-type encryption policy d' and the Modification policy m' for the IPX provider I used for the peer SEPP.

-	 There are three cases to test:

a) 	the data encryption policies d and d' are identical, the modification policies m and m' are different

b)	the data encryption policies d and d' are different, the modification policies m and m' are identical

c)	both the data encryption policies d and d' and the modification policies m and m' are different

NOTE:	The test case below only applies in case the SEPP under test supports manual configuration of the data encryption policy and/or modification policy for the specific roaming partner and IPX provider.

-	The tester has configured on SEPP under test the action to be taken for policy mismatch, which is sending error message.

Execution Steps:

For each of the three cases above, the following is executed:

1.	The peer SEPP sends a Security Parameter Exchange Request message to the SEPP under test including the peer SEPP's Data-type encryption policy d', and the Modification policy m'.

2.	The SEPP under test stores the received Data-type encryption policy d' and the Modification policy m', then compare them with the Data-type encryption policy d and the Modification policy m configured on it.

Expected Results:

The SEPP under test sends an error signalling message to the peer SEPP on the N32-c connection or logs the error.

Expected format of evidence:

Logs and the communication flow saved in a .pcap file.

********** END OF 3rd CHANGE *********

********** START OF 4th CHANGE **********

#### 4.2.2.7	JWS profile restriction

Requirement Name:	JWS profile restriction

Requirement Reference: TS 33.501 [3], clause 13.2.4.9

Requirement Description:

SEPPs and IPXs follow the JWS profile as defined in TS 33.210 [8] with the restriction that they shall only use ES256 algorithm.

Threat References: TR 33.926 [4], clause G.2.4.1, Use of weak JWS algorithm

Test case:

Test Name: TC_JWS_PROFILE_RESTRICTION

Purpose:

Verify that the SEPP under test is able to restrict the JWS profile to only use ES256 algorithm with IPX entities.

Procedure and execution steps:

Pre-Conditions:

-	Network product documentation of the SEPP under test, containing the information about the supported signature algorithms for JWS operation.

-	Test environment with a peer SEPP instance, which may be simulated.

-	Test environment with one node simulating an IPX-provider, which supports JWS operation among others.

-	The SEPP under test and the peer SEPP have mutually authenticated and already established N32-c connection.

-	The tester shall have access to the interfaces of the SEPP under test, the peer SEPP, and the simulated IPX node.

-	The tester has configured both the SEPP under test and peer SEPP for N32-f communication via the simulated IPX node.

-	The tester has configured a JWS profile differently from what is required in TS 33.501 [3], clause 13.2.4.9 in the simulated IPX node for JWS operation.

Execution Steps:

1.	The tester shall check that the supported JWS algorithms in the network product documentation complies with the requirement on the restriction.

2.	The tester sends a N32-f message from the peer SEPP via the intermediate IPX node towards the SEPP under test.

3.	The IPX node modifies one or more attributes of the N32-f message from the peer SEPP and creates a modifiedDataToIntegrityProtect object, which is protected by the IPX node using the JWS algorithm configured by the tester.

4.	The IPX node forwards the modified N32-f message to the SEPP under test.

5.	Based on the internal log files, the tester validates how the received N32-f message is handled by the SEPP under test.

Expected Results:

The modified N32-f message from the IPX node is discarded by the SEPP under test.

Expected format of evidence:

Logs and the communication flow saved in a .pcap file.

********** END OF 4th CHANGE *********

********** START OF 5th CHANGE **********

##### 4.2.2.9	Correct Handling of Inter-PLMN Routing

Requirement Name: Correct Handling of Inter-PLMN Routing

Requirement Reference: TS 29.500 [7], clause 6.1.4.3.3

Requirement Description:

If the SEPP receives an HTTP request from a NF with a request URI containing a telescopic FQDN and with a 3gpp-Sbi-Target-apiRoot header, the SEPP ignores the 3gpp-Sbi-Target-apiRoot header and route the request using the telescopic FQDN.

Threat References: TR 33.926 [4], clause G.2.x.a, Inter-PLMN routing using the incorrect reference

Test Case:

Test Name: TC_CORRECT_INTER_PLMN_ROUTING

Purpose:

Verify that the SEPP under test correctly route the NF request to a remote PLMN when receiving both a 3gpp-Sbi-Target-apiRoot header and a telescopic FQDN contained in the Request URI in the HTTP request from a NF.

Procedure and execution steps:

Pre-Conditions:

-	System documentation of the SEPP under test, which details the methods supported for TLS protection between the NF and the SEPP and how internal log files can be accessed.

-	A peer SEPP instance of a remote PLMN for N32 communication with the SEPP under test, which may be simulated.

-	A NF for sending HTTP request to the remote PLMN of the peer SEPP via the SEPP under test, which may be simulated and supports both telescopic FQDN and the custom 3gpp-Sbi-Target-apiRoot header. The NF is configured with:

-	The NF service profile containing service URI with "https" scheme and an authority of the remote PLMN for communication with the NF producer in the remote PLMN.

-	The telescopic FQDN of the NF producer in the remote PLMN, having the FQDN of the SEPP under test as the trailing part.

-	The FQDN of the SEPP under test.

-	The SEPP under test is configured with:

-	The FQDN of the peer SEPP in the remote PLMN.

-	The security mechanism negotiated with the peer SEPP in the remote PLMN.

Execution Steps

1)	The NF sets up a TLS connection with the authoritative server for the configured telescopic FQDN, i.e. the SEPP under test.

2)	The NF sends a HTTP service request with the request URI containing the configured telescopic FQDN within the TLS connection to the SEPP under test, before which the tester inserts in the HTTP request a 3gpp-Sbi-Target-apiRoot header set to the apiRoot of a NF producer in another PLMN different from the remote PLMN.

3)	The NF sends a HTTP service request within the TLS connection to the SEPP under test, before which the tester inserts in the HTTP request a 3gpp-Sbi-Target-apiRoot header set to the apiRoot of the NF producer in the remote PLMN and changes the telescopic FQDN in request URI to be different from the configured one.

Expected Results:

After step 2), the peer SEPP received the HTTP request from the NF through the SEPP under test.

After step 3), the peer SEPP did not receive the HTTP request from the NF through the SEPP under test

Expected format of evidence:

Evidence suitable for the interface, e.g. screenshot containing the operational results.

********** END OF 5th CHANGE *********

********** START OF 6th CHANGE **********

##### 4.2.2.10	Correct Handling of Custom HTTP Header with PRINS Security

Requirement Name: Correct Handling of the Custom HTTP Header with PRINS Security

Requirement Reference: TS 29.500 [7], clause 6.1.4.3.4

Requirement Description:

The 3gpp-Sbi-Target-apiRoot header is not used between SEPPs if PRINS security is negotiated between the SEPPs.

Threat References: TR 33.926 [4], clause G.2.x.b, Tampering of target API root

Test Case:

Test Name: TC_HANDLING_CUSTOM_HTTPHEADER_WITH_PRINS

Purpose:

Verify that the SEPP under test correctly handle the 3gpp-Sbi-Target-apiRoot custom HTTP header received from a NF when PRINS security is negotiated with the peer SEPP in a remote PLMN.

Procedure and execution steps:

Pre-Conditions:

-	System documentation of the SEPP under test, including the security mechanisms supported for protection between SEPPs.

-	A peer SEPP instance of a remote PLMN for N32 communication with the SEPP under test, which may be simulated.

-	A NF for sending HTTP request to the remote PLMN of the peer SEPP via the SEPP under test, which may be simulated and supports 3gpp-Sbi-Target-apiRoot header. The NF is configured to route all HTTP messages with inter PLMN FQDN as the "authority" part of the URI via the SEPP under test.

-	The SEPP under test is configured with PRINS security as the security mechanism negotiated with the peer SEPP in the remote PLMN.

-	A TLS connection is setup between the SEPP under test and the peer SEPP in the remote PLMN for N32-f forwarding.

Execution Steps

1)	The NF initiates a HTTP message sent to the SEPP under test, which includes the 3gpp-Sbi-Target-apiRoot header containing the apiRoot of the target URI in the remote PLMN and the apiRoot in the request URI set to the apiRoot of the SEPP under test.

2)	The SEPP under test forwards the HTTP request to the peer SEPP in the remote PLMN within the N32-f TLS tunnel.

Expected Results:

The peer SEPP received the protected HTTP Request from the NF through the SEPP under test, in which the apiRoot in the request URI is the apiRoot of the target URI in the remote PLMN and no 3gpp-Sbi-Target-apiRoot header is present.

Expected format of evidence:

Evidence suitable for the interface, e.g. screenshot containing the operational results.

********** END OF 6th CHANGE *********

********** START OF 7th CHANGE **********

### 4.4.1	Introduction

There are no SEPP specific additions to clause 4.4.1 of TS 33.117 [2].

********** END OF 7th CHANGE *********

********** START OF 8th CHANGE **********

### 4.4.2	Port Scanning

There are no SEPP specific additions to clause 4.4.2 of TS 33.117 [2].

********** END OF 8th CHANGE *********

********** START OF 9th CHANGE **********

### 4.4.3	Vulnerability scanning

There are no SEPP specific additions to clause 4.4.3 of TS 33.117 [2].

********** END OF 9th CHANGE *********

********** START OF 10th CHANGE **********

### 4.4.4	Robustness and fuzz testing

The test cases under clause 4.4.4 of TS 33.117 [2] are applicable to SEPP.

The interface defined for the SEPP are in 4.2.4 of TS 23.501 [8].

According to clause 4.4.4 of TS 33.117 [2], the transport protocols available on the interfaces providing IP-based protocols need to be robustness tested. Following TCP/IP layer model and considering all the protocols over transport layer, for SEPP, the following interfaces and protocols are in the scope of the testing:

-	For SBI: the TCP, HTTP2 and JSON protocols.

-	For N32-C: The TCP, HTTP2 and JSON protocols.

-	For N32-F: The TCP, HTTP2 and JSON protocols.

NOTE:	There could be other interfaces and/or protocols requiring testing under clause 4.4.4 of TS 33.117 [2]

********** END OF CHANGE *********