3GPP TSG-SA WG4 Meeting #132	S4-251081

# Japan, Fukuoka, 19 – 23 May 2025	revision of S4-250886

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.510 | CR | 0026 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [iRTCW, 5GMS_Pro_Ph2] Correction of Media AF Handling of PDU Set QoS and Media Transport Parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | iRTCW, 5GMS_Pro_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to clause 5.2.7, a Media Application Provider shall provide a QoS specification in downlink/uplink direction indicating the PDU Set QoS limits, i.e. the minimal delay budget and minimal error rates which are permitted to be requested by a Dynamic Policy invoker on (respectively) downlink and uplink Service Data Flows. In addition, according to clause 5.3.3.2, a Dynamic Policy invoker may indicate the desired downlink/uplink PDU Set QoS parameters in the QoS specification to indicate the desired delay budget and error rate for PDU Sets in the downlink/uplink direction when it attempts to activate a QoS-related Policy Template. However, the usage of PDU Set QoS limits and desired PDU Set QoS parameters is currently unspecified since in the case of the Dynamic Policies feature (clause 5.5.3), the text describing their usage is left in square brackets. Further, it is currently unspecified how the media transport parameters associated to an Application Flow Description indicated by the Media AF are mapped to the MediaComponent object in PCF when the Media AF directly invokes the Npcf_PolicyAuthorization service at the reference point N5, and to the AsSessionMediaComponent object when the Media AF invokes the Nnef_AFsessionWithQoS service at reference point N33. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the brackets around the text that defines the usage of PDU Set QoS limits and desired PDU Set QoS parameters in clause 5.5.3, and rearrange existing paragraphs into a subclause structure in preparation for Rel-19 additions. - Add a new clause 5.5.3.3.2 on mapping of media transport parameters associated to an Application Flow Description into the PCF. - Revise the definition of the mediaTransport‌Parameters property of the ApplicationFlowDescription type defined in clause 7.3.3.2 to reflect its broader role beyond PDU Set identification and end of data burst detection, due to new traffic detection features to be specified in CRs 0018 and 0019. - Clarify in Annex D.1.3 that the Protocol Description and QoS properties are populated in the AsSessionMediaComponent object. Also correct a typo: pduSerQosDl needs to be changed to pduSetQosDl. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - PDU Set QoS limits provided by the Media Application Provider and desired PDU Set QoS parameters that may be provided by the Dynamic Policy invoker are not used by the Media AF. The Media AF cannot indicate the PDU Set QoS parameters to the 5GC. - The Media AF cannot indicate the media transport parameters associated to an Application Flow Description to the 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3, 5.5.3.1 (new), 5.5.3.2 (new), 5.5.3.3 (new), 5.5.3.3.1 (new), 5.5.3.3.2 (new), 5.5.3.4 (new), 5.5.3.5 (new), 7.3.3.2, D.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Create new top-level clause 5.5.3.3 on mapping application-specific PDU handling parameters and rearrange existing paragraphs into a subclause structure in preparation for Rel-19 additions - Add a new clause 5.5.3.3.2 on mapping of media transport parameters - Revise the definition of the mediaTransport‌Parameters property of the ApplicationFlowDescription type - Remove the change proposed for AF-based network assistance in clause 5.5.4 |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.5.3	Policy control interactions for Dynamic Policies

#### 5.5.3.1	General

The Dynamic Policies feature operates at reference point M5 between the Media Session Handler in the Media Client and a Media AF that has been appropriately provisioned with Policy Templates (see clause 5.2.7). The Dynamic Policy API at reference point M5 (see clauses 5.3.3 and 9.3) is specified in a generic way such that the associated functionality in the 5GC may be realised by various means.

NOTE 1:	This clause does not limit the possible set of 5G System exposure functionalities for realising dynamic policies.

In this release, the Media AF converts Dynamic Policies API invocations received at reference point M5 into direct or indirect invocations of the Policy Authorization Service exposed by the PCF, and converts responses from the PCF into their equivalents at reference point M5 for return to the Media Session Handler.

To realise dynamic policies, the Media AF shall interact with the PCF using one of the following methods:

A.	If the Media AF is deployed in the Trusted DN, it may directly invoke the Npcf_Policy‌Authorization service at reference point N5, as specified in TS 29.514 [18].

NOTE 2:	It is the responsibility of the Media AF in this case to discover and track changes to the PCF instance responsible for the PDU Session supporting the media streaming session at reference point M4 using the discovery services provided by the NRF and/or BSF.

B.	If the Media AF is deployed outside the Trusted DN, or if it is more convenient for a Media AF deployed in the Trusted DN to do so, it invokes the Nnef_AFSession‌With‌QoS and/or Nnef_Chargeable‌Party services exposed by the NEF, as specified in clauses 4.4.9 and 4.4.8 respectively of TS 29.522 [19], to indirectly invoke the PCF at reference point N33.

NOTE 3:	Per clause 4.4.9 of TS 29.522 [19], the Nnef_AFSession‌With‌QoS service is realised at reference point N33 by the AsSession‌With‌QoS exposure API. Similarly, the Nnef_Chargeable‌Party service is realised by the Chargeable‌Party exposure API per clause 4.4.8 of [19].

NOTE 4:	Configuration of the NEF endpoint address and access credentials in the Media AF in this case is beyond the scope of the present document.

When the first Dynamic Policy is created by the Media Session Handler for a particular media delivery session (per clause 5.3.3.2), the Media AF shall create an AF application session context in the PCF responsible for the PDU Session corresponding to the M4 application flows indicated in the DynamicPolicy.‌application‌Flow‌Bindings array.

If no corresponding AF application session context already exists, the Media AF shall use the Npcf_‌Policy‌Authorization_‌Create operation at reference point N5 (or, if deployed outside the Trusted DN, the equivalent Nnef_AFsession‌WithQoS service operation at reference point N33) with the appropriate service information to create and provision a new AF application session context. The information in the AppSessionContext‌ReqData shall be derived from the application flow descriptions in the Dynamic Policy Instance resource and/or the requested QoS.

The mapping of application flows listed in the DynamicPolicy.‌application‌Flow‌Bindings array to media components and sub-components of the AF application session context is implementation-dependent. Example mappings are provided in annex D.

When requesting QoS provisioning for a media delivery session, the Media AF shall use the configured Policy Template of the Dynamic Policy Instance to determine the list of the QoS references within altSerReqs. The lowest priority index shall be assigned to the Policy Template with the lowest QoS requirement, and the highest priority shall be assigned to the Service Operation Point requested by the UE (if the UE is allowed to use that operation point).

When a dynamic policy is subsequently destroyed by the Media Session Handler (per clause 4.7.3), the Media AF shall destroy the corresponding AF application session context in the relevant PCF instance.

#### 5.5.3.2	Mapping of Background Data Transfer parameters

When instantiating a Policy Template that has a Background Data Transfer policy, the Media AF needs to populate some of the properties in the ClientBdtSpecification object specified in clause 9.3.3.3 for inclusion in the Dynamic Policy Instance resource returned to the Media Session Handler at reference point M5.

Where the Policy Template references an existing Background Data Transfer policy by including the bdtPolicyId property, in order to populate the properties of the ClientBdtSpecification object the Media AF shall first retrieve the individual Background Data Transfer policy resource referenced by bdtPolicyId from the PCF. The Npcf_‌BDT‌Policy‌Control service operation specified in clause 5.3.3.3.1 of TS 29.554 [46] shall be used for this purpose.

#### 5.5.3.3	Mapping application-specific PDU handling parameters

##### 5.5.3.3.1	Mapping of media transport parameters

If the Media AF directly invokes the Npcf_PolicyAuthorization service at reference point N5 according to TS 29.514 [18] and the Application‌Flow‌Description.‌media‌Transport‌Parameters property of an Application‌Flow‌Binding object is populated, then the Media‌Component object in the PCF at reference point N5 associated with the Application‌Flow‌Binding shall be populated as follows by the Media AF to enable traffic detection by the 5G Core for application-specific PDU handling based on parameters present in the media transport at reference point M4:

-	The MediaComponent.‌protoDescDl property shall be set to the values of the Application‌Flow‌Description.‌media‌Transport‌Parameters object if the Client‌Qos‌Specification.‌desired‌Downlink‌Pdu‌Set‌Qos‌Parameters property is present in the corresponding Application‌Flow‌Binding.

-	The MediaComponent.‌protoDescUl property shall be set to the values of the Application‌Flow‌Description.‌media‌Transport‌Parameters object if the Client‌Qos‌Specification.‌desired‌Uplink‌Pdu‌Set‌Qos‌Parameters property is present in the corresponding Application‌Flow‌Binding.

If the Media AF invokes the Nnef_AFsessionWithQoS service at reference point N33 according to TS 29.522 [19] and TS 29.122 [20] and the Application‌Flow‌Description.‌media‌Transport‌Parameters property of an Application‌Flow‌Binding object is populated, then the AsSession‌Media‌Component object in the NEF at reference point N33 associated with the Application‌Flow‌Binding shall be populated as follows by the Media AF to enable traffic detection by the 5G Core for application-specific PDU handling based on parameters present in the media transport at reference point M4:

-	The AsSessionMediaComponent.‌protoDescDl property shall be set to the values of the Application‌Flow‌Description.‌media‌Transport‌Parameters object if the Client‌Qos‌Specification.‌desired‌Downlink‌Pdu‌Set‌Qos‌Parameters property is present in the corresponding Application‌Flow‌Binding.

-	The AsSessionMediaComponent.‌protoDescUl property shall be set to the values of the Application‌Flow‌Description.‌media‌Transport‌Parameters object if the Client‌Qos‌Specification.‌desired‌Uplink‌Pdu‌Set‌Qos‌Parameters property is present in the corresponding Application‌Flow‌Binding.

##### 5.5.3.3.2	Mapping of PDU Set QoS parameters

If the pduSetQoSLimits property is populated in QosRange.‌downlink‌QoS‌Specification, then the Media‌Component.‌pduSetQosDl object shall be populated as follows by the Media AF:

-	The pduSetDelayBudget property shall be set to the larger value of pduSetQoSLimits.‌pduSetDelayBudget and desiredDownlink‌PduSet‌QosParameters.‌pduSetDelayBudget.

-	The pduSetErrorRate property shall be set to the larger value of pduSetQoSLimits.pduSetErrorRate and desiredDownlink‌PduSet‌QosParameters.‌pduSetErrorRate.

-	The pduSetHandlingInfo property shall be set to the value of pduSetQoSLimits.pduSetHandlingInfo, ignoring the value of desiredDownlink‌PduSet‌QosParameters.‌pduSetHandlingInfo, if any.

Otherwise, the Media‌Component.‌pduSetQosDl object shall be populated directly from the desiredDownlink‌PduSet‌QosParameters object.

If the pduSetQoSLimits property is populated in QosRange.‌uplink‌QoS‌Specification, then the Media‌Component.‌pduSetQosUl object shall be populated as follows by the Media AF:

-	The pduSetDelayBudget property shall be set to the larger value of pduSetQoSLimits.‌pduSetDelayBudget and desiredUplink‌PduSet‌QosParameters.‌pduSetDelayBudget.

-	The pduSetErrorRate property shall be set to the larger value of pduSetQoSLimits.‌pduSetErrorRate and desiredUplink‌PduSet‌QosParameters.‌pduSetErrorRate.

-	The pduSetHandlingInfo property shall be set to the value of pduSetQoSLimits.‌pduSetHandlingInfo, ignoring the value of desiredUplink‌PduSet‌QosParameters.‌pduSetHandlingInfo, if any.

Otherwise, the Media‌Component.‌pduSetQosUl object shall be populated directly from the desiredUplink‌PduSet‌QosParameters object.

#### 5.5.3.4	Subscription to PCF notifications

For each of the Dynamic Policy Instances it is managing, the Media AF shall subscribe to the following PCF notifications on the corresponding AF application session context:

-	Service Data Flow QoS notification control;

-	Service Data Flow deactivation;

-	Resources allocation outcome.

When requesting QoS provisioning for a media delivery session, the Media AF shall use the configured Policy Template of the Dynamic Policy Instance to determine the list of the QoS references within altSerReqs. The lowest priority index shall be assigned to the Policy Template with the lowest QoS requirement, and the highest priority shall be assigned to the Service Operation Point requested by the UE (if the UE is allowed to use that operation point).

When instantiating a Policy Template that has a Background Data Transfer policy, the Media AF needs to populate some of the properties in the ClientBdtSpecification object specified in clause 9.3.3.3 for inclusion in the Dynamic Policy Instance resource returned to the Media Session Handler at reference point M5.

Where the Policy Template references an existing Background Data Transfer policy by including the bdtPolicyId property, in order to populate the properties of the ClientBdtSpecification object the Media AF shall first retrieve the individual Background Data Transfer policy resource referenced by bdtPolicyId from the PCF. The Npcf_‌BDT‌Policy‌Control service operation specified in clause 5.3.3.3.1 of TS 29.554 [46] shall be used for this purpose.

When a dynamic policy is subsequently destroyed by the Media Session Handler (per clause 4.7.3), the Media AF shall destroy the corresponding AF application session context in the relevant PCF instance.

# * * * * Second change * * * *

#### 7.3.3.2	ApplicationFlowDescription type

This data type is used to declare the properties of an application data flow to the Media AF during the course of a media delivery session. Its properties are used to describe a Service Data Flow to the 5G Core for the purpose of application traffic detection.

Table 7.3.3.2-1: Definition of type ApplicationFlowDescription

| Property name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| filterMethod | SdfMethod | 1..1 | The filtering method used to identify packets belonging to this application flow (see clause 7.3.4.2). |
| packetFilter | IpPacketFilterSet | 0..1 | Description of the application flow in terms of packet header field values (see below). |
| domainName | string | 0..1 | Description of the application flow in terms of the Fully-Qualified Domain Name (FQDN) of the Media AS targeted at reference point M4 (see below). |
| mediaType | MediaType | 0..1 | The type of media carried by this application flow (see NOTE 1). |
| mediaTransport‌Parameters | Protocol‌Description | 0..1 | The set of media transport protocol parameters to be used by the 5G Core to detect traffic on this application flow for the purpose of application-specific PDU handling based on parameters present in the media transport at reference point M4 (see clause 5.5.3.3.1 and NOTE 2). |
| NOTE 1: Enumeration MediaType is specified in clause 5.6.3.3 of TS 29.514 [18]. NOTE 2: Data type ProtocolDescription is specified in clause 5.5.4.13 of TS 29.571 [33]. |  |  |  |

Exactly one of the following properties shall be populated: packetFilter, domainName.

# * * * * Third change * * * *

## D.1.3	QoS mapping for Dynamic Policy at reference point N33

When the Media AF invokes the Nnef_AFsessionWithQoS service at reference point N33 according to TS 29.522 [19] and TS 29.122 [20], each DynamicPolicy resource is mapped by the Media AF to an AsSession‌WithQoS‌Subscription resource in the NEF. The qosReference, protoDescDl, protoDescUl, pduSetQosDl and pduSetQosUl properties are not populated in this resource; the QoS requirements of the Service Component are instead populated in the AsSession‌Media‌Component object.

Each Application‌Flow‌Binding object of the Dynamic Policy Instance (each one representing a distinct Service Component) is associated with a different AsSession‌Media‌Component object in the NEF, as shown in figure D.1.3-1.

The QoS requirements of the Service Component are instead populated in the AsSession‌Media‌Component.

The descriptions of the downlink and/or uplink directions of the application flow corresponding to the Service Component are populated the flowInfos array of the AsSession‌Media‌Component unless a qosReference is cited in the corresponding QosRange.

Figure D.1.3-1: Mapping of ApplicationFlowBinding to NEF AsSessionMediaComponent at reference point N33