3GPP TSG-SA3 Meeting #122	S3-252267

# Fukuoka, Japan, 19 - 23 May 2025														revision of S3-251998

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.310 | CR | 0211 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for CMP over TLS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | ACM_SBA |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As discussed in SA3 #120, OAM signature as initial trust is susceptible to the credential stealing attacks, and need to be fixed by making the CMP over TLS mandatory for the Initial Request message. Besides, if the operator root certificate is not pre-configured in the NF and carried in the extraCerts field for the Initialization Response, there may be threats that the root certificate is replaced or changed. This also can be fixed by making the CMP over TLS mandatory. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Mandating CMP over TLS for the scenarios of using OAM signature as initial trust and when root certificates are not pre-configured to the NF and carried in the extraCerts field for the Initialization Response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unsecure NF enrollment procedures in some scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.1.4.2, 10.3.1.4.3, 10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Spelling correction and formatting into a bullet list for clarity. |  |  |  |  |  |  |  |  |

**** START OF 1st CHANGES ****

##### 10.3.1.4.2	Initialization Request

The Initialization Request as specified in IETF RFC 4210 [4] shall contain exactly one CertReqMessages as specified in IETF RFC 4210 [4] and IETF RFC 4211 [19], i.e., the size of the sequence for CertReqMessages shall be 1 in all cases.

The following profile shall be applied to the CertReqMessage field and its sub-fields:

-	The subjectAltName field of the CertTemplate contains the nfInstanceID of the NF.

-	The publicKey field of the CertTemplate is mandatory and shall contain the public key of the NF to be certified by the RA/CA. The private/public key pair may be pre-provisioned to the NF, or generated inside the NF, or generated by a certificate management NF acting on behalf of the NF, for the CMPv2 protocol run. The format of this field shall follow IETF RFC 5280 [14].

-	The CertReqMessage shall contain a POP field of type ProofOfPossession. The POP field shall contain a signature field of type POPOSigningKey. The algorithmIdentifier field of the POPOSigningKey field shall contain the signing algorithm which is used by the NF to produce the Proof-of-Possession value, i.e., the signature within POPOSigningKey field.

-	If the poposkInput field of type POPOSigningKeyInput within POPOSigningKey field is used, the sender field within POPOSigningKeyInput shall be mandatory and shall contain the identity of the NF Instance (“nfInstanceID”).

NOTE 1:	According to IETF RFC 4211 [19], the poposkInput field is mandatory if either the subject field or the publicKey field of the CertTemplate field is omitted.

NOTE 2:	According to IETF RFC 4211 [19], the sender field of POPOSigningKeyInput is used only if an authenticated identity has been established by the sender.

The PKIMessage sent by the NF is signed by the generated or provided private key.

NOTE 3:	During the initial trust set-up procedure (specified in subclause 10.2.3), the NF is required to authenticate the origin of the "ir" message to the operator CA/RA. If the selected mechanism for initial trust is an OAM certificate, the NF signs the "ir" using the OAM certificate corresponding private key and includes the digital signature in the PKIMessage. The OAM certificate and any intermediate certificates are included in the extraCerts field of the PKIMessage carrying the "ir". The operator CA/RA verifies the digital signature on the "ir" message against the preprovisioned OAM local CA root certificate using the certificate(s) sent by the NF. If the selected mechanism for initial trust is an IAK, MAC protection is used for the "ir" message, based on the IAK value shared between the entity end entity and the operator CA/RA and distributed out of band prior to the initial trust set-up procedure. If the selected mechanism for initial trust is a signature of certain NF profile selected parameters, the signature is included in the "ir" message and CMP over TLS for the client to authenticate the operator CA is to be used to protect the procedure. The operator CA/RA uses the corresponding public key distributed out of band prior to the initial trust set-up procedure, to verify the signature, i.e., the authenticity of the "ir" message.

**** END OF 1st CHANGES ****

**** START OF 2nd CHANGES ****

##### 10.3.1.4.3	Initialization Response

The Initialization Response as specified in RFC 4210 [4] shall contain exactly one generated NF certificate, i.e., the size of the sequence for CertResponse shall be 1 in all cases.

The following profile shall be applied to the CertRepMessage field and its sub-fields:

-	The generated certificate shall be transferred to the NF in the certifiedKeyPair field of the CertResponse field. The transfer shall not be encrypted (i.e., the certificate field in CertorEncCert is mandatory).

The extraCerts field of the PKIMessage carrying the initialization response shall be mandatory and shall contain the operator root certificate (or ‘full chain’ if NF contacted to SubCA using CMPv2) and the RA/CA certificate (or certificates if separate private keys are used for signing of certificates and CMP messages) if the operator root certificate is not pre-configured to the NF. If the RA/CA certificate(s) are not signed by the operator root CA, also the intermediate certificates for the chain(s) up to the operator root certificate shall be included in the extraCerts field. If additional (self-signed) Root CA certificates are required, they shall be carried in the extraCerts field or caPubs field of the PKIMessage. Since extraCerts field is not under CMP message integrity protection, CMP over TLS shall be used as a security transport mechanism if the root certificates are carried in the extraCerts field. Since CMP already supports integrity protection for caPubs field, the use of security transport mechanisms is optional.

**** END OF 2nd CHANGES ****

**** START OF 3nd CHANGES ****

### 10.3.2	CMPv2 transport

Transport of CMPv2 messages between end entities (network elements) and RA/CA shall be done using HTTP-based protocol as specified in IETF RFC 6712 [18], with the exception that support for TLS is not mandated.

Support is mandatory for communication initiated by the end entities where every CMP request triggers a CMP response message from the CA or RA. Support for RA/CA initiated HTTP requests (i.e., announcements) is not mandatory.

TLS shall be mandatory for the following CMP messages:

- Initialization Request when OAM signature is used as initial trust, and

- Initialization Response when the root certificate is carried in extraCerts field.

NOTE 1:	CMP provides built-in integrity protection and authentication. For optional usage of HTTP over TLS (HTTPS) according to RFC 9110 or virtual private networks see IETF RFC 6712 [18].

NOTE 2:	If CMP over TLS is implemented, the certificates used in this TLS communication are to be provided offline.

**** END OF 3nd CHANGES ****