3GPP SA# 108	SP-250741

Prague, CZ, June 10 – 13, 2025

Source:	Nokia, Siemens AG, Deutsche Telekom

Title:	Comments to objection on “New WID on Industrial Networks Loop Prevention”

Document for:	Discussion

Agenda Item:	6.5.2

Abstract of the contribution: This paper provides comments to the objection documented in SA2 report related to S2-2505919.

# 1.	Discussion

## 1.1	Overview

During the SA2#169 meeting, there were discussions related to WID on IIoT which is documented in S2-2505919. There was objection from one company (Huawei) and the company shared reasons for objection to be added to the report. This paper provides comments on the reasons for objection.

## 1.2	Details and comments

The text in italics is the information related to reasons for objection provided in meeting report.

The text in bold face is the corresponding comment / clarification.

Loop is a technical problem since Rel-15 Ethernet type PDU session, in the previous releases, the specification states that this is left for implementation, so Rel-15 ~ Rel19 works well to use UPF implementation to address the loop problem for Ethernet type PDU session.

In Rel-15 Ethernet type PDU sessions were introduced, but loop prevention topic started with Rel-16 when 5GS was enabled to act as an IEEE 802.1Q bridge. In a standardized bridged network, the 5GS needs to interoperate with other IEEE 802.1Q bridges for MSTP/RSTP communication. MSTP/RSTP support by UPF needs has not been addressed in previous releases.

TSN needs special transmission handling and resource reservation for the Ethernet packets, but TSN has nothing special to address loop prevention, this loop prevention issue is general for Ethernet type PDU session.

Time-Sensitive Networking (TSN) is a set of standards specified by IEEE 802 which also includes the IEEE Std 802.1Q-2022. IEEE Std 802.1 group commented in LS S2-2004782: “Loop prevention is a must in a bridged network”. Based on this, loop prevention is not (only) a general topic for Ethernet type PDU session but mainly an integration issue when 5GS needs to be integrated as IEEE 802.1Q bridge. Indeed, the specification effort on loop prevention will work in a standard (IEEE 802) bridged network either with or without TSN features.

Interested companies want to optimize the way to support “loop prevent” mechanism, that is understood and we don’t disagree on the optimization support of loop prevention of Ethernet type PDU session.

Loop prevention is not optimization. It is essential for IEEE 802.1Q bridges to support all types of deployments. This poses requirements on the 3GPP system when enabled to act as an IEEE 802.1Q bridge. As a mandatory requirement of Industrial profile in IEC/IEEE 60802, MSTP serves for multiple VLANs in scalable manner. Loop prevention is fundamental in Ethernet, as the physical and logical (VLAN) topologies shall be loop free to avoid crashes. So, in multi-vendor interoperations the IEEE standard must be followed.

Our concerns of STP solution is that the STP protocol is not widely used for the legacy routers and switches on the market, and see the same justification as in the SID “many end switches do not have the loop detection capability”, so the marketing motivation to force UPF Ethernet Bridge to support STP protocol is questionable.

RSTP was then incorporated into IEEE 802.1D-2004 making the original STP standard obsolete. That is, there is no standard for STP for more than two decades. STP has been replaced by RSTP/MSTP. Nearly all vendors of managed bridges support MSTP/RSTP, including Huawei. ( https://support.huawei.com/enterprise/en/doc/EDOC1000114001/47775abd/overview-of-spanning-tree-protocols-on-huawei-switchesstates ).

The technical challenge is that the support of the STP is not an easy work, as Huawei evaluated, STP functionality is a big feature to standardize, the 5G needs to participate the management of the topology of the whole network (including 5G segment), management of the state of 5GS bridge and relative interfaces (N6, N19, dynamic PDU Session N3/N9), handling of the periodic BPDU messages, and more importantly, it requires the other end devices (like UE) connecting to 5GS as STP capable device for interworking. And the delay performance of the STP in practice is questionable to be used in critical industrial ethernet, MRP behaves even much better that STP in practice.

The WID aims at support of RSTP/MSTP according to IEEE 802.1Q as selected by IEC/IEEE 60802.  3GPP internal interfaces are not affected as we clarified. In Rel-19 Stage 1 requires supporting MSTP as defined in IEC/IEEE 60802.

Instead of STP, there are other already well-known ways to address loop problem in the legacy routers and switches, those mechanisms (MAC FLAP control or Loop Detection or Loopback Detection) other than STP that are widely used by routers, since they are more simple, more general, and friendly to the other neighbor devices in the network. If those mechanisms are used, then the UPF, behaving as a Ethernet Bridge, don’t need any STP configuration or functionality, the UPF blocks the PDU session if it detects any loop caused by such PDU session, and forward any received BPDUs to the downstream nodes. That is it.

In the 3GPP system we should not have many options on supporting loop prevention, so it is in order to take a standardized approach with limited options. A standardized loop prevention is a must have to minimize integration costs of 3GPP solutions. Analysis of STP or comparison of any non-standard solutions with required standard solutions is out of scope of this work.

Huawei compromise is to make this as study item with general loop prevention support with bigger TU (2.5+2.5 at least), same as the original objectives in previous meetings, but this is not reasonable considered in any revision.

The WID explains all the corresponding requirements coming from IEEE, 5G-ACIA, SA1, which precisely mention MSTP is a requirement. The WID aims at specifying MSTP/RSTP support in the 3GPP system, as required by stage 1. So, the work should be limited to the introduction of MSTP/RSTP

# 2	Way forward

We strongly recommend that the above information is considered and have SA2 progress on the WID as documented in SP-250750, which aims at Industrial Ethernet networks (Industrial Automation environments) progress on their deployments effectively where all vendors of managed bridges already have been supporting MSTP/RSTP. The Work Item is proposed with a 1 TU normative work, assuming we will also need a discussion in e.g. a drafting session to finalize the level of details that would be captured in the TSs affected by the Work Item.