3GPP SA# 108	SP-250750

Prague, CZ, June 10 – 13, 2025

3GPP SA WG2# 169	S2-2505919

Fukuoka, Japan, May 19 – 23, 2025

Source:	Nokia, Siemens AG, Mitsubishi Electric, ZTE, Deutsche Telekom, Verizon, Samsung, Robert Bosch GmbH

# Title:	New WID on Industrial Networks Loop Prevention

# Document for:	Approval

# Agenda Item:	6.5.2

Abstract: This document includes all WTs and detailed discussion and can be used as place holder for potential SI creation if agreements can be reached in any of the WTs to conduct study in Rel-20.

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Industrial Networks Loop Prevention

{Free text. It has to be the same as in the "Title:" section above. Studies have to start by "Study on"}

Acronym:	Vertical_LAN_Loop_Ph2

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| x | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

{"Parent" Work Item refers to the related, earlier-Stage, Work Item, e.g. the related Stage 1 Work Item shall be indicated here when a Stage 2 normative Work Item or Study Item is presented. "Parent" Work Item can also refer to the related preceding Study Item e.g. the related Study Item and the earlier-stage Work Item shall be indicated here when a normative-work Work Items is started. List here all parent Work Items of which requirements are either fully or partially covered by the proposed Item. }

{This section is mandatory to be filled out by the rapporteur. This section is to be filled with care: it indicates to the companies monitoring the parent Work Item that it will be addressed in this study/work item.}

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820017 | Stage 2 of Vertical_LAN | Vertical_LAN |
| 800007 | Service requirements for cyber-physical control applications in vertical domains | Stage 1 work item |

Dependency on non-3GPP (draft) specification:

# 3	Justification

In industrial network, most of the communication takes place through the Ethernet based networks. These networks have been deployed over time as factories/industries evolved. To provide wireless communication in such a deployment, 5G systems need to be integrated transparently as a logical bridge as introduced in R16. Industrial ethernet networks are already available and deployed and 5GS must adapt to such industrial ethernet networks, needing to support such brownfield integrations. In such networks, loops may occur when two or more UEs are served by different 5G logical bridges and are connected to each other through a wired network.

The above figure shows an example with three potential loops when two logical 5GS bridges are integrated into an Ethernet Network.

In 2023 and 2024 5G-ACIA investigated potential gaps of 5GS when it acts as one or more IEEE 802.1Q bridges in an Industrial Automation network and identified the missing MSTP functionality as a critical issue, especially for non-stream IA traffic. (LS from 5G-ACIA “5G-ACIA-LS-2024-008” to SA and SA2).

LS S2-2004336 from IEEE 802.1 group comments on SA2 specification as “Loop prevention is a must in a bridged network because a forwarding loop may cause a network meltdown” in context to TS 23.501 NOTE 10:	This Release of the specification does not guarantee that the Ethernet network remains loop-free. Deployments need to be verified on an individual basis that loops in the Ethernet network are avoided. Further, SA1 specified in TS 22.261 Rel-16 the requirement to support STP. This requirement was modified in Rel-17 to support RSTP, and in Rel-19 to support MSTP as required in IEC/IEEE 60802.

5GS Bridge that is integrated into an ethernet network may be part of a loop (e.g. when the 5GS Bridge resides between the ethernet switches that are part of a physical redundant path). The current specification has not addressed how the loop prevention operations of MSTP/RSTP can be supported on 5GS Bridge.

# 4	Objective

WT#1: Provide loop prevention functionality (i.e., MSTP/RSTP) such that the 5GS logical bridge’s behavior meets the Bridge component requirements for MSTP/RSTP specified in IEC/IEEE 60802.

Note 1:	The Spanning tree algorithm is executed at UPF. Optionally (i.e., when there is a TSN-AF or similarly if a TSCTSF is deployed) configuration/results can be via the existing PMIC/UMIC container framework. In other cases, i.e. when there is no TSN-AF nor TSCTSF, it is possible to use pre-configuration, so that the support for MSTP/RSTP is possible even without any TSN-AF or TSCTSF.

Note 2: 	Some preparatory discussions will be required to scope the level of details to be documented in 3GPP specs, similar to other IEEE functionality support.

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 0 | 1 | No | self-contained |

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| {Possible values: "TS" or "Internal TR" or "External TR". See Note 1} | {e.g. "22.XXX" or actual number if known} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {e.g. "TSG#87"} | {e.g. "TSG#89"} | {<FamilyName>, <GivenName>, <Company>, <email address>. See Note 2} |

{Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Add MSTP/RSTP support | TSG SA#113 (September 2026) |  |
| TS 23.502 | Add MSTP/RSTP support | TSG SA#113 (September 2026) |  |

# 6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

# 7	Work item leadership

SA WG2

# 8	Aspects that involve other WGs

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

Nokia

Siemens AG

Mitsubishi Electric

ZTE

Deutsche Telekom

Verizon

Samsung

Robert Bosch GmbH