3GPP TSG-WG2 Meeting #168	S2-2504132

# Göteborg, Sweden, April 07-11, 2025 												    (was S2-2502964)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0193 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Assigned trajectory event for in-flight monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Step 5 of the in-flight monitoring procedure, it is specified that the UAS NF / NEF performs Step 2-4 of clause 5.3.3 for UAV’s trajectory monitoring. However, these clause can only be used to determine UE’s presence in the area in cases AMF event exposure for UE’s presence in the area of interest is used or UE’s location information in cases location services from GMLC are invoked instead. However, a flight plan and assigned trajectory can also include altitude component and a time schedule that needs to be monitored when the UAV UE moves from a starting to a destination point. For that reason, SA2 has introduced a new event fiter for AMF event exposure (“Assigned Trajectory”, see Table 5.2.2.3.1-1 of TS 23.502), and this newly introduced event filter is now used for UAV UE’s trajectory monitoring in the USS changeover procedure (see Step 14a in clause 5.13.2 of TS 23.256). Therefore, the in-flight monitoring needs to be aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Assigned Trajectory filter (instead of UE’s presence in the Area of Interest) is used for UAV’s trajectory monitoring via AMF event exposure service. -Adding a clarification in Step 1 of clause 5.12.3 that USS can request more than one monitoring mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete/partial functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

### 5.12.3	Procedure for NEF Assisted In-flight Monitoring

NFs

USS/UTM

UAS NF/ NEF

GMLC

1. Nnef_UAVFlightAssistance_Create request

2. Determining NFs and services for the request

3. Nnef_UAVFlightAssistance_Create response

4. Steps 2-5 of clause 5.3.2 for UAV location monitoring and reporting

5. UAV flight trajectory/presence monitoring

6. Steps 2-5 of clause 5.3.4 for a list of aerial UEs in a geographical area

7. Obtaining QoS Sustainability analytics as specified in clause 6.9.1 of TS 23.288 [20]

8. Location monitoring via Ranging/Sidelink positioning services as specified in clause 6.20.4 of TS 23.273 [8]

9. Nnef_UAVFlightAssistance_Notify

Figure 5.12.3-1: Procedure for NEF Assisted In-flight Monitoring

NOTE:	The NFs in Figure 5.12.3-1 represents all the other NFs beside GMLC and NEF/UAS NF involved in the procedure.

1.	The USS/UTM sends an Nnef_UAVFlightAssistance_Create request with an indication to provide the in-flight monitoring of a target UAV to NEF. The request includes an identifier of the UAV (e.g. GPSI) and information about the flight path(s), monitoring mode(s) (e.g. UAV location monitoring, flight trajectory monitoring, flight environment monitoring, Ranging/Sidelink positioning location monitoring, QoS Sustainability analytics), report requirements (e.g. report format (event triggered or periodically), assistance information (e.g. the monitoring results, information generated from the monitoring results)). Additionally, the request may include the planned flight path(s) if the flight trajectory is set for the monitoring mode, a list of UAVs in the areas of interest if the flight environment is set for the monitoring mode, preset value for distance if the monitoring mode is set to Ranging/Sidelink positioning.

2.	The NEF maps the parameters in the Nnef_UAVFlightAssistance_Create request from the USS/UTM to information used by the 3GPP system. The NEF determines services needed for the message and relevant NFs, e.g. NEF service on UAV tracking (UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), GMLC service (Ranging/Sidelink Positioning location), NWDAF analytics service (QoS Sustainability Analytics).

3.	If the NEF is not able to provide the requested information for in-flight monitoring, the NEF rejects the USS/UTM's request for the in-flight monitoring and the NEF may include in the Nnef_UAVFlightAssistance_Create response the details about the reason for rejecton, if available.

4.	For UAV's location monitoring, the NEF executes steps 2-5 of clause 5.3.2 to obtain the location of the target UAVs.

5.	For UAV’s flight trajectory monitoring, the NEF may use the AMF event exposure service with Event ID = Assigned Trajectory (specified in clause 5.2.2.3.1 of TS 23.502 [3]) to get notified when the UAV deviates from the assigned flight plan provided in Step 1.

When the NEF receives a report from the AMF indicating the UAV is not following the assigned flight path/trajectory, the NEF may invoke an Ngmlc_Location_ProvideLocation service request to the GMLC to get a accurate position of the UAV for which the GMLC performs the 5G-MT-LR procedure (as specified in TS 23.273 [8]) and provide the UAV location to the NEF/UAS NF in an Ngmlc_Location_ProvideLocation service response.

6.	For flight environment monitoring, the NEF executes steps 2-5 of clause 5.3.4 to obtain the information of aerial UEs presence in the same geographic area as the target UAV.

7.	For QoS Sustainability analytics, the NEF requests the NWDAF service as described in clause 6.9.1 of TS 23.288 [20]. The subscribe/request message include an identifier of the UAV (e.g. GPSI) obtained in step 4 and the UAV's flight path information; other parameters included in the request are described in clause 6.9.1 of TS 23.288 [20]. The list of UAVs in the area of interest is obtained in step 8 and it may be used by the NEF to request the NWDAF on QoS Sustainability analytics.

8.	For location monitoring via Ranging/Sidelink positioning service, the NEF requests the GLMC service for Ranging/Sidelink Positioning location results as described in clause 6.20.4 of TS 23.273 [8], the procedure of SL-MT-LR for periodic, triggered Location Events (steps 1-20 for initiation of the monitoring, steps 21-31 for periodic monitoring). If the NEF does not have a list of UAVs in the area of interest, the NEF obtains the list of UAVs in the areas of interest in step 6 and use the obtained list as an input to request GLMC service for Ranging/Sidelink Positioning location.

9.	The NEF notifies the USS/UTM using an Nnef_UAVFlightAssistance_Notify about the monitoring results (e.g. the location of the target UAV, indication on whether the UAV follows the flight path, the target UAV's presence in areas of interest, information about whether any other aerial UE/UAV is present in the same geographic area as the target UAV, distance to other UAVs and whether this distance is smaller than the preset value(s)).

Steps 4-9 may be repeated for report monitoring results periodically.

>>>> End of Changes <<<<