3GPP TSG-SA WG6 Meeting #66	S6-251395

Goteborg, Sweden, 7th – 11th April 2025	(revision of S6-251176)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0279 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Preconditions of nested API authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPP |  |  |  |  |  | Date: |  |  | 2025-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 8.32.3 specifies as precondition that the resource owner function can communicate with the API invoker, while the CAPIF architecture does not specify any reference point between the resource owner function and the API invoker |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Replacement of the faulty precondition by “The resource owner function (ROF) can communicate with the CCF (Authorization function)”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect procedure of nested API authorization, leading to misunderstandings and implementation mistakes |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.32.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************************* Start of Changes *****************************/

### 8.32.3		Procedure

Figure 8.32.3-1 illustrates the procedure to obtain authorization information in a nested API invocation, in which an API exposing function receiving the service API invocation request interacts with another API exposing function to provide the service.

Pre-conditions:

1.	The resource owner function can communicate with the CCF (Authorization function).

2.	The API exposing functions 1 and 2 are in the same trust domain.

Figure 8.32.3-1: Procedure for obtaining authorization information in a nested API invocation

1.	The API invoker requests authorization information to invoke the service API exposed by API exposing function 1.

NOTE 1:	This step use either the procedure to obtain authorization to access service API specified in clause 8.11 or the procedure to obtain authorization to access service API that involve the resource owner function as specified in clause 8.31.

2.	The API invoker sends a service API invocation request to API exposing function 1 with the authorization information received in step 1.

3.	Based on the service API invocation request, API exposing function 1 decides to invoke another service API exposed by API exposing function 2.

4.	API exposing function 1, acting as an API invoker, obtains from the CCF the authorization information to access the service API exposed by the API exposing function 2.

NOTE 2:	To obtain the authorization information further interaction with the API invoker can be avoided if API exposing function 1 exchanges the authorization information provided in the API invoker's request (in step 2) with authorization information for accessing to access the service API exposed by API exposing function 2. The security mechanism involved in this exchange are out of scope of the present document.

5.	API exposing function 1, acting as an API invoker sends a service API invocation request to API exposing function 2 with the authorization information received in step 4.

6.	API exposing function 1 receives the service API invocation response resulting from the service API invocation once API exposing function 2 has checked whether the API invoker is authorized to invoke that service API based on the authorization information.

7.	The API invoker receives the service API invocation response resulting from the service API invocation.

# /******************************* End of Changes *****************************/