3GPP TSG-SA3 Meeting #122																			S3-252189

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.310 | CR | 0214 | rev | - | Current version: | 19.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Making NF type as pre-registered parameter in CA/RA for IAK method |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As discussed in SA3 #120, the NF type is mandatory in the NF certificate and can also be a part of the NF identity. For IAK method as initial trust option, the NF type shall be pre-registered in the operator CA/RA by OAM system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the scenario of using IAK method as initial trust, the NF type shall be pre-registered in the operator CA/RA by OAM system. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IAK method as initial trust mechanism can not issue the certificate with the NF type of the NF instance that request the certificate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.2, 10.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN OF CHANGES ***

## 10.2	Set up of initial trust

### 10.2.1	General

This clause describes the architecture and the procedures for the set up of the initial trust between the operator CA/RA and the NF or the end entity acting on behalf of the NF for the certificate enrolment.

### 10.2.2	Architecture

The protection of the NF certificate enrolment procedure has the prerequisite to build initial trust between the 5GC NF and the operator CA/RA.

OAM facilitates the initial trust establishment between NF and operator CA/RA.

Figure 10.2.2-1 depicts the general schema to set up initial trust between 5GC NF and operator CA/RA.

Figure 10.2.2-1: Initial trust general schema

The assumption is that the OAM system is trusted for the operator CA/RA, i.e., the trust between the OAM system and the operator CA/RA shall have been preestablished.

The OAM system of the 5GC NF, which instantiates the NF, shall provide it with the initial trust to be used during the 5GC NF operator certificate enrolment procedure, as part of the initial configuration of the NF.

Three options are described below on how to set up the initial trust. The initial trust can be implemented by 1) OAM issued certificates, 2) an Initial Authentication Key (IAK), or 3) OAM issued signature of certain NF profile parameters, at least including the NF instance ID. The initial trust shall be implemented by one of these mechanisms. The requirements are the following:

1. The deployment of the initial trust as OAM certificate requires the configuration of a local CA used for 5GC NFs within SBA domain (specifically within the same security trust domain of the NF managed by the OAM system), and the configuration of the root CA public certificate of the local CA as trust anchor in the operator CA/RA.

2. The deployment of the initial trust as IAK requires the distribution of such key out-of-band between the operator CA/RA and the NF via OAM system. This initial credential is used as initial trust by the operator CA/RA to authenticate the NF. The management aspects of IAK are left to implementation (e.g., provisioning, one time use, etc.).

3. The deployment of the signature of certain NF profile selected parameters requires the pre-configuration of such signature out-of-band in the NF. The signature is used by the operator CA/RA to authenticate the NF. The selection of the parameters considered in the signature is left to implementation.

NOTE 1: When the pre-configured signature option is used, the signature can be carried in the regToken control field. The regInfo field may indicate that the certificate request message carries a trusted entity’s signature for certain NF profile parameters.

NOTE 2: If the initial trust is established by certificate, care needs to be taken to avoid the misuse of such certificate (e.g., by revoking the certificate, limiting the validity, etc.).

The operator CA/RA shall be able to verify that the NF Instance Id in the certificate enrolment request belongs to the NF instance requesting the certificate.

### 10.2.3	Procedure

Figure 10.2.3-1 depicts the procedure for the set-up of initial trust in 5GC NFs.

Figure 10.2.3-1: Procedure for set up of initial trust

Prerequisites of the procedure:

-	If the initial trust has been established by initial digital certificate or OAM signature, the public root certificate of the OAM local CA or the OAM certificate shall be configured as trust anchor for the verification of the initial trust in the operator CA/RA.

-	If the initial trust has been established by IAK, the key shall have been securely distributed to the NF by OAM.

-	All other necessary parameters to enable the communication with operator CA/RA such as the address shall have been configured by OAM.

1. The OAM system shall configure the initial trust used for the enrolment of the operator certificate in the 5GC NF. If the initial trust is established by an initial certificate during or after the NF initialization, the local CA in the OAM system should issue such initial certificate to the NF as part of its configuration. This certificate shall be configured with the NF Instance Id in SubjectAltName field. The fetching procedure of this certificate by the NF is left to implementation.

If the initial trust has been established by IAK, certain NF profile parameters (at least including the NF instance ID and NF type) shall be pre-registered in the operator CA/RA by OAM system.

2. The 5GC NF generates the private-public key pair and the request of an end entity operator certificate to the operator CA/RA.  The certificate enrollment request shall include the initial trust (initial OAM issued certificate, signature of NF profile parameters, or IAK) fetched in step 1 and the NF Instance Id in SubjectAltName field of the certTemplate. The NF shall sign the request with its private key and includes the digital signature in the request.

If the initial trust is established by an initial certificate, the request shall include the certificate chain of local CA.

If the initial trust is established by IAK, the Operator CA/RA shall validate certificate enrollment request using the IAK.

If the initial trust is established by a signature of NF profile parameters, the operator CA/RA shall verify the signature in the certificate enrollment request .

NOTE: Some 5GC NF implementations may include separate certificate management function(s) acting on behalf of the NF towards the CA/RA. The requirements of this procedure are applicable to those functions.

3. Certificate enrolment request is sent to the operator CA/RA.

4. The operator CA/RA shall verify the initial trust in the request from the NF and the identity of the NF (NF Instance Id). If verified, the operator CA/RA shall generate the end entity operator certificate for the NF. Specifically, by checking the digital signature on the certificate enrolment request against the trust anchor configured in step 1, and the proof of possession of the private key for the requested operator certificate. It shall verify as well that the NF Instance Id in the SubjectAltName field of the certTemplate for the Certificate Enrolment Request corresponds to the NF Instance Id of the initial OAM issued certificate, or the NF instance ID signed by the OAM issued signature. If those verifications are successful, the operator CA/RA shall generate an end entity certificate for the 5GC NF.

If the initial trust has been established by IAK, the operator CA/RA shall issue the certificate with the pre-registered NF profile parameters (at least including NF instance ID and NF type) for the 5GC NF after successful verification of the initial trust.

5. The operator CA/RA shall include the end entity certificate for the requestor NF in certificate enrolment response.

*** END OF CHANGES ***