3GPP TSG-SA3 Meeting #122	S3‑252424

# Fukuoka, Japan,  19 - 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR | 0727 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security aspects of 5G satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_SEC |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Security for 5G satellite access phase 3 is not specified yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide specification for security aspects of 5G satellite access phase 3 in a new normative annex in TS 33.401. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of 1st Change ***

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 33.102 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Access Security Management Entity: entity which receives the top-level keys in an access network from the HSS. For E-UTRAN access networks, the role of the ASME is assumed by the MME

Activation of security context: the process of taking into use a security context.

Authentication data: Data that is part of a security context or of authentication vectors.

Chaining of KeNB: derivation of a new KeNB from another KeNB (i.e., at cell handover)

Current EPS security context: The security context which has been activated most recently. Note that a current EPS security context originating from either a mapped or native EPS security context may exist simultaneously with a native non-current EPS security context.

ECM-CONNECTED state: This is as defined in TS 23.401 [2]. The term ECM-CONNECTED state corresponds to the term EMM-CONNECTED mode used in TS 24.301 [9].

ECM-IDLE state: As defined in TS 23.401 [2]. The term ECM-IDLE state corresponds to the term EMM-IDLE mode used in TS 24.301 [9].

EPS-Authentication Vector: KASME, RAND, AUTN, XRES

EPS security context: A state that is established locally at the UE and a serving network domain. At both ends "EPS security context data" is stored, that consists of the EPS NAS security context, and the EPS AS security context.

NOTE 1:	An EPS security context has type "mapped", "full native" or "partial native". Its state can either be "current" or "non-current". A context can be of one type only and be in one state at a time. The state of a particular context type can change over time. A partial native context can be transformed into a full native. No other type transformations are possible.

EPS AS security context: the cryptographic keys at AS level with their identifiers, the Next Hop parameter NH, the Next Hop Chaining Counter parameter NCC used for next hop access key derivation, the identifiers of the selected AS level cryptographic algorithms, counters used for replay protection and SCG Counter used as freshness input into S-KeNB derivations. Note that the EPS AS security context only exists when cryptographically protected radio bearers are established and is otherwise void.

NOTE 2: NH and NCC need to be stored also at the MME during connected mode.

EPS AS Secondary Cell security context: This context consists of the cryptographic keys for SeNB (KUPenc), the identifier of the selected AS SC level cryptographic algorithm and counters used for replay protection.

EPS NAS security context: This context consists of KASME with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values. In particular, separate pairs of NAS COUNT values are used for each EPS NAS security contexts, respectively. The distinction between native and mapped EPS security contexts also applies to EPS NAS security contexts. The EPS NAS security context is called "full" if it additionally contains the keys KNASint and KNASenc and the identifiers of the selected NAS integrity and encryption algorithms.

Feeder link: as defined in TS 36.300 [5].

Full native EPS security context: A native EPS security context for which the EPS NAS security context is full according to the above definition. A full native EPS security context is either in state "current" or state "non-current".

Forward security: In the context of KeNB key derivation, forward security refers to the property that, for an eNB with knowledge of a KeNB, shared with a UE, it shall be computationally infeasible to predict any future KeNB, that will be used between the same UE and another eNB. More specifically, n hop forward security refers to the property that an eNB is unable to compute keys that will be used between a UE and another eNB to which the UE is connected after n or more handovers (n=1 or 2).

IAB-node: As defined in TS 23.401 [2].

IAB-donor: As defined in TS 23.401 [2].

Legacy security context: A security context which has been established according to TS 33.102 [4].

Mapped security context: Security context created by converting the current security context in the source system to a security context for the target system in inter-system mobility, e.g., UMTS keys created from EPS keys. The EPS NAS security context of a mapped security context is full and current.

Native EPS security context: An EPS security context whose KASME was created by a run of EPS AKA.

Non-current EPS security context: A native EPS security context that is not the current one. A non-current EPS security context may be stored along with a current EPS security context in the UE and the MME. A non-current EPS security context does not contain an EPS AS security context. A non-current EPS security context is either of type "full native" or of type "partial native".

Partial native EPS security context: A partial native EPS security context consists of KASME with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values, which are initially set to zero before the first NAS SMC procedure for this security context. A partial native EPS security context is created by an EPS AKA, for which no corresponding successful NAS SMC has been run. A partial native context is always in state "non-current".

Re-derivation of NAS keys: derivation of new NAS keys from the same KASME but including different algorithms (and no freshness parameter)

Refresh of KeNB: derivation of a new KeNB from the same KASME and including a freshness parameter

Re-keying of KeNB: derivation of a new KeNB from a new KASME in ECM-CONNECTED (i.e., . to activate a partial native EPS security context, or to re-activate a non-current full EPS security context)

Re-keying of NAS keys: derivation of new NAS keys from a new KASME

Service link: as defined in TS 36.300 [5].

Store and Forward Satellite operation: as defined in TS 23.401 [2].

UE security capabilities: The set of identifiers corresponding to the ciphering and integrity algorithms implemented in the UE. This includes capabilities for EPS AS and NAS, and includes capabilities for UTRAN and GERAN if these access types are supported by the UE.

UE EPS security capabilities: The UE security capabilities for EPS AS and NAS.

User plane: Within the context of TS 33.401, this means the data path between UE and Serving Gateway that does NOT go via the MME.

(User) Data via MME: User Data sent to or from the UE that uses an RRC connection established using the Control Plane CIoT EPS optimisation specified in TS 23.401[2].

IOPS-capable eNB: an eNB that has the capability of IOPS mode operation, which provides local IP connectivity and Public Safety services to IOPS-enabled UEs via a Local EPC when the eNB has lost backhaul to the Macro EPC or it has no backhaul to the Macro EPC.

IOPS network: an IOPS network consists of one or more eNBs operating in IOPS mode and connected to a Local EPC.

Local EPC: a Local EPC is an entity which provides functionality that eNBs in IOPS mode of operation use, instead of the Macro EPC, in order to support Public Safety services.

Macro EPC: the EPC which serves an eNB when it is not in IOPS mode of operation.

Nomadic EPS: a deployable system which has the capability to provide radio access (via deployable IOPS-capable eNB(s)), local IP connectivity and Public Safety services to IOPS-enabled UEs in the absence of normal EPS.

IOPS-enabled UE: is an UE that is configured to use networks operating in IOPS mode.

*** Next Change ***

Annex X (normative):
Security for Store and Forward Satellite operation

# X.1	General

This Annex describes the security aspects of Store and Forward Satellite operation. The general features of Store and Forward Satellite operation are described in 23.401 [2].

There are two example deployment options for Store and Forward Satellite operation given in Annex O of TS 23.401 [2], i.e. Split MME architecture and Full EPC in each satellite. In both cases, regular LTE procedures shall be used to provide the security between the UE and network, e.g. authentication and protection of traffic between the UE and network.

The security of communications between the proxies on satellite and the ground station(s) is out of 3GPP scope.

NOTE:	The following informative text in clauses X.2 and X.3 outlines security aspects of the two deployment options as described in Annex O.2 and O.3 of TS 23.401 [2].

# X.2	Security aspects of Split MME architecture

Mutual authentication between the UE and the network in a split MME architecture can involve more than one satellite (i.e., more than one MME-onboard), in which case the ground segment of the network is responsible for the selection and provisioning of MME-onboard the same, or another satellite, with the necessary information (e.g., Authentication Vector) to perform or finish an authentication procedure. The MME on-board obtains the EPS authentication vectors when the feeder link is available and stores the authentication vectors when the service link is unavailable.

According to clause 4.13.9.1 of TS 23.401 [2] the MME rejects a NAS procedure if the MME-onboard cannot complete the NAS procedure with the information it has available. Hence the NAS security is terminated on the MME-onboard, and the ground segment of the network ensures that the latest NAS security context of the UE, or an Authentication Vector, is available at the MME-onboard.

NOTE 1: 	The distribution and synchronization of the latest NAS security context between the MME-ground and MME-onboard is out of the scope of 3GPP.

NOTE 2: 	The security environment on board a satellite is characterized by relatively low processing and storage capabilities of the equipment on board the satellite compared to terrestrial communication equipment. Due to the feeder link temporary unavailability in split-MME deployment, a malicious entity can flood satellites with spoofed Attach Requests (unauthenticated Attach Requests). This consumes storage space in the MME-onboard before authentication occurs, hence potentially resulting in (D)DoS attack on the Satellite. Addressing said (D)DoS attack in the Split MME deployment is left to implementation.

# X.3	Security aspects of Full EPC in each satellite

The security credentials required for the mutual authentication between the UE and network are stored in the HSS onboard the satellite. To enable having only a satellite-specific subscriber key stored in the satellite, IOPS-based keying method described in Annex F of the present document is used to limit the impact of exposing long-term keys in the HSS onboard the satellite.

*** End of Change ***