3GPP TSG SA Meeting #110	SP-251375

Baltimore, US, December 9 – 12, 2025

# Source:	vivo Mobile Communication Co., Ltd. (Rapporteur)

# Title:	Work Item Summary for EVS Codec Extension for Immersive Voice and Audio Services, Phase 2 (IVAS_Codec_Ph2)

# WI code(s):	IVAS_Codec_Ph2 (UID: 1040021)

# Leading WG:	SA WG4

# Release:	Rel-19

# Document for:	Endorsement

# Agenda Item:	21.5

3GPP TSG SA WG4#134	                                                               Not reviewed by SA4

Dallas, US, 17 - 21 November, 2025	Revision of S4-252023

# Title:	Summary for IVAS_Codec_Ph2 WI

# Source:	vivo Mobile Communication Co., Ltd. (Rapporteur)

# Document for:	Endorsement

# Agenda Item:	7.5

1	Introduction

The “EVS Codec Extension for Immersive Voice and Audio Services, Phase 2 (IVAS_Codec_Ph2)” work item is completed with specification of reference fixed-point C-code for the IVAS codec (completed in IVAS_Codec work item, Rel-18) along with codec characterization and enhanced conformance procedures.

2	Description

The IVAS_Codec_Ph2 work item [1][2] provides enhancements for the set of IVAS codec specifications (as defined by TS 26.250), initially developed under the IVAS_Codec [3] work item (Rel-18). It includes specification of reference fixed-point code and enhanced conformance procedure with the inclusion of non-bit-exact conformance for floating-point implementations. Further an extensive characterization was performed to document the performance of the codec at its various operation modes (encoded audio formats, bitrates, rendering, discontinuous transmission (DTX), packet loss concealment (PLC), jitter buffer management (JBM) etc.). The characterization tests also confirm the interoperability between the existing floating-point reference code in TS 26.258 and the new fixed-point reference code in TS 26.251.

The outcome of the IVAS_Codec_Ph2 is summarized as follows:

- Specification of IVAS fixed-point reference C-code in TS 26.251 “Codec for Immersive Voice and Audio Services (IVAS); C code (fixed-point)” with the same functionalities as the floating-point C-code standard in TS 26.258, having

  - Comparable audio quality and complexity to the corresponding floating-point code [6][7][8].

- IVAS codec characterization by an extensive set of subjective listening tests according to agreed test and processing plans [4][5], with test results [8] added to TR 26.997 including:

  - Performance characterization of the codec over various operation modes (encoded audio formats, bitrates, rendering, discontinuous transmission (DTX), packet loss concealment (PLC), jitter buffer management (JBM) etc.),

  - Evidence of interoperability between reference floating-point (TS 26.258) and fixed-point (TS 26.251) specifications.

- Updated IVAS conformance procedure in TS 26.252 with new test sequences for fixed-point C-code in TS 26.251 and a new non-bit-exact conformance method for floating-point implementations using TS 26.258.

3 References

[1]	SP-241000 New WID on EVS Codec Extension for Immersive Voice and Audio Services, Phase 2.

[2]	SP-250262 Revised WID of IVAS_Codec_Ph2.

[3]	SP-220608 EVS Codec Extension for Immersive Voice and Audio Services

[4]	S4-251561 IVAS-7b Processing plan for Characterization phase, v.1.1.0.

[5]	S4-252024 IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase v.1.3.0

[6]	S4-251013 Verification of IVAS BASOP encoder.docx

[7]	S4-242054 Verification of IVAS BASOP decoder and renderer

[8]	S4-251668 IVAS Characterization Tests - Global Analysis Lab Report