3GPP TSG-SA5 Meeting #163	S5-254224

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0734 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 Add measurements of UPF related information exposed |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom Corporation Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Events of QoS Monitoring, User Data Usage Measures and User Data Usage Trends are used for UPF data collection, as defined in TS 23.502. Based on the input parameters in subscription to UPF event exposure events, target of event reporting can be a UE, any UE and AOI. Different types of event measurement or target of evnet reporting would have a different impact on UPF. Therefore, it is useful to monitor related measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UPF Event exposure related measurements： - Number of UPF event exposure subscription for type of event requested - Number of UPF event exposure subscription for Target of Event Reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot monitor UPF Event exposure related measurements for the subcounter of “type of event requested” and “target of event reporting”. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.4.X(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 23.501 [4].

AOI	Area of Interest

CHO	Conditional Handover

CLI	Cross Link Interference

DAPS	Dual Active Protocol Stack

EN-DC	E-UTRA-NR Dual Connectivity

GP	Guard Period

HO	Handover

ITI	Interrupted Transmission Indication

kbit	kilobit (1000 bits)

KPI	Key Performance Indicator

LHO	Legacy Handover

LTM	L1/L2 Triggered Mobility

MA PDU	Multi-Access PDU

MN	Master Node.

MPQUIC	Multi-Path QUIC

MPTCP	Multi-Path TCP Protocol

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NG-RAN	Next Generation Radio Access Network

RNA	RAN-based Notification Area

PI	Performance Indicator

PLR	Packet Loss Rate

PMF	Performance Measurement Function

SA PDU	Single-Access PDU

SDT	Small Data Transmission

SN	Secondary Node.

SRS	Sounding Reference Signal

TEID	Tunnel Endpoint IDentifier

next change

5.4.X	UPF Event exposure related measurements

5.4.X.1	Number of UPF event exposure subscription for type of event requested

a)	This measurement provides the number of UPF event exposure subscription received by the UPF. The measurement is split into subcounters per UPF Event ID (QoS Monitoring, User DataUsage Measures, User DataUsage Trends, as specified in TS 23.502 [7]).

b)	CC.

c)	Receipt by the UPF of an Nupf_EventExposure_Subscribe message (see TS 23.502 [7]). The measurement is split into subcounters per UPF Event ID (QoS Monitoring, User DataUsage Measures, User DataUsage Trends).

d)	A single integer value.

e)	UPF.EventExpSubs.EventId, where EventId identifies the UPF Event ID (mapped QoS Monitoring, User DataUsage Measures, User DataUsage Trends)

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

5.4.X. 2	Number of UPF event exposure subscription for target of event reporting

a)	This measurement provides the number of UPF event exposure subscription received by the UPF. The measurement is split into subcounters per Target of Event Reporting (Any UE, Certain UE, UE(s) located in the AOI, as specified in TS 23.502 [7]).

b)	CC.

c)	Receipt by the UPF of an Nupf_EventExposure_Subscribe message (see TS 23.502 [7]). The measurement is split into subcounters per Target of Event Reporting (Any UE, Certain UE, UE(s) located in the AOI).

d)	A single integer value.

e)	UPF.EventExpSubs.EventTarget, where EventTarget identifies the Target of UPF Event Reporting (mapped Any UE, Certain UE, UE(s) located in the AOI).

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

next change

A.X Monitoring of UPF event exposure for UPF Data Collection

To get exposure data from UPF, NF consumer may use UPF event exposure service (Notify, Subscribe, UnSubscribe) for UPF data collection targets one UE, a group of UE or Any UE. The NF consumer may use the events exposed by UPF for QoS Monitoring, User DataUsage Measures and User DataUsage Trends. Different care needs to be taken with regard to load and major signalling impacts for different target event reporting or requested type of event. To monitor and optimise the impact on UPF performance, it is necessary to measure UPF event exposure from different aspects.

End of changes