3GPP TSG-SA WG6 Meeting #69	S6-254215

# Wuhan, P.R.China, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0404 | rev | - | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to SS_LocationReporting Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2025-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CR is bringing some consistency updates to align the location information description. For example, in 9.3.10, the step 1 indicates some details of the expected location information, such as geographic area. Or clause 9.3.12 where it’s stated that “location information criteria may include the geographic location information”. The information elements defined in Table 9.3.2.4-1 and Table 9.3.2.4-2 are updated to align the location information description with examples mentioned in other information flows (e.g., geographical area). The “Inside or outside indication” description in Table 9.3.2.4-2 is restrictive to the VAL Service Area ID(s); however, that functionality is useful for other cases when the geographical area is set as list of coordinates. In order to avoid complication at stage 3, e.g., a service consumer shall create and provision the VAL Service Area ID (that requires availability of another VAL Service Area Configuration service) for using the inside/outside location tracking. Thus, it is proposed to remove limitation and align with the requested location information in Table 9.3.2.4-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes essential correction for the SS_LocationReporting service: - Update the use-case description in clause 9.3.3.1. to be consistent with the Table 9.3.2.4-2. - align the ”Requested location information” IE in Table 9.3.2.4-1 to use clarification for the location informaiton that is already available in other clauses in the TS. - Update of the “Inside or outside indication” IE description in Table 9.3.2.4-2 to reuse the location information provided in the request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistent and unclear use-cases for the adaptive location reporting that does not follow the specification style for other specified functionality in the TS. - Unclarity in definition of the ”Requested location information” IE in Table 9.3.2.4-1., potential for misalignment with the “location information” as described in other clauses. - Restrictive description of “Inside or outside indication” IE in Table 9.3.2.4-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.3.1, 9.3.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

#### 9.3.3.1	General

The location management server provides location reporting configuration to the location management clients, indicating what information the location management server expects and what events will trigger the sending of this information to the location management server. The decision to report location information can be triggered at the location management client by different conditions, e.g., the reception of the location reporting configuration, initial registration, distance travelled, elapsed time, cell change, MBMS SAI change, MBMS session change, leaving a specific MBMS bearer service area, tracking area change, PLMN change, call initiation, GPS coordinates change inside or outside a service area, periodic reporting after leaving a geographic area, or other types of events such as emergency. The location report can include information described as ECGI, MBMS SAIs, geographic coordinates and other location information.

* * * Next change * * * *

#### 9.3.2.4	Location reporting trigger

Table 9.3.2.4-1 describes the information flow from the location management client or VAL server to the location management server for triggering a location reporting procedure.

Table 9.3.2.4-1: Location reporting trigger

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identity | M (see NOTE 1) | Identity of the requesting authorized VAL user or VAL UE or VAL server |
| Identity | M (see NOTE 1) | Identity of the requested VAL user or VAL UE |
| VAL service ID | O | Identity of the VAL service for which the location reporting trigger is set. |
| Immediate Report Indicator | O (see NOTE 2) | Indicates whether an immediate location report is required |
| Requested location information | O (see NOTE 2) | Identifies what location information is requested (e.g., geographic area). This information element may be represented as VAL service area ID(s) when this information flow from VAL server to the location management server. |
| Triggering criteria | O (see NOTE 2) | Identifies when the requesting client/VAL server is expecting to receive the location report from the requested VAL user/ VAL UE as specified in table 9.3.2.4-2. |
| Minimum time between consecutive reports | O (see NOTE 2) | Defaults to 0 if absent otherwise indicates the interval time between consecutive reports |
| Endpoint information | O | Information of the endpoint of the requesting VAL server to which the location report notification has to be sent. It is provided if Immediate Report Indicator is set to required. |
| Adaptive reporting | O | It indicates the request for an adaptive location reporting by dynamically adjusting the configuration. Following are possible values: - "DIRECT UPDATE", indicates the SEAL LMS to directly update the SEAL LM client once configuration is adjusted; or - "SUGGESTIVE UPDATE", indicates that the VAL Server subscribes to receive the suggested configurations from the SEAL LMS. The related notifications shall be sent by SEAL LMS before updating to the SEAL LM client. |
| NOTE 1: The identity of the requesting VAL user/UE/VAL server and the requested VAL user/UE should belong to the same VAL service. NOTE 2: At least one of these rows shall be present. |  |  |

Table 9.3.2.4-2: Location reporting Triggering criteria

| Information element | Status | Description |
| --- | --- | --- |
| Frequency of reporting | O | It indicates the requested frequency of reporting. The reporting frequency may be periodic or event-triggered. |
| Reporting periodicity | O | If the Frequency of reporting is periodic, the reporting periodicity shall be provided. |
| Location change condition | O (see NOTE) | It indicates the location change condition when the reporting shall occur, e.g., VAL user or VAL UE changes a cell, NodeB, TA, RA, WLAN access network, civic address, GPS coordinates, SAI, ECGI, RAT, VAL service area. If the Frequency of reporting is event-triggered, this IE shall be provided. |
| Inside or outside indication | O (see NOTE) | It indicates the condition when the reporting shall occur, i.e., the VAL user or VAL UE is inside or outside the locations represented by the requested location information in Table 9.3.2.4-1. |
| Reporting schedule | O | It indicates the requested reporting schedule, e.g., days of the week and/or time period for the location reporting. |
| NOTE: Triggering criteria based on the VAL service area represented by the VAL service area ID(s) may be provided when this information flow is from the VAL server to the location management server. |  |  |

* * * End of changes * * * *