3GPP TSG-SA WG6 Meeting #70	S6-255349

Dallas, United States 17th – 21st November 2025	(revision of S6-255066)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0396 | rev | 1 | Current version: | 19.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of procedure descriptions on various clauses |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The data service procedure message flow descriptions in the affected clauses are not fully consistent with the naming of the corresponding information flow data structures. This misalignment may lead to ambiguity in interpretation and hinder correct implementation. The change ensures that message flow descriptions and data structure naming are harmonized to maintain clarity, consistency and technical accuracy within the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The message flow descriptions in the affected clauses are updated to align with the naming of the corresponding information flow data structures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies will remain between the message flow descriptions and the corresponding information flow data structures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2.3.2, 7.4.2.4.2, 7.4.2.5.2, 7.4.2.6.2, 7.4.2.7.2, 7.4.2.9.2, 7.4.2.10.2, 7.4.3.4.2, 7.5.2.6.2, 7.5.2.7.2, 7.5.2.10.2, 7.5.3.4.2, 7.7.2.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

##### 7.4.2.3.2	Procedure

The procedure in figure 7.4.2.3.2-1 describes the case where an MCData user is initiating one-to-one MCData data communication for sending standalone SDS data to other MCData user, with or without disposition request. Standalone refers to sending unidirectional data in one transaction. The SDS payload data size is assumed to be above the configured maximum payload data size for SDS over signalling control plane.

Pre-conditions:

1.	MCData users on MCData client 1 and MCData client 2 are already registered for receiving MCData service.

2.	MCData client 1 and MCData client 2 belong to the same MCData system.

3.	Optionally, the MCData client may have an activated functional alias to be used.

4.	The MCData server may have subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.4.2.3.2-1: One-to-one standalone short data service using media plane

1.	User at MCData client 1 would like to initiate an SDS data transfer request for the chosen MCData user.

2.	MCData client 1 sends a MCData standalone session data request towards the MCData server. The MCData standalone session data request contains one MCData user for one-to-one data communication as selected by the user at MCData client 1. The MCData standalone session data request contains conversation identifier for message thread indication. The MCData standalone session data request may include additional implementation specific information in the application metadata container. The MCData standalone session data request may contain disposition request if indicated by the user at MCData client 1. MCData user at MCData client 1 may include a functional alias within the SDS data transfer and addresses the target MCData client 2 using a functional alias.

a)	If the MCData user at the MCData client 1 initiates an MCData emergency short data service communication or MCData emergency state is already set for the MCData client 1 (due to previously triggered MCData emergency alert):

i)	The MCData standalone session data request shall contain emergency indicator; and

ii)	If MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state of MCData client 1 is retained until explicitly cancelled by the user of MCData client 1.

NOTE 1:	While MCData client 1 is in the emergency state, all types of MCData one-to-one and group communications initiated by MCData client 1 are initiated as MCData emergency communications.

3.	MCData server checks whether the MCData user at MCData client 1 is authorized to send MCData standalone session data request. MCData server verifies whether the provided functional alias of MCData client 1, if present, can be used and has been activated for the user. The MCData server also checks whether any policy is to be asserted to limit certain types of message or content to certain members due, for example, to location or user privilege. MCData server determines the eligible MCData user(s) after policy assertion for sending the MCData standalone session data request. If functional alias is used to address that target MCData user, the MCData server resolves the functional alias to the corresponding MCData ID(s) for which the functional alias is active and proceed with step 4 otherwise proceed with step 6. The resulting list contains all associated MCData IDs/MCData users that share this functional alias. The MCData server allows only two participating MCData clients for a standalone short data service.

NOTE 2:	The MCData server prioritizes the MCData emergency communication over the other MCData communication. How the MCData server prioritizes MCData emergency communication is not in the scope of the present document.

4.	The MCData server responds back to MCData client 1 with a functional alias resolution response message that contains the resolved MCData ID.

NOTE 3:	If the MCData server detects that the functional alias used as the target of the MCData standalone session data request is simultaneously active for multiple MCData users, then the MCData server can proceed by selecting an appropriate MCData ID based on some selection criteria. The selection of an appropriate MCData ID is left to implementation. These selection criteria can include rejection of the MCData standalone session data request, if no suitable MCData ID is selected.

5.	If the MCData server replies with a MCData functional alias resolution response message, the MCData client 1 abandons the MCData standalone session data request in step 2 and sends a new MCData standalone session data request towards the resolved MCData ID.

6.	MCData server initiates the MCData standalone session data request towards the MCData users determined. The MCData standalone session data request towards the MCData user contains an emergency indicator if it is present in the received MCData standalone session data request from MCData client 1.

NOTE 4:	MCData client 2 corresponds to the MCData user(s) after resolution of the functional alias.

NOTE 5:	MCData client 2 does not set its emergency state as a result of receiving the MCData standalone session data request containing the emergency indicator.

7.	The receiving MCData client 2 automatically accepts the MCData standalone session data request and responds with MCData standalone session data response towards MCData server.

8.	MCData server forwards the MCData client 2 accepted response to the MCData Client 1 initiating the MCData standalone session data request.

9.	MCData client 1 and MCData client 2 have successfully established media plane for data communication and the MCData client 1 transmits the SDS data.

10.	If the payload is for MCData user consumption (e.g. is not application data, is not command instructions, etc.) then the MCData user of MCData client 2 may be notified. Otherwise if the payload is not for MCData user consumption, then the MCData user of MCData client 2 shall not be notified. The action taken when the payload contains application data or command instructions are specific based on the contents of the payload. Payload content received by MCData client 2 which is addressed to a known local non-MCData application that is not yet running shall cause the MCData client 2 to start the local non-MCData application (i.e., remote start application) and shall pass the payload content to the just started application.

11.	If the MCData data disposition for delivery was requested by the user at MCData client 1, then the receiving MCData client initiates a MCData data disposition notification for delivery report. The MCData data disposition notification from MCData client 2 may be stored by the MCData server for disposition history interrogation from authorized MCData users.

12.	MCData data disposition notification is sent to the disposition requesting user at MCData client 1.

13. If the MCData disposition for read was requested by the user at MCData client 1, then once the receiving user reads the data, the receiving MCData client 2 initiates a MCData disposition notification for read report. The MCData data disposition notification from MCData client 2 may be stored by the MCData server for disposition history interrogation from authorized MCData users.

14.	MCData data disposition notification is sent to the disposition requesting user at MCData client 1.

# * * * * 2nd change * * * *

##### 7.4.2.4.2	Procedure

The procedure in figure 7.4.2.4.2-1 describes the case where an MCData user is initiating data communication session with another MCData user for exchanging at least one SDS data transaction between them, with or without disposition request using MCData-SDS-1 and MCData-SDS-2 or MCData-SDS-3 reference points.

Pre-conditions:

1.	MCData users on MCData client 1 and MCData client 2 are already registered for receiving MCData service.

2.	Optionally, the MCData client may have activated functional alias to be used.

3.	The MCData server may have subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.4.2.4.2-1: One-to-one short data service session

1.	User at MCData client 1 would like to initiate an SDS data communication session request for the chosen MCData user.

2.	MCData client 1 sends a MCData session data request towards the MCData server. The MCData session data request contains one MCData user for one-to-one data communication as selected by the user at MCData client 1. The MCData session data request contains conversation identifier for message thread indication. The MCData session data request may include additional implementation specific information in the application metadata container. MCData user at MCData client 1 may include a functional alias within the SDS data transfer and addresses the target MCData client 2 using a functional alias.

a)	If the MCData user at the MCData client 1 initiates an MCData emergency short data service communication or MCData emergency state is already set for the MCData client 1 (due to previously triggered MCData emergency alert):

i)	The MCData session data request shall contain emergency indicator; and

ii)	If MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state of MCData client is retained until explicitly cancelled by the user of MCData client 1.

NOTE 1:	While MCData client 1 is in the emergency state, all types of MCData one-to-one and group communications initiated by MCData client 1 are initiated as MCData emergency communications.

3.	MCData server checks whether the MCData user at MCData client 1 is authorized to send MCData session data request. The MCData server also checks whether any policy is to be asserted to limit certain types of message or content to certain members due, for example, to location or user privilege. MCData server determines the eligible MCData user(s) after policy assertion for sending the MCData session data request. MCData server also verifies whether the provided functional alias of MCData client 1, if present, can be used and has been activated for the user. If functional alias is used to address that target MCData user, the MCData server resolves the functional alias to the corresponding MCData ID(s) for which the functional alias is active and proceed with step 4 otherwise proceed with step 6. The MCData server allows only two participating MCData clients for a standalone short data service.

NOTE 2:	The MCData server prioritizes the MCData emergency communication over the other MCData communication. How the MCData server prioritizes MCData emergency communication is not in the scope of the present document.

NOTE 3:	If the MCData server detects that the functional alias used as the target of the MCData session data request is simultaneously active for multiple MCData users, then the MCData server can proceed by selecting an appropriate MCData ID based on some selection criteria. The selection of an appropriate MCData ID is left to implementation. These selection criteria can include rejection of the SDS data transfer request, if no suitable MCData ID is selected.

4.	The MCData server responds back to MCData client 1 with a functional alias resolution response message that contains the resolved MCData ID.

5.	If the MCData server replies with a MCData functional alias resolution response message, the MCData client 1 abandons the MCData session data request in step 2 and sends a new MCData session data request towards the resolved MCData ID.

6.	MCData server initiates the MCData session data request towards the MCData users determined. The MCData session data request towards the MCData user contains the emergency indicator if it is present in the received MCData session data request from MCData client 1.

NOTE 4:	MCData client 2 corresponds to the MCData user(s) after resolution of the functional alias.

NOTE 5:	MCData client 2 does not set its emergency state as a result of receiving the MCData session data request containing the emergency indicator.

7.	If the emergency indicator is present, the receiving MCData client 2 notifies the user about the incoming MCData session data request.

8.	The receiving MCData client 2 accepts the MCData session data request and responds with MCData session data response towards MCData server.

9.	MCData server forwards the MCData client 2 accepted response to the MCData user initiating the MCData session data request.

10. and 11.	MCData client 1 and MCData client 2 have successfully established media plane for data communication and either MCData client can transmit SDS data. The MCData session data request may contain disposition request if indicated by the client sending data. If MCData data disposition was requested by the user, then the receiving MCData client initiates a MCData data disposition notification for delivery, read reports to the disposition requesting user. The MCData data disposition notification from MCData user may be stored by the MCData server for disposition history interrogation from authorized users.

12. and 13.	If the payload is for MCData user consumption (e.g. is not application data, is not command instructions, etc.) then the MCData user of MCData client 2 may be notified, otherwise the MCData user of MCData client 2 shall not be notified.

14. After SDS data transaction is complete, the established media plane is released.

# * * * * 3rd change * * * *

##### 7.4.2.5.2	Procedure

The procedure in figure 7.4.2.5.2-1 describes the case where an MCData user is initiating group standalone MCData data communication with or without disposition request, to a group.

Pre-conditions:

1.	MCData users on MCData clients 1 to n belong to the same group and are already registered for receiving MCData service and affiliated.

2.	Optionally, the MCData client may have activated functional alias to be used.

3.	The MCData server may have subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.4.2.5.2-1: Group standalone SDS using signalling control plane

1.	The user at MCData client 1 initiates an SDS data transfer to multiple MCData users selecting a pre-configured group (identified by MCData group ID) and optionally particular members from that group.

2.	MCData client 1 sends a MCData group standalone data request towards the MCData server. The MCData group standalone data request contains MCData group ID as selected by the user at MCData client 1. The MCData group standalone data request contains conversation identifier for message thread indication. The MCData group standalone data request may include additional implementation specific information in the application metadata container. The MCData group standalone data request may contain disposition request if indicated by the user at MCData client 1. MCData user at MCData client 1 may include a functional alias within the SDS data transfer.

If the MCData user at MCData client 1 initiates an MCData emergency short data service communication or the MCData emergency state is already set for the MCData client 1 (due to a previously triggered MCData emergency alert):

i)	the MCData group standalone data request shall contain an emergency indicator;

ii)	the MCData group standalone data request shall set an alert indicator if configured to send an MCData emergency alert while initiating an MCData group standalone data request for the emergency short data service communication;

iii)	if the MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state is retained until explicitly cancelled; and

iv)	once an MCData emergency communication has been initiated, the MCData group is considered to be in an in-progress emergency state until cancelled.

If the MCData user at MCData client 1 initiates an MCData imminent peril short data service communication:

i)	the MCData group standalone data request shall contain imminent peril indicator; and

ii)	once an MCData imminent peril communication has been initiated, the MCData group is considered to be in an in-progress imminent peril state until cancelled.

2a.	If either emergency indicator or imminent peril indicator is present in the received MCData group standalone data request, the MCData server implicitly affiliates MCData client 1 to the MCData group if the client is not already affiliated.

3.	MCData server checks whether the MCData user at MCData client 1 is authorized to send MCData group standalone data request. The MCData server resolves the MCData group ID to determine the members of that group and their affiliation status, based on the information from the group management server. The MCData server also checks whether any policy is to be asserted to limit certain types of message or content to certain members due, for example, to location or user privilege or affiliation. MCData server also verifies whether the provided functional alias, if present, can be used and has been activated for the user.

i)	If an emergency indicator is present in the received MCData group standalone data request and if the MCData group is not in the in-progress emergency state, the MCData group is considered to be in the in-progress emergency state until cancelled; and

ii)	If an imminent peril indicator is present in the received MCData group standalone data request and if the MCData group is not in the in-progress imminent peril state, the MCData group is considered to be in the in-progress imminent peril state until cancelled.

4.	MCData server initiates the MCData group standalone data request towards each MCData client determined in Step 3. The MCData ID list shall not be included in a unicast downlink delivery to an individual MCData client. The Disposition Type IE shall not be included in a unicast downlink delivery to MCData clients who are not in the MCData ID list in step 2. The MCData group standalone data request towards each MCData client contains:

i)	an emergency indicator, if it is present in the received MCData group standalone data request from the MCData client 1;

ii)	an imminent peril indicator, if it is present in the received MCData group standalone data request from the MCData client 1; and

iii)	an alert indicator, if requested to initiate an emergency alert in the received MCData group standalone data request from the MCData client 1.

5.	If the payload is for MCData user consumption (e.g. is not application data, is not command instructions, etc.) then the MCData user of MCData clients 2 to n may be notified. Otherwise if the payload is not for MCData user consumption, then the MCData user of MCData clients 2 to n shall not be notified. The action taken when the payload contains application data or command instructions are specific based on the contents of the payload. Payload content received by MCData client 2 which is addressed to a known local non-MCData application that is not yet running shall cause the MCData client 2 to start the local non-MCData application (i.e., remote start application) and shall pass the payload content to the just started application.

NOTE:	On receiving MCData group standalone data request over MBMS, the receiving MCData client(s) shall check if the MCData ID list IE is included. If so, the receiving MCData client(s) shall check if their own MCData ID is in the list. If not, step 6 and 7 are not required.

6.	If the MCData data disposition for delivery was requested by the user at MCData client 1, then the receiving MCData client(s) initiates a MCData data disposition notification for delivery report.

7.	If the MCData data disposition for read was requested by the user at MCData client 1, then once the receiving user reads the data, the receiving MCData client 2 initiates a MCData data disposition notification for read report.

8.	The MCData data disposition notification(s) from MCData client may be stored by the MCData server for disposition history interrogation from authorized MCData users. The MCData data disposition notification(s) from each MCData user may be aggregated.

9.	Aggregated or individual MCData data disposition notification(s) is sent to the disposition requesting user at MCData client 1.

# * * * * 4th change * * * *

##### 7.4.2.6.2	Procedure

The procedure in figure 7.4.2.6.2-1 describes the case where an MCData user is initiating group standalone MCData data communication with or without disposition request to a group.

Pre-conditions:

1.	MCData users on MCData client 1 to n belong to the same group and are already registered for receiving MCData service and affiliated.

2.	Optionally, the MCData client may have activated functional alias to be used.

3.	The MCData server may have subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.4.2.6.2-1: Group standalone SDS using media plane

1.	User at MCData client 1 would like to initiate a SDS data transfer request to multiple MCData users selecting a pre-configured group (identified by MCData group ID) and optionally particular members from that group.

2.	MCData client 1 sends a MCData group session standalone data request towards the MCData server. The MCData group session standalone data request contains target recipient(s) as selected by the user at MCData client 1. The MCData group session standalone data request contains conversation identifier for message thread indication. The MCData group session standalone data request may include additional implementation specific information in the application metadata container. The MCData group session standalone data request may contain disposition request if indicated by the user at MCData client 1. MCData user at MCData client 1 may include a functional alias within the SDS data transfer.

If the MCData user at MCData client 1 initiates an MCData emergency short data service communication or the MCData emergency state is already set for MCData client 1 (due to a previously triggered MCData emergency alert):

i)	the MCData group session standalone data request shall contain an emergency indicator;

ii)	the MCData group session standalone data request shall set the alert indicator if configured to send an MCData emergency alert while initiating an MCData group session standalone data request for the emergency short data service communication;

iii)	if the MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state is retained until explicitly cancelled; and

iv)	once an MCData emergency communication has been initiated, the MCData group is considered to be in an in-progress emergency state until cancelled.

If the MCData user at MCData client 1 initiates an MCData imminent peril short data service communication:

i)	the MCData group session standalone data request shall contain an imminent peril indicator; and

ii)	once an MCData imminent peril communication has been initiated, the MCData group is considered to be in an in-progress imminent peril state until cancelled.

2a.	If either an emergency indicator or an imminent peril indicator is present in received MCData group session standalone data request, the MCData server implicitly affiliates MCData client 1 to the MCData group if the client is not already affiliated.

3.	MCData server checks whether the MCData user at MCData client 1 is authorized to send MCData group session standalone data request. The MCData server resolves the MCData group ID to determine the members of that group and their affiliation status, based on the information from the group management server. The MCData server also checks whether any policy is to be asserted to limit certain types of message or content to certain members due, for example, to location or user privilege. MCData server also verifies whether the provided functional alias, if present, can be used and has been activated for the user.

i)	if an emergency indicator is present in the received MCData group session standalone data request and if the MCData group is not in the in-progress emergency state, the MCData group is considered to be in the in-progress emergency state until cancelled; and

ii)	if an imminent peril indicator is present in the received MCData group session standalone data request and if the MCData group is not in the in-progress imminent peril state, the MCData group is considered to be in the in-progress imminent peril state until cancelled.

3a.	The MCData server configures the priority of the underlying bearers for all participants in the MCData group.

4.	MCData server initiates the MCData group session standalone data request towards each MCData user determined in Step 3. The MCData ID list shall not be included in a unicast downlink delivery to an individual MCData client. The Disposition Type IE shall not be included in a unicast downlink delivery to MCData clients who are not in the MCData ID list in step 2. The MCData group session standalone data request towards each MCData client contains:

i)	an emergency indicator, if it is present in the received MCData group session standalone data request from the MCData client 1;

ii)	an imminent peril indicator, if it is present in the received MCData group session standalone data request from the MCData client 1; and

iii)	an alert indicator, if requested to initiate an emergency alert in the received MCData group session standalone data request from MCData client 1.

5.	The receiving MCData clients 2 to n automatically accepts the MCData group session standalone data request and responds with MCData group standalone data response towards MCData server.

6.	MCData server forwards the MCData clients 2 to n accepted response to the MCData user initiating the MCData group session standalone data request.

NOTE 1:	Step 6 can occur at any time following step 4, and prior to step 7 depending on the conditions to proceed with the data transmission.

7.	MCData client 1 and MCData server have successfully established media plane for data communication and the MCData client 1 transmits the SDS data.

8.	MCData server distributes the data received from MCData client 1 to MCData clients 2 to n over the established media plane. After completion of the MCData transfer from MCData client 1, media plane resources associated to the data communication are released.

NOTE 2:	MCData server is not required to wait for the complete reception of SDS data from MCData client 1 prior to initiating transmission to MCData client 2 to n.

9.	If the payload is for MCData user consumption (e.g. is not application data, is not command instructions, etc.) then the MCData user of MCData client 2 to n may be notified. Otherwise if the payload is not for MCData user consumption, then the MCData user of MCData client 2 to n shall not be notified. The action taken when the payload contains application data or command instructions are specific based on the contents of the payload. Payload content received by MCData client 2 which is addressed to a known local non-MCData application that is not yet running shall cause the MCData client 2 to start the local non-MCData application (i.e., remote start application) and shall pass the payload content to the just started application.

NOTE 3:	On receiving MCData group session standalone data request over MBMS, the receiving MCData client(s) shall check if the MCData ID list IE is included. If so, the receiving MCData client(s) shall check if their own MCData ID is in the list. If not, step 10 and 11 are not required.

10.	If the MCData data disposition for delivery was requested by the user at MCData client 1, then the receiving MCData client(s) initiates a MCData data disposition notification for delivery report.

11.	If the MCData data disposition for read was requested by the user at MCData client 1, then once the receiving user reads the data, the receiving MCData client 2 initiates a MCData data disposition notification for read report.

12.	The MCData data disposition notification(s) from MCData client may be stored by the MCData server for disposition history interrogation from authorized MCData users. The MCData data disposition notification(s) from each MCData user may be aggregated.

13.	Aggregated or individual MCData data disposition notification(s) is sent to the disposition requesting user at MCData client 1.

# * * * * 5th change * * * *

##### 7.4.2.7.2	Procedure

The procedure in figure 7.4.2.7.2-1 describes the case where an MCData user is initiating SDS data communication session with an MCData group for exchanging SDS data transactions between the group participants, with or without disposition request, using MCData-SDS-1 and MCData-SDS-2 reference points.

Pre-conditions:

1.	MCData users on MCData client 1 to n belong to the same group and are already registered for receiving MCData service and affiliated.

2.	Optionally, the MCData client may have activated functional alias to be used.

3.	The MCData server may have subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.4.2.7.2-1: Group SDS session

1.	User at MCData client 1 would like to initiate a SDS group data transfer request to multiple MCData users selecting a pre-configured group (identified by MCData group ID) and optionally particular members from that group.

2.	MCData client 1 sends a MCData group data request towards the MCData server. The MCData group data request contains MCData group ID as selected by the user at MCData client 1. The MCData group data request contains conversation identifier for message thread indication. The MCData group data request may include additional implementation specific information in the application metadata container. MCData user at MCData client 1 may include a functional alias within the SDS data transfer.

If the MCData user at MCData client 1 initiates an MCData emergency short data service communication or the MCData emergency state is already set for the MCData client 1 (due to a previously triggered MCData emergency alert):

i)	the MCData group data request shall contain an emergency indicator;

ii)	the MCData group data request shall set an alert indicator if configured to send an MCData emergency alert while initiating an MCData group data request for the emergency short data service communication; and

iii)	if MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state of MCData client 1 is retained until explicitly cancelled by the user of MCData client 1.

NOTE 1:	While MCData client 1 is in the emergency state, all types of MCData one-to-one and group communications initiated by MCData client 1 are initiated as MCData emergency communications.

If the MCData user at MCData client 1 initiates an MCData imminent peril short data service communication:

i)	the MCData group data request shall contain an imminent peril indicator.

2a.	If either emergency indicator or imminent peril indicator is present in received MCData group data request, the MCData server implicitly affiliates MCData client 1 to the MCData group if the client is not already affiliated.

3.	MCData server checks whether the MCData user at MCData client 1 is authorized to send MCData group data request. The MCData server resolves the MCData group ID to determine the members of that group and their affiliation status, based on the information from the group management server. The MCData server also checks whether any policy is to be asserted to limit certain types of message or content to certain members due, for example, to location or user privilege. MCData server also verifies whether the provided functional alias, if present, can be used and has been activated for the user.

i)	if an emergency indicator is present in the received MCData group data request and if MCData group is not in in-progress emergency state, the MCData group is considered to be in the in-progress emergency state until cancelled;

NOTE 2:	While the MCData group is in the in-progress emergency state, all types of MCData communications within the group are processed as emergency group communications by the MCData server. MCData group members that are not in the emergency state do not indicate emergency in group communication requests.

ii)	if an imminent peril indicator is present in the received MCData group data request and if the MCData group is not in the in-progress imminent peril, the MCData group is considered to be in the in-progress imminent peril state until cancelled;

3a.	The MCData server configures the priority of the underlying bearers for all participants in the MCData group.

4.	MCData server initiates the MCData group data request towards each MCData user determined in Step 3. The MCData group data request towards each MCData client contains:

i)	an emergency indicator if it is present in the received MCData group data request from the MCData client 1;

ii)	an imminent peril indicator if it is present in the received MCData group data request from the MCData client 1; and

iii)	an alert indicator if requested to initiate an emergency alert in the received MCData group data request from MCData client 1;

5.	The receiving MCData clients 2 to n optionally notify the user about the incoming MCData group data request.

6.	The receiving MCData client 2 to n accept or reject the MCData group data request and the corresponding result is in the MCData group data response towards MCData server.

7.	MCData server forwards the MCData group data response received from MCData client 2 to n to the MCData user initiating the MCData group data request.

NOTE 3:	Step 7 can occur at any time following step 4, and prior to step 8 depending on the conditions to proceed with the data transmission.

8.	MCData client 1 and the MCData group data request accepted clients have successfully established media plane for data communication and either MCData client can transmit SDS data. The MCData group data request may contain disposition request if indicated by the client sending data. If the payload is for MCData user consumption (e.g. is not application data, is not command instructions, etc.) then the SDS data receiving MCData users may be notified, otherwise those MCData users shall not be notified.

9.	If MCData data disposition was requested by the user, then the SDS data receiving MCData client initiates a MCData data disposition notification for delivery, read reports to the disposition requesting user. The MCData data disposition notification from the receiving MCData clients may be stored by the MCData server for disposition history interrogation from authorized users.

10.	Based on the MCData user action or conditions to release, the established media plane for SDS data exchange is released.

# * * * * 6th change * * * *

##### 7.4.2.9.2	Procedure

The procedure in figure 7.4.2.9.2-1 describes the case where an authorized MCData user is upgrading an ongoing MCData group SDS communication to an MCData emergency group SDS communication. This procedure is applicable only when group MCData communication is established as described in subclause 7.4.2.6 "Group standalone short data service using media plane" or as described in subclause 7.4.2.7 "Group short data service session".

NOTE 1:	For simplicity, a single MCData server is shown in place of a user home MCData server and a group hosting MCData server.

Pre-conditions:

1.	The MCData group is previously defined on the group management server with MCData client 1, MCData client 2 and MCData client 3 affiliated to that MCData group.

2.	All members of the MCData group belong to the same MCData system.

3.	MCData group SDS communication is already in progress.

4.	The initiating MCData client 1 has been configured to send an MCData emergency alert when upgrading an MCData emergency group communication.

Figure 7.4.2.9.2-1: MCData group SDS communication upgraded to MCData emergency group SDS communication

1.	The MCData user at MCData client 1 initiates a group emergency. MCData client 1 sets its MCData emergency state. The MCData emergency state of MCData client 1 is retained until explicitly cancelled by the user of MCData client 1.

NOTE 2:	While MCData client 1 is in the emergency state, all types of MCData one-to-one and group communications initiated by MCData client 1 are initiated as MCData emergency communications.

2.	MCData client 1 requests the MCData server to upgrade the MCData group to an in-progress emergency state by sending a MCData group SDS communication upgrade request. The MCData client 1 sets the emergency indicator in the request. If configured to send an MCData alert when initiating an MCData emergency group SDS upgrade, the request also contains an indication that an MCData alert is to be initiated.

3.	The MCData server sets the emergency state of the MCData group and adjusts the priority of the underlying bearer for all or selected participants in the MCData group SDS communication that receive the communication over unicast.

NOTE 3:	The determination of the selected participants whose bearers have to be upgraded is left to implementation.

NOTE 4:	While the MCData group is in the in-progress emergency state, all types of MCData communications within the group are processed as emergency group communications by the MCData server. MCData group members that are not in the emergency state do not indicate emergency in group communication requests.

4.	MCData server sends the MCData group SDS communication upgrade request towards the MCData clients of each of those affiliated MCData group members. The request contains an indication of an MCData emergency alert if the request from the originator indicated MCData emergency alert.

5.	MCData users are notified of the in-progress emergency state of the MCData group.

6.	The receiving MCData clients send the MCData group SDS communication upgrade response to the MCData server to acknowledge the MCData group SDS communication upgrade request. For a multicast call, these acknowledgements are not sent.

7.	The MCData server sends the MCData group SDS communication upgrade response to the MCData user 1 to confirm the upgrade request.

NOTE 5:	Step 7 can occur at any time following step 3, depending on the conditions to proceed with the call.

MCData client 1, MCData client 2 and MCData client 3 continue with the MCData group SDS communication, which has been transformed into an MCData emergency group SDS communication.

# * * * * 7th change * * * *

##### 7.4.2.10.2	Procedure

The procedure in figure 7.4.2.10.2-1 describes the case where an authorized MCData user cancels MCData group's in-progress emergency.

Pre-conditions:

1.	The MCData group is previously defined on the group management server with MCData client 1, MCData client 2 and MCData client 3 affiliated to that MCData group.

2.	All members of the MCData group belong to the same MCData system.

3.	MCData group members have been notified about the in-progress emergency.

4.	The MCData group is in the in-progress emergency state and has prioritized bearer support.

5.	MCData client 1 previously initiated the in-progress emergency for the group.

Figure 7.4.2.10.2-1: MCData group SDS in-progress emergency group state cancel

1.	The user at the MCData client 1 initiates an MCData group SDS in-progress emergency group state cancel.

NOTE 1:	An MCData user authorized to cancel in-progress emergencies on the MCData group can also be authorised to cancel the MCData emergency alert in addition to the initiator. However, only the initiator can cancel the initiator's local MCData emergency state.

2.	The MCData client 1 sends an MCData group SDS communication in-progress priority state cancel request to the MCData server. The MCData client 1 also resets the emergency indicator in the request to inform MCData server about cancellation of in-progress emergency group state.

NOTE 2:	If an MCData emergency alert relating to MCData client 1 is in effect together with an MCData in-progress emergency group state on the MCData group, the MCData emergency alert of MCData client 1 can be cancelled at the same time. In that case, the MCData group SDS communication in-progress priority state cancel request carries an indication that the emergency alert of MCData client 1 is also being cancelled.

NOTE 3:	If an MCData group SDS communication in-progress priority state cancel request is received by the MCData server while a group member that is in the emergency state is transmitting, the MCData group SDS communication in-progress priority state cancel request is rejected by the MCData server.

3.	The MCData server adjusts the priority of the underlying bearer; priority treatment is no longer required. The MCData server cancels/resets the emergency in-progress state of the MCData group.

4.	The MCData server sends an MCData group SDS communication in-progress priority state cancel request to the MCData group members.

5.	MCData group members are notified of the MCData group SDS in-progress emergency state cancel.

6.	The receiving MCData clients send the MCData group SDS communication in-progress priority state cancel response to the MCData server to acknowledge the MCData in-progress emergency group state cancel. For a multicast call scenario, these acknowledgements are not sent.

7.	The MCData server sends the MCData group SDS communication in-progress priority state cancel response to the MCData user 1 to confirm the MCData in-progress emergency group state cancel. If the MCData group SDS communication in-progress priority state cancel request (in step 2) contained the "Alert indicator" IE, the MCData client 1 resets its local emergency status.

NOTE 4:	Step 7 can occur at any time following step 3, depending on the conditions to proceed with the call.

# * * * * 8th change * * * *

##### 7.4.3.4.2	Procedure

Figure 7.4.3.4.2-1 describes procedures for an off-network MCData client 1 initiating group MCData data communication for sending SDS data to a MCData group, with or without disposition request. The SDS data size limit is pre-configured.

Pre-conditions:

1.	MCData user 1 has initiated group communication for sending SDS data to the MCData group.

2.	Information for ProSe direct communications corresponding to the MCData group and its mapping to ProSe Layer-2 Group ID are pre-configured in MCData client 1.

3.	MCData client 1 to MCData client N are members of the same MCData group.

Figure 7.4.3.4.2-1: Group standalone short data service using signalling control plane

1.	MCData client 1 checks whether the MCData user 1 is authorized to send MCData group standalone data request.

2.	If MCData user 1 is authorised MCData client 1 sends a MCData group standalone data request towards the MCData group. The MCData group standalone data request contains conversation identifier for message thread indication. The MCData group standalone data request may include additional implementation specific information in the application metadata container. The MCData group standalone data request may contain disposition request if indicated by the user at MCData client 1. If MCData group standalone data request contains disposition request, MCData group standalone data request shall also contain the IP address of the MCData client 1. If MCData user at the MCData client 1 initiates an MCData emergency communication, then the emergency indicator or the imminent peril indicator is included in the MCData group standalone data request. If an MCData emergency state is not set already when MCData emergency communication is initiated, the MCData client 1 sets its MCData emergency state and is retained until explicitly cancelled. The value of ProSe Per Packet Priority is upgraded according to the state of the MCData communication.

3.	On receiving a MCData group standalone data request, the MCData clients check whether any policy is to be asserted to limit certain types of message or content to certain members due, for example, to location or user privilege.

4.	If the policy assertion is positive and the payload is for MCData user consumption (e.g. is not application data, is not command instructions, etc.) then the MCData user may be notified. Otherwise if the payload is not for MCData user consumption, then the MCData user shall not be notified. The action taken when the payload contains application data or command instructions are specific based on the contents of the payload. Payload content received by MCData clients 2 to N which is addressed to a known local non-MCData application that is not yet running shall cause the MCData clients 2 to N to start the local non-MCData application (i.e., remote start application) and shall pass the payload content to the just started application.

NOTE:	If the policy assertion was negative, the MCData clients sends an appropriate notification to MCData client 1.

5.	If the MCData data disposition for delivery was requested by the user at MCData client 1, then the receiving MCData clients initiate a MCData data disposition notification for delivery report.

6.	If the MCData data disposition for read was requested by the user at MCData client 1, then once the receiving user reads the data, the receiving MCData clients 2 to N initiate a MCData data disposition notification for read report.

# * * * * 9th change * * * *

##### 7.5.2.6.2	Procedure

The procedure in figure 7.5.2.6.2-1 describes the case where a MCData user is initiating group standalone data communication for sending a file to multiple MCData users, with or without download completed report request from the MCData user.

Pre-conditions:

1.	The MCData users on the MCData clients 1 to n belong to the same MCData group and are already registered for receiving MCData service and affiliated to the group.

2.	The file to be distributed is uploaded to the media storage function on the MCData content server using the procedures defined in subclause 7.5.2.2.

3.	The MCData client may have an activated functional alias to be used.

4.	The MCData server has subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.5.2.6.2-1: Group standalone FD using HTTP

1.	The user at the MCData client 1 initiates a file distribution request to multiple MCData users selecting a pre-configured group (identified by MCData group ID) and optionally particular members from that group.

2.	The MCData client 1 sends a MCData group standalone FD request towards the MCData server. The MCData group standalone FD request contains content payload in the form of file URL and may contain the file metadata information. The MCData group standalone FD request contains either the selected MCData group ID or the target recipients as selected by the user at MCData client 1. The MCData group standalone FD request contains conversation identifier for message thread indication. The MCData group standalone FD request may include additional implementation specific information in the application metadata container. If MCData user at MCData client 1 has requested to mandatory download at the recipient side, then MCData group standalone FD request contains mandatory download indication. The MCData group standalone FD request may contain a download completed report indication if selected by the user at MCData client 1. The MCData user at MCData client 1 may include a functional alias within the FD data transfer. If the MCData user at MCData client has requested to deposit the file content into his/her MCData message store account, then MCData group standalone FD request contains deposit file indication set.

If the MCData user at MCData client 1 initiates an MCData emergency FD communication or the MCData emergency state is already set for the MCData client 1 (due to a previously triggered MCData emergency alert):

i)	the MCData group standalone FD request shall contain an emergency indicator;

ii)	the MCData group standalone FD request shall set an alert indicator if configured to send an MCData emergency alert while initiating an MCData group standalone FD request for the emergency FD communication; and

iii)	if the MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state of MCData client 1 is retained until explicitly cancelled by the user of MCData client 1.

NOTE 1:	While MCData client 1 is in the emergency state, all types of MCData one-to-one and group communications initiated by MCData client 1 are initiated as MCData emergency communications.

If the MCData user at MCData client 1 initiates an MCData imminent peril FD communication:

i)	the MCData group standalone FD request shall contain an imminent peril indicator.

2a.	If either emergency indicator or imminent peril indicator is present in the received MCData group standalone FD request, the MCData server implicitly affiliates MCData client 1 to the MCData group if the client is not already affiliated.

3.	MCData server checks whether the MCData user at MCData client 1 is authorized to send an MCData group standalone FD request and that the size of the file is below maximum data size for FD from the group configuration. MCData server verifies whether the provided functional alias, if present, can be used and has been activated for the user. If the MCData group ID is used, the MCData server resolves the MCData group ID to determine the members of that group and their affiliation status, based on the information from the group management server.

i)	If an emergency indicator is present in the received MCData group standalone FD request and if the MCData group is not in the in-progress emergency state, the MCData group is considered to be in the in-progress emergency state until cancelled; and

NOTE 2:	While the MCData group is in the in-progress emergency state, all types of MCData communications within the group are processed as emergency group communications by the MCData server. MCData group members that are not in the emergency state do not indicate emergency in group communication requests.

ii)	If an imminent peril indicator is present in the received MCData group standalone FD request and if the MCData group is not in the in-progress imminent peril state, the MCData group is considered to be in the in-progress imminent peril state until cancelled.

4.	The MCData server may verify whether the corresponding file is available in the MCData content server (not shown in the figure) via the MCData-FD-5 reference point using the received file URL in the MCData group standalone FD request. For that, the MCData server sends an MCData file availability request to the MCData content server. Upon the receipt of the request, the MCData content server provides an MCData file availability response to the MCData server. If the MCData server identifies that the file is not available in the MCData content server, the MCData server provides a response to the MCData client 1 indicating that the file distribution request cannot proceed due to the unavailability of the file in the MCData content server and skip rest of the steps. If the deposit file indication information element is set to true in the received MCData group standalone FD request, MCData server shall follow the procedure as defined in the subclause 7.13.3.8 with the retrieve file indication element set to true while depositing this MCData communication to the MCData message store account of the user at MCData client 1.

5.	MCData server initiates the MCData group standalone FD request towards each MCData user determined in step 3. The MCData group standalone FD request towards each MCData client contains:

i)	an emergency indicator if it is present in the received MCData group standalone FD request from the MCData client 1;

ii)	an imminent peril indicator if it is present in the received MCData group standalone FD request from the MCData client 1; and

iii)	an alert indicator if requested to initiate an emergency alert in the received MCData group standalone FD request from the MCData client 1.

6.	The receiving MCData clients 2 to n notify the user about the incoming MCData group standalone FD request (including file metadata, if present) which may be either accepted or rejected or ignored.

7.	If the target MCData user on MCData clients 2 to n provides a response (accept or reject) to the notification, then respective MCData client sends the MCData group standalone FD response to the MCData server. MCData client 2 to n automatically sends accepted MCData group standalone FD response when the incoming request included mandatory download indication.

8.	The MCData server forwards the MCData group standalone FD responses to the MCData client 1.

NOTE 3:	Step 8 can occur at any time following step 5, and prior to step 9 depending on the conditions to proceed with the file transmission.

9.	The media storage client on the MCData client(s) accepting the request downloads the file from the MCData content server (not shown in the figure) using the procedures defined in subclause 7.5.2.3, either automatically (for mandatory download) or based upon the MCData user subsequent action. The MCData clients successfully receiving the file through the media storage clients, record file download completed and notify the MCData users.

10.	The MCData clients, receiving the file through the media storage client, provide MCData download completed reports for reporting file download completed, if requested by the user at MCData client 1.

11.	The MCData file download completed reports from MCData clients may be stored by the MCData server for download history interrogation from the authorized MCData users. The MCData file download completed report from each MCData user may be aggregated.

12.	Aggregated or individual MCData download completed reports are sent by the MCData server to the MCData user at MCData client 1, if requested by the MCData client 1.

# * * * * 10th change * * * *

##### 7.5.2.7.2	Procedure

The procedure in figure 7.5.2.7.2-1 describes the case where an MCData user is initiating group standalone data communication for sending file to multiple MCData users, with or without download completed report request.

Pre-conditions:

1.	The MCData users on the MCData client 1 to n belong to the same group and are already registered for receiving MCData service and affiliated.

2.	Optionally, the MCData client may have an activated functional alias to be used.

3.	The MCData server has subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.5.2.7.2-1: Group standalone FD using media plane

1.	The user at the MCData client 1 initiates a file distribution request to multiple MCData users selecting a pre-configured group (identified by MCData group ID) and optionally particular members from that group.

2.	MCData client 1 sends a MCData group standalone FD request towards the MCData server. File metadata information is included in the SDP. The MCData group standalone FD request contains target recipient(s) as selected by the user at MCData client 1. The MCData group standalone FD request contains conversation identifier for message thread indication. The MCData group standalone FD request may include additional implementation specific information in the application metadata container. MCData group standalone FD request may contain mandatory download indication. The MCData group standalone FD request may contain download completed report indication if selected by the user at MCData client 1. MCData user at MCData client 1 may include a functional alias within the FD data transfer.

If the MCData user at MCData client 1 initiates an MCData emergency file distribution communication or the MCData emergency state is already set for the MCData client 1 (due to a previously triggered MCData emergency alert):

i)	the MCData group standalone FD request shall contain an emergency indicator;

ii)	the MCData group standalone FD request shall set an alert indicator if configured to send an MCData emergency alert while initiating an MCData group standalone FD request for the emergency file distribution service communication; and

iii)	if the MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state is retained until explicitly cancelled.

NOTE 1:	While MCData client 1 is in the emergency state, all types of MCData one-to-one and group communications initiated by MCData client 1 are initiated as MCData emergency communications.

If the MCData user at MCData client 1 initiates an MCData imminent peril file distribution communication:

i)	the MCData group standalone FD request shall contain an imminent peril indicator.

2a.	If either emergency indicator or imminent peril indicator is present in the received MCData group standalone FD request, the MCData server implicitly affiliates MCData client 1 to the MCData group if the client is not already affiliated.

3.	MCData server checks whether the MCData user at MCData client 1 is authorized to send MCData group standalone FD request. MCData server verifies whether the provided functional alias, if present, can be used and has been activated for the user. The MCData server resolves the MCData group ID to determine the members of that group and their affiliation status, based on the information from the group management server.

i)	If an emergency indicator is present in the received MCData group standalone FD request and if the MCData group is not in the in-progress emergency state, the MCData group is considered to be in the in-progress emergency state until cancelled; and

NOTE 2:	While the MCData group is in the in-progress emergency state, all types of MCData communications within the group are processed as emergency group communications by the MCData server. MCData group members that are not in the emergency state do not indicate emergency in group communication requests.

ii)	If an imminent peril indicator is present in the received MCData group standalone FD request and if the MCData group is not in the in-progress imminent peril state, the MCData group is considered to be in the in-progress imminent peril state until cancelled.

4.	The MCData server also applies transmission and reception control and the necessary policy to ensure that appropriate data is transmitted between the MCData UEs.

5.	MCData server initiates the MCData group standalone FD request towards each MCData user determined in step 3. The MCData group standalone FD request towards each MCData client contains:

i)	an emergency indicator if it is present in the received MCData group standalone FD request from the MCData client 1;

ii)	an imminent peril indicator if it is present in the received MCData group standalone FD request from the MCData client 1; and

iii)	an alert indicator if requested to initiate an emergency alert in the received MCData group standalone FD request from the MCData client 1.

6.	The receiving MCData clients 2 to n notifies the user about the incoming MCData group standalone FD request which may be either accepted or rejected or ignored. If the request includes mandatory download indication in the MCData group standalone FD request an accepted response is assumed.

7.	If the target MCData user on MCData clients 2 to n provides a response (accept or reject) to the notification, then the respective MCData client sends the MCData group standalone FD response to the MCData server. MCData client 2 to n automatically sends accepted MCData group standalone FD response when the incoming request included mandatory download indication.

8.	MCData server forwards the MCData group standalone FD response to the MCData client 1.

NOTE 3:	Step 8 can occur at any time following step 5, and prior to step 9 depending on the conditions to proceed with the file transmission.

9.	MCData client 1 and MCData server have successfully established media plane for file transmission and the MCData client 1 transmits the file data.

10.	MCData server distributes the file received from MCData client 1 to MCData clients 2 to n over the established media plane. Distribution of file can be via unicast or via MBMS bearer(s). For distribution via MBMS bearer(s), the procedure described in subclause 7.3	Use of MBMS transmission (on-network) is executed. File download report is shared by the receiving MCData clients, if requested by the user at MCData client 1. After file transaction is completed, the media plane is released.

NOTE 4:	MCData server is not required to wait for the complete download of file from MCData client 1 prior to initiating file distribution to MCData client 2.

11.	The MCData clients successfully receiving the file, records file download completed and notifies MCData user.

12.	MCData client 2 initiates a MCData download completed report for reporting file download completed, if requested by the user at MCData client 1.

13.	The MCData file download completed report(s) from MCData client(s) may be stored by the MCData server for download history interrogation from the authorized MCData users. The MCData file download completed report from each MCData user may be aggregated.

14.	Aggregated or individual MCData file download completed report is sent to the disposition requesting user at MCData client 1.

# * * * * 11th change * * * *

##### 7.5.2.10.2	Procedure

The procedure in figure 7.5.2.10.2-1 describes the case where a MCData user is initiating group standalone data communication for sending a file to multiple MCData users, with or without download completed report request.

Pre-conditions:

1.	The MCData users on the MCData client 1 to n belong to the same group and are already registered for receiving MCData service and affiliated.

2.	The file to be distributed is uploaded to the media storage function on the MCData content server using the procedure defined in subclause 7.5.2.2.

Figure 7.5.2.10.2-1: Group standalone FD using the MBMS download delivery method

1-3.	Steps 1-3 are the same as in the procedure for Group standalone FD using HTTP (7.5.2.6).

4.	The MCData server executes the procedure described in subclause 7.3.5. The MCData server defines, in the MBMS session properties (subclause 5.4 of 3GPP TS 26.348 [19]), the ingest mode to provide the file into the BM-SC via xMB-U. As described in clause 7.3.5.3.3, the MCData server decides how the file stored in the MCData content server is provided for distribution over the MBMS session.

If the pull ingest mode is defined, the MCData server may provide in this step the file list. As described in 3GPP TS 26.348 [19], the file list includes, among other information, the file URL to be used by the BM-SC to fetch the file and the earliest fetch time. The earliest fetch time may be configured with a long enough delay so that the MBMS session is established and steps 6 to 8 are executed before the delivery over MBMS. The MCData server can also update the MBMS session with the file list in a later step.

If the push ingest mode is defined, the MCData server obtains the URL from the BM-SC to be used to push the file via xMB-U. The MCData server ingests the content into the BM-SC after the MBMS session is established and steps 6 to 8 are performed.

5.	The MCData server initiates the MCData group standalone FD over MBMS request towards each MCData user determined in step 3. The request is sent over unicast or within an MBMS bearer for application level control signalling.

6.	The receiving MCData clients 2 to n notify the users about the incoming MCData group standalone FD over MBMS request (including file metadata, if present).

7.	The MCData clients 2 to n automatically send accepted MCData group standalone FD response when the incoming request included mandatory download indication.

NOTE 1:	When the UE is in idle mode, MCData clients may skip step 8.

NOTE 2:	If the pull ingest mode was defined in step 5 and the file list has not been provided yet, the MCData server updates the MBMS session with the file list. If the push ingest mode was defined, the MCData server can start pushing the file for distribution over MBMS.

8.	The MCData server forwards the MCData group standalone FD responses to the MCData client 1.

NOTE 3:	Step 8 can occur at any time following step 6, and prior to step 10 depending on the conditions to proceed with the file transmission.

9.	The MCData clients receive the file delivered over MBMS.

10.	If losses occurred during the file delivery over MBMS, the MCData clients may download the missing parts using the procedures defined in subclause 7.5.2.3.

NOTE 4:	If the file is not successfully received over MBMS, e.g. due to a poor MBMS reception quality, the media storage client of the MCData client(s) can download the file using the procedure defined in subclause 7.5.2.3.

11.	The MCData clients, after reception, initiate MCData download completed reports for reporting file download completed, if requested by the user at MCData client 1.

12.	The MCData file download completed reports from the MCData clients may be stored by the MCData server for download history interrogation from authorized MCData users. The MCData file download completed report from each MCData user may be aggregated.

13.	Aggregated or individual MCData download completed reports are sent by the MCData server to the MCData user at MCData client 1.

# * * * * 12th change * * * *

##### 7.5.3.4.2	Procedure

Figure 7.5.3.4.2-1 describes procedures for an off-network MCData client 1 initiating group MCData data communication for sending FD data to a MCData group, with or without download completed report request.

Pre-conditions:

1.	MCData user 1 has initiated group communication for sending FD data to the MCData group.

2.	Information for ProSe direct communications corresponding to the MCData group and its mapping to ProSe Layer-2 Group ID are pre-configured in MCData client 1.

3.	MCData client 1 to MCData client N are members of the same MCData group.

Figure 7.5.3.4.2-1: Group standalone file distribution using media plane

1.	MCData client 1 sends a MCData group standalone FD request towards the MCData group. File metadata information is included in the SDP. The MCData group standalone FD request contains MCData group ID as selected by the user at MCData client 1. The MCData group standalone FD request contains conversation identifier for message thread indication. The MCData group standalone FD request may include additional implementation specific information in the application metadata container. MCData group standalone FD request may contain mandatory download indication. The MCData group standalone FD request may contain download completed report indication if selected by the user at MCData client 1.

2.	On receiving a MCData group standalone FD request, the MCData clients check whether any policy is to be asserted to limit certain types of message or content to certain members, for example, due to location or user privilege.

3.	If the policy assertion is positive, the receiving MCData clients 2 to n notifies the user about the incoming MCData group standalone FD request which may be either accepted or rejected or ignored. MCData user may not be sought consent if the request includes mandatory download indication in the MCData group standalone FD request and instead only notify the MCData user about file downloading.

4.	If the target MCData user on MCData clients 2 to n provides a response (accept or reject) to the notification, then the respective MCData client sends the MCData group standalone FD response to the MCData client 1. MCData client 2 to n automatically sends accepted MCData group standalone FD response when the incoming request included mandatory download indication.

5.	MCData client 1 and MCData client 2 to n have successfully established media plane for file transmission and the MCData client 1 transmits the file data.

6. The MCData client 2 to n successfully receiving the file, records file download completed and notifies MCData users.

7. MCData client 2 to n initiate a MCData download completed report for reporting file download completed, if requested by the user at MCData client 1.

# * * * * 13th change * * * *

###### 7.7.2.3.2.2	Procedure

The procedure in figure 7.7.2.3.2.2-1 describes signalling control plane procedure for the case where during an ongoing MCData communication, based on communication release conditions, MCData server initiates the communication release. The procedure is applicable for one-to-one and group communication.

Pre-conditions:

1.	MCData users on MCData client 1, client 2 and client 3 are already registered for receiving MCData service.

2.	A MCData administrator has configured the limits for the maximum amount of data and time that a participant transmits from a single request to transmit.

3.	A MCData communication is ongoing between MCData client 1, client 2 and client 3.

Figure 7.7.2.3.2.2-1: MCData server initiated release of MCData communication using media plane

1.	MCData server would like to release the ongoing MCData communication, identified by conversation identifier, since at least one of the release conditions are met e.g. lack of capacity, limit for the maximum amount of data or time that a participant transmits from a single request to transmit exceeded. Based on configuration, MCData server decides to pre-empt the MCData communication without giving prior indication to MCData client 1.

2.	MCData server identifies the participants of the ongoing MCData communication and generates communication release request to release ongoing MCData communication.

3.	MCData server sends MCData server communication release request towards each participant of the MCData communication.

4.	MCData users are notified about the release of the MCData communication.

5.	MCData client at each MCData communication participant sends MCData server communication release response towards the MCData server.

6.	All participants of the MCData communication have successfully released the media plane resources associated with the MCData communication that is released.