3GPP TSG-SA WG6 Meeting #70	S6-255619

Dallas, United States 17th – 21st November 2025	(revision of S6-255179, 255554)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-35 | CR | 0009 | rev | 2 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on the solution 6 for the conclusion of technical gap#1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_SEAL_Ph4 |  |  |  |  |  | Date: |  |  | 2025-11-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - It is suggested to complete the solution description about NRM API for multiple NRM services and correct some description in overall evaluation according to the discussion paper in S6-254038. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Solution changes to 8.1.6.2, 8.1.6.3 to clarify the link between NRM service and NRM API. - Correcting the wrong description in clause 9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Some misunderstanding may exist, which casue difficult for conclusion. - The overall evaluation is not capture the solution correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.1.1, 8.1.6.1.1.2, 8.1.6.1.1.2.1, 8.1.6.1.1.2.2, 8.1.6.2, 8.1.6.3,8.1.6.4, 9.1.1, 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.1.6.1.1.1		QoS control service for network transport with additional indication of SLA on QoS service type

The figure 8.1.6.1.1.1-1 illustrates the high-level procedures of creating the QoS control service for network transport with high data rate and low latency service.

Pre-conditions:

SLA on QoS service type (e.g., Table 8.a.1.3-1) has been signed as part of agreement between ASP and MNO.

Figure 8.1.6.1.1.1-1: Create QoS control service for network transport with high data rate and low latency

1.	The VAL server invokes the QoS service with the indication of SLA on the QoS service type if there is SLA agreed for the QoS type between VALprovider and NRM service provider (e.g., indicating Nrm_QosCtrlHighDataRateLowLatency, or other type of value listed in Table 8.a.1.3-1) create service.For example, VAL can use the indication of Nrm_QosCtrlHighDataRateLowLatency to request the network transport resource with high data rate and low latency QoS guarantee. The high data rate requirements (e.g., requested maximum bitrate, requested guaranteed bitrate, media type, media format, bandwidth), low latency requirements (e.g., UL, DL and/or round-trip latency) are included. The service identifier which indicates the high data rate requirements and the low latency requirements are applied and the UE ID are also included.

This step can also be used by VAL server invoking QoS service type with the indication of SLA on other type of QoS service (e.g., listed in Table 8.1.6.2-1).

2.	Based on the request in step 1, the NRM server perform authorization check e.g., check whether the indicated SLA on QoS service type (e.g., Nrm_QosCtrlHighDataRateLowLatency service, or other value listed in Table 8.a.1.3-1) is included in the SLA profile assciacted with VAL server if provided in step1. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_create request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_create request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_create response and the Npcf_PolicyAuthorization_create response.

3.	The NRM server returns the QoS service with the indication of SLA on the QoS service type create response indication the result.

8.1.6.1.1.2	Information flow

8.1.6.1.1.2.1 Network resource adaptation request

Table 8.1.6.1.1.2-1 describes the information flow network resource adaptation request from the VAL/SEALDD server to the NRM server for request the different QoS services.

Table  8.1.6.1.1.2.1-1: Network resource adaptation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL UE IDs | O (see NOTE 1) | List consisting of one or more VAL UE IDs for whom the network resource adaptation occurs. |
| VAL group ID | O (see NOTE 1) | The VAL group ID for whom the network resource adaptation occurs. |
| Resource adaptation requirement(s) | M | The resource adaptation requirement corresponds to the VAL service QoS requirements as applied for a UE or group of UEs (E.g. bandwidth, resource). |
| >SLA on QoS service type indication | O | SLA on QoS service type which is part of the agreement between VAL and the NRM service provider. It may indicate one of the following indication, for example: Nrm_QosCtrlLowLatencyHighBWHighReliability, or Nrm_QosCtrlLowLatencyHighReliability, Nrm_QoSCtrlLowLatencyHighBW, Nrm_QoSCtrlLowLatency). |
| >QoS requirement | M | The QoS requirement corresponds to the VAL service QoS requirement on the value as per the traffic type (e.g, data rate, GBR, MBR, latency, reliability, bandwidth, etc., and/or its combinations). |
| BDT Reference ID | O | Indicates the Background data transfer Reference ID provided by the 3GPP network |
| NOTE 1: Either of the information elements should be present. |  |  |

8.1.6.1.1.2.2	Network resource adaptation response

Table 8.1.6.1.1.2.2-1 describes the information flow network resource adaptation response from the NRM server to the VAL server.

Table 8.1.6.1.2.2.2-1: Network resource adaptation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network resource adaptation with the underlying network. The response can also include an updated value for some of the parameters included in the network resource adaptation request (e.g. negotiation of resource offering) |
| Cause | O (See NOTE) | Cause of failure (e.g. QoS requirement for traffic type cannot be satisfied). |
| NOTE: The information element shall be present only when the result is failure. |  |  |

* * * First Change * * * *

8.1.6.2	NRM services

The following new NRM service listed in table 8.1.6.2-1 are introduced:

Table 8.1.6.2-1: List of new QoS SLA service names for network resource control

| Service Name | Known Consumer(s) | Communication Type | NOTE |
| --- | --- | --- | --- |
| Nrm_QoSCtrlLowLatencyHighBW, | VAL | Request/response | No special reliable requirement comparing to network average performance, so no special treatment for reliable will be activiated by network. |
| Nrm_QoSCtrlLowLatency | VAL | Request/response | No special reliable and data rate requirement comparing to network average performance, so no special treatment for reliable and data rate will be activiated by network. |
| Nrm_QosCtrlLowLatencyHighReliability, | VAL | Request/response | No special data rate requirement comparing to network average performance, so no special treatment for data rate will be activiated by network. |
| Nrm_QosCtrlLowLatencyHighBWHighReliability | VAL | Request/response |  |

8.1.6.3	NRM services API

NRM services defined in 8.1.6.2 is to label the addition resource type (comparing to the average QoS performance based best effort) what API invoker can request for different application traffic. It is possible that specifying one NRM service API to support SLA on multiple QoS services type (listed in 8.1.6.1)which are signed as the party of agreement between ASP and MNO and used for the authirozation check when the service creat request is received.

8.1.6.4	Solution evaluation

This solution provides several new NRM services for network service QoS control to the application service providers/developers. Each of the new NRM service represents a different QoS service catalog and allow the application stakeholders to choose a proper QoS control service corresponding to their application requirements.

* * *Next Change * * * *

9.1.1	Overall evaluation on technical gap #1

Technical gap #1 focuses on NRM service to support the latest network capabilities, such as multi-modal traffic. Technical solutions 6, 7, and 10 in clause 8.1.6, 8.1.7, and 8.1.10 respectively address this gap.

Solution 6 focuses on application layer requirements by introducing a new set of NRM services to support resource adaption by different applications for different traffic type. The NRM services focuses on specific application requirements such as QoS control service for network transport with high data rate and low latency, QoS control service for network transport with low latency, QoS control service for network transport with low latency and high reliability, QoS control service for network transport with low latency & high bandwidth & high reliability. This enables to create, update and delete the QoS control service for network transport with traffic type specific QoS requirements. It allows to enhance network resource adaptation API request by adding indication on SLA of QoS type to indicating different type of traffic QoS requirements.

On the other hand, solution 7 focuses on the enhancement of input parameters of the network resource adaptation request from VAL server to the NRM server. The network resource adaptation API request is enhanced by adding traffic type to indicate different types of traffic involved (for e.g., audio, video, haptic), priority of the traffic types, and QoS requirement per traffic. The enhanced NRM network resource adaptation service allows the application client/server to add the QoS requirements for each traffic type separately. This addition supports newer network capabilities such as multi-modal application traffic that requires to send traffic type specific QoS requirements to the 5GC. This is later mapped with the AFSessionWithQoS for multi-modal service. Hence, it introduces an enhanced NRM service that makes it suitable for scenarios with single or multiple traffic types, irrespective of the application type. The only difference with solution 6 is on that criterial of NRM resource adaption services 1) Solution 6 suggest to design the service based on the QoS type required by different traffic type 2) Solution 7 suggest to using traffic type directly.

Furthermore, Solution 10 focuses on single NRM service for network service QoS control for applications. The single NRM API includes inputs to define the basic QoS requirements for multiple applications. The input includes QoS requirements such as Data rate (Maximum uplink data rate and Maximum downlink data rate), Delay characteristics (maximum uplink delay, Maximum downlink delay, round trip delay, delay variance), and Maximum packet loss rate.

* * * Next Change * * * *

10.2	Conclusions of SEAL services technical gap #x

This clause provides the final conclusion for each technical gap.

Technical Gap#1:

- Take the technical solution#6,#7,10 as the input for normotive work based on the following principle:

- Allow QoS requirement value without service type indications the basis, optionally support the solutions of supporting additional service type indication (technical solution# 6 as the input) or additional traffic type indication (as technical solution 7 ).……