3GPP TSG-SA6 Meeting #69	S6-254187

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.392 | CR | 0022 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DC application and profile configuration request update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile M2M Company Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MMTel_App |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The minimum UE software version (e.g. OS version, SDK version, etc.) required for this Data Channel Application is proposed to be added in DC application list. If UE’s OS version or SDK version cannot fulfill the minimum version requirement, the user can see the application but the installation and usage of application will face failure which will impact the experiences, or when the version of application makes upgrade and have higher requirement of UE minimum software version, the existing end user experiences may be impacted. Therefore, this CR is proposed to add this detailed requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is proposed to add the minimum UE software version required for this Data Channel Application in the DC application profile. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Data Channel Application installation and usage may fail due to the low SW version of UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.3.1, 8.3.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

Table 8.2.2.3.1-2: Information elements in DC application list

| Information element | Status | Description |
| --- | --- | --- |
| Application Identifier | M | Identifier of the Data Channel Application assigned by the Application Provider |
| Application Name (see NOTE 1) | O | Name of the Data Channel Application |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application |
| Application Icon Url | O | Icon url of the Data Channel Application |
| Application Version (see NOTE 1) | O | The latest version of the application. |
| Application Validity | O | The latest validity of the application. |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: Precall only: The Data Channel Application is allowed to be used before the MMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. Incall: The Data Channel Application is allowed to be used after the MMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. Both precall and Incall: The Data Channel Application is allowed to be used during the entire Precall and incall. |
| Requested UE Version | O | Indicates minimum UE software version (e.g. OS version, SDK version, etc.) required for this Data Channel Application. |
| Autoload (see NOTE 3) | O | Indicates whether this Data Channel Application is needed to be loaded to the UE automatically. |
| Autolaunch (see NOTE 3) | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and launched automatically. |
| Usage Indication (see NOTE 2) | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Requested Media Type (see NOTE 2) | O | Indicates media type required for this Data Channel Application. The values of this IE include: Voice call only: This Data Channel Application can be used if and only if the corresponding call is a voice call. Video call only: This Data Channel Application can be used if and only if the corresponding call is a video call. Both Voice and video call: This Data Channel Application can be used in both voice call and video call. |
| Application Usage Condition (see NOTE 2) | O | Indicates whether this Data Channel Application can be used based on certain condition. The values of this IE include, but not limited: Service area: the application is allowed to be used in a certain area. Time period: the application is allowed to be used for a certain period of time. |
| 3gpp QoS Hint (see NOTE 2) | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| Personal Data Collection Indication (see NOTE 4) | O | Indicates whether this Data Channel Application will collect the user personal data (along with related consent opt-in/opt-out procedures). |
| Personal Data Collection Info URL (see NOTE 4) | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected, their processing their protection etc. |
| Application Package (see NOTE 1) | O | The Code package of the Data Channel Application. |
| NOTE 1: This IE shall be present when when configuring the DC application. NOTE 2: This IE shall be present when when configuring the DC application profile. NOTE 3: These IEs can be applied only if the specific service using this data channel application is agreed to be used by the user based on the subscription of this specific service or explicitly agreed to be applied by the user on the UE. The relationship with Personal Data Collection should be considered, which is out of 3GPP scope. NOTE 4: These IEs are mandatory when regulatory requirements request that the user has to be informed if the personal data are collected by the application and how they are treated. |  |  |

* * * Next Change * * * *

Table 8.3.2.3.4-2: Information elements in DC application profile

| Information element | Status | Description |
| --- | --- | --- |
| APPID | M | Identifier of the Data Channel Application. |
| Application Name | O | Name of the Data Channel Application. |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application. |
| Application Icon | O | Icon of the Data Channel Application |
| Application Version | O | The latest version of the application. |
| Application Validity | O | The latest validity of the application. |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: Precall only: The Data Channel Application is allowed to be used before the MMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. Incall only: The Data Channel Application is allowed to be used after the MMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. |
| Requested UE Version | O | Indicates minimum UE software version (e.g. OS version, SDK version, etc.) required for this Data Channel Application. |
| Autoload (see NOTE 1) | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch (see NOTE 1) | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and launched automatically. |
| Usage Indication | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Requested Media Type | O | Indicates media type required for this Data Channel Application. The values of this IE include: Voice call only: This Data Channel Application can be used if and only if the corresponding call is a voice call. Video call only: This Data Channel Application can be used if and only if the corresponding call is a video call. Both voice and video call: This Data Channel Application can be used in both voice call and video call. |
| Application Usage Condition | O | Indicates whether this Data Channel Application can be used if under this condition. The values of this IE include but not limited: Service area: The application is allowed to be used in a certain area. Time period: The application is allowed to be used for a certain period of time. |
| 3gpp QoS Hint | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| Personal Data Collection (see NOTE 2) | O | Indicates whether this Data Channel Application will collect the user personal data. |
| Personal Data Collection Info URL (see NOTE 2) | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected (along with related consent opt-in/opt-out procedures), their processing their protection etc. |
| NOTE 1: These IEs can be applied only if the specific service using this data channel application is agreed to be used by the user based on the subscription of this specific service or explicitly agreed to be applied by the user on the UE. NOTE 2: These IEs are mandatory when regulatory requirements request that the user has to be informed if the personal data are collected by the application and how they are treated. |  |  |

* * * End of Changes * * * *