3GPP TSG-SA WG6 Meeting #69	S6-254209

Wuhan, China 13th – 17th October 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0161 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe multihop configurations addition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MultiHop-MC |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR includes the necessary configurations to enable MC services over 5G ProSe multihop UE-to-Network relay and 5G ProSe multihop UE-to-UE relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - add MC service UE configuration data related to support utilizing 5G ProSe multihop UE-to-UE. - add MC service UE configuration data related to support utilizing UE-to-Network relay. - UE-to-UE indicator is added to table A.4-2 as it is missing for UE-to-UE relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing 5G ProSe multihop UE-to-Network relay and multihop UE-to-UE relay configurations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4 and A.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# A.4	MC service group configuration data

The configuration data defined in 3GPP TS 23.379 [6] in Annex A.4, 3GPP TS 23.281 [4] in Annex A.4, 3GPP TS 23.282 [5] in Annex A.4, apply, with the following exceptions:

-	" Default PQI (as specified in 3GPP TS 23.304 [17]) values" instead of "Default ProSe Per-Packet priority (as specified in 3GPP TS 23.303 [7]) values" shall be used;

Furthermore, Table A.4-1 and Table A.4-2 include the necessary configurations to enable 5G ProSe UE-to-Network Relay and 5G ProSe UE-to-UE Relay services (off-network communication), respectively.

Table A.4-1: Common group configuration data to enable 5G ProSe UE-to-network relay service (on-network)

| Reference | Parameter description | MC service UE | MC service Server | Group management server |
| --- | --- | --- | --- | --- |
| Subclause 4.8.2 | RSC(s) associated to 5G ProSe UE-to-Network Relay services (as specified in 3GPP TS 23.304 [17]) | Y | N | Y |
|  | > UE-to-Network Relay Layer indicator per RSC (NOTE 1) |  |  |  |
| Subclause 4.8.3 | RSC(s) associated to 5G ProSe multihop UE-to-Network Relay services (as specified in 3GPP TS 23.304 [17]. | Y | N | Y |
|  | > UE-to-Network Relay Layer indicator per RSC (NOTE 1) | Y | N | Y |
|  | > Multihop indicator per RSC (NOTE 2) | Y | N | Y |
|  | > Maximum number of hops per RSC (NOTE 3) | Y | N | Y |
| NOTE 1: The UE-to-Network Relay Layer indicator indicates whether the associated RSC offers 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. NOTE 2: Indicates whether the associated RSC supports multihop connectivity, i.e., 5G ProSe multihop UE-to-Network Relay operation. NOTE 3: Indicates the maximum number of hops which can be supported by the associated RSC. |  |  |  |  |

Table A.4-2: Common group configuration data to enable 5G ProSe UE-to-UE relay service (off-network)

| Reference | Parameter description | MC service UE | MC service server | Group management server |
| --- | --- | --- | --- | --- |
| Subclause 4.10.2 | RSC(s) associated to 5G ProSe UE-to-UE Relay services (as specified in 3GPP TS 23.304 [17]) | Y | N | Y |
|  | > UE-to-UE Relay Layer indicator per RSC (NOTE 1) |  |  |  |
| Subclause 4.10.2 | ProSe layer-2 group ID (as specified in 3GPP TS 23.304 [17]) | Y | N | Y |
| Subclause 4.10.3 | RSC(s) associated to 5G ProSe multihop UE-to-UE Relay services (as specified in 3GPP TS 23.304 [17]) |  |  |  |
|  | > UE-to-UE Relay Layer indicator per RSC (NOTE 1) | Y | N | Y |
|  | > Multihop indicator per RSC (NOTE 2) | Y | N | Y |
|  | > Maximum number of hops per RSC (NOTE 3) | Y | N | Y |
| NOTE 1: The UE-to-UE Relay Layer indicator indicates whether the associated RSC offers 5G ProSe Layer-2 or Layer-3 UE-to-UE Relay service. NOTE 2: Indicates whether the associated RSC supports multihop connectivity, i.e., 5G ProSe multihop UE-to-UE Relay operation. NOTE 3: Indicates the maximum number of hops which can be supported for the associated RSC. |  |  |  |  |

# * * * * Second change * * * *

# A.x 	MC UE configuration data

Annex A.2 in 3GPP TS 23.379 [6], Annex 2 in 3GPP TS 23.281 [4] and Annex 2 in 3GPP TS 23.282 [5] applies with the following updates in Table A.2-2 in 3GPP TS 23.379 [6], Table A.2-2 in 3GPP TS 23.281 [4] and Table A.2-2 in 3GPP TS 23.282 [5] to support 5G ProSe UE-to-Network Relay services as described in clause 4.8.

Table A.x-1: UE configuration data to enable 5G ProSe UE-to-Network Relay service (on network)

| Reference | Parameter description |
| --- | --- |
| Subclause 4.8.2 | Relay service (Y/N) |
| Subclause 4.8.2 | List of allowed relayed MCPTT groups, MCVideo groups, or MCData groups and their RSC(s) (as specified in 3GPP TS 23.304 [17]) (optional) (see NOTE 1) |
|  | > MCPTT group ID, MCVideo group ID, or MCData group ID |
|  | > RSC(s) for 5G ProSe UE-to-Network Relay service (as specified in 3GPP TS 23.304 [17]) |
|  | >> UE-to-Network Relay Layer indicator (Note 2) |
| Subclause 4.8.3 | RSC(s) for 5G Prose multihop UE-to-Network Relay service (as specified in 3GPP TS 23.304 [17]) |
|  | > UE-to-Network Relay Layer indicator per RSC (NOTE 2) |
|  | > Multihop indicator per RSC (NOTE 3) |
|  | > Maximum number of hops per RSC (NOTE 4) |
| NOTE 1: When the value of the parameter Relay service is N, this parameter and its child parameters are not needed. NOTE 2: The UE-to-Network Relay Layer indicator indicates whether the associated RSC offers 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. NOTE 3: Indicates whether the associated RSC can be supported for 5G ProSe multihop UE-to-Network Relay operation for the associated RSC. NOTE 4: Indicates the maximum number of hops which can be supported for the associated RSC. |  |

# * * * * End of changes * * * *