3GPP TSG-SA3 Meeting #124	S3‑253669

# Wuhan, China, 13 – 17 October 2025						    					       is the revision of S3-253408

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.545 | CR | 0024 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on AMF knows the message from a NR Femto |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto_Sec |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | From RAN3 decision, Femto will take its ID in NGAP message, the formant of the ID is exactly the same as traditional gNB ID. So, the AMF cannot distinguish a NR Femto from a tradional gNB from the ID. No other information can be used, either. This brings an issue the AMF may not know the CAG ID carried in NG-AP message shall be verified as defined in clause 5.8 in TS 33.545. If this happens, the CAG ID verification is bypassed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an additional indication in NGAP message is taken to make the AMF know this is a message from NR Femto. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scheme defined in clause 5.8 is bypassed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************Start of Change************

## 5.8	CAG ID verification

The NR Femto GW shall verify that all UE associated messages from a NR Femto cell can be mapped to a specific CAG ID and that this CAG ID is allowed for the identity of the originating NR Femto. In the absence of a NR Femto GW, the NR Femto shall include in NG-AP message an information that the message is orignated from an NR Femto cell, and then the AMF shall perform this CAG ID verification whether the CAG ID is allowed to be used by the NR Femto cell.

One NR Femto can host multiple Femto cells. Cells may belong to PNI-NPN. Hence, the CAG ID verification shall be applied to all the CAG IDs mapping to individual Femto cells. The existing CAG concept defined for PNI-NPN in TS 23.501 [2] clause 5.30.3 shall be re-used for Femtocell access control.

NOTE 1: 	The CAG ID being verified may be explicitly present in the message as an information element or may be mapped by other means.

NOTE 2:	The above requirement implies that the network has to ensure that all UE-associated messages from Femto cell are subject to the above verification, even if a (rogue) message from a compromised Femto would not indicate to originate from a NR Femto.

NOTE 3: It is left to implementation on how the AMF or Femto GW knows the CAG ID that the NR Femto cell can use.

************End of Change************