3GPP TSG-SA Meeting #110	SP-251561

Baltimore, United States, 9 - 12 December, 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.369 | CR | 0070 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIoT correction to the authentication in inventory service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | China Mobile,Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-SEC |  |  |  |  |  | Date: |  |  | 2025-11-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Meeting #131bis, RAN2 sent an LS (R2-2507920) on Security parameter in A-IoT paging to SA3, stating that: From RAN2 point of view, while it is feasible to include 128 bits security parameter in A-IoT paging message from signaling perspective, RAN2 has following concerns with making the 128 bits mandatory for every paging message: - Deployment: secure indoor deployments, where it may not be needed to always keep re-authenticating a specific known device in every single-device CFA paging message if it has already completed the authentication. - Overhead: high overhead to transmit 128 bit RANDAIOT_n in A-IoT paging message and to transmit 128 bit RANDAIOT_d and 32bit RESAIOT in D2R message. It should be noted there is no segmentation in R2D message. - Complexity and time consuming: complexity and time consuming for devices to perform authentication calculations. - Power consumption: power consumption overhead with authentication. - Coverage reduction: coverage reduction due to larger payload size of the A-IoT paging message and D2R message. RAN2 asked SA3: Whether the 128 bits security parameter has to be included in every paging message? To address RAN2’s concerns, by considering the overhead and complexity, authentication in inventory service may not always be needed in all scenarios, for example, in some indoor deployments where the physical security can be ensured, operators can have the flexibility to decide the activation of authentication. Therefore, the AIoT authentication requirements need to be refined with the consideration above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the authentication requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The overhead of AIoT authentication makes the inventory service not usable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************BEGIN OF CHANGES*************

4.2.2	Requirements on the AIOTF

4.2.2.1	Requirement on Authentication

The AIOTF shall authenticate the AIoT device.

NOTE: In the indoor deployments in private network where operators can have the flexibility to decide the activation of authentication, the authentication of AIoT device in inventory functional service may not be supported.

******************** CHANGES END******************