3GPP TSG-SA5 Meeting #104 	S5-156251

# 16-20 November 2015, Anaheim (USA)												revision of S5-156117

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.659 | CR | 0015 | rev | 1 | Current version: | 11.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Align id attribute definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM11 |  |  |  |  |  |  | Date: |  | 2015-11-17 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - Align with IS (28.658) – id attribute is missing but has to be defined for IOCs both in mapping table and IDL. - The id attribute, which is inherited from IOC Top_ in TS 28.620 via IOC ManagedFunction, is repeated in the XxxFunction mapping table but according to the IRP SS methodology, inherited items shall not be repeated in mapping tables. - IDL definitions for a number of classes inheriting from ManagedFunction are repeating the definition of id (or xxxId) attribute, but they should not as it is inherited from ManagedFunction via IDL inheritance. - Scope needs to be updated for SS-IS version link, as the IS (28.658) is updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Align with IS (28.658) – id attribute has to be defined for IOCs both in mapping table and IDL. - Remove the id attribute from the XxxFunction mapping table. - Remove the definition of id (or xxxId) attribute from IDL definitions for a number of classes inheriting from ManagedFunction. - Scope needs to be updated for SS-IS version link to point to the latest IS version (28.658). - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TS would have faults which potentially could lead to implementation errors and interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, A.2.2.1, A.2.2.2, A.2.2.3, A.2.2.6, A.2.2.7, A.2.2.9, A.2.2.12, A.2.2.14, A.2.2.15, A.2.2.16, A.2.2.17, A.2.2.18, A.2.2.19, A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 1	Scope

The present document is part of an Integration Reference Point (IRP) named E-UTRAN Network Resource Model (NRM) IRP, through which an IRPAgent can communicate configuration management information to one or several IRPManagers concerning E-UTRAN resources. The E-UTRAN NRM IRP comprises a set of specifications defining Requirements, a protocol neutral Information Service and one or more Solution Set(s).

# The present document specifies the Solution Sets for the E-UTRAN NRM IRP.

# This Solution Set specification is related to 3GPP TS 28.658 V11.6.X [4].

Next Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TS 32.153: “Telecommunication management; Integration Reference Point (IRP) technology specific templates, rules and guidelines”.

[3]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[4]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[5]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[6]	3GPP TS 32.606: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[7]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[8]	3GPP TS 28.623: “Generic network resources Integration Reference Point (IRP); Solution Set (SS) definition”.

[9]	W3C REC-xml11-20060816: "Extensible Markup Language (XML) 1.1 (Second Edition)".

[10]	W3C XML Schema Definition Language (XSD) 1.1 Part 1: Structures.

[11]	W3C XML Schema Definition Language (XSD) 1.1 Part 2: Datatypes.

[12]	W3C REC-xml-names-20060816: "Namespaces in XML 1.1 (Second Edition)".

Next Modified Section

### A.2.2.1	IOC ENBFunction

Mapping from NRM IOC ENBFunction attributes and associations to SS equivalent MOC ENBFunction attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| eNBId | eNBId | unsignedLong |
| intraANRSwitch | intraANRSwitch | boolean |
| iRATANRSwitch | iRATANRSwitch | boolean |
| x2BlackList | x2BlackList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| x2WhiteList | x2WhiteList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| x2HOBlackList | x2HOBlackList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| x2IpAddressList | x2IpAddressList | genericEUTRANNRMAttributeTypes:: ipAddressListType |
| tceIDMappingInfoList | tceIDMappingInfoList | genericEUTRANNRMAttributeTypes:: TceIDMappingInfoListType |
| sharNetTceMappingInfoList | sharNetTceMappingInfoList | genericEUTRANNRMAttributeTypes:: SharNetTceMappingInfo |
| Note: For all conditional qualifiers, see attribute constraints in 28.658 [4] |  |  |

Next Modified Section

### A.2.2.2	IOC EUtranGenericCell

Mapping from NRM IOC EUtranGenericCell attributes and associations to SS equivalent MOC EUtranGenericCell attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| cellLocalId | cellLocalId | unsignedShort |
| cellSize | cellSize | genericEUTRANNRMAttributeTypes:: cellSizeEnumType |
| plmnIdList | plmnIdList Note: the first plmnId in the SS attribute plmnIdList is the primary PLMN id | genericEUTRANNRMAttributeTypes:: plmnIdListType |
| tac | tac | long |
| pci | pci | short |
| pciList | pciList | genericEUTRANNRMAttributeTypes:: pciListType |
| maximumTransmissionPower | maximumTransmissionPower | short |
| referenceSignalPower | referenceSignalPower | short |
| pb | pb | short |
| partOfSectorPower | partOfSectorPower | short |
| relatedTmaList | relatedTmaList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| relatedAntennaList | relatedAntennaList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| relatedSector | relatedSector | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReference |
| operationalState | operationalState | StateManagementIRPOptConstDefs:: OperationalStateTypeOpt |
| administrativeState | administrativeState | StateManagementIRPOptConstDefs:: AdministrativeStateTypeOpt |
| availabilityStatus | availabilityStatus | StateManagementIRPOptConstDefs:: AvailabilityStatusTypeOpt |
| cellResvInfo | cellResvInfo | genericEUTRANNRMAttributeTypes:: cellResvInfoType |
| allowedAccessClasses | allowedAccessClasses | genericEUTRANNRMAttributeTypes:: allowedAccessEnumClassesType |
| isChangeForEnergySavingAllowed | isChangeForEnergySavingAllowed | GenericNetworkResourcesIRPSystem:: AttributeTypes::yesNoType |
| Note: For all conditional qualifiers, see attribute constraints in 28.658 [4] |  |  |

Next Modified Section

### A.2.2.3	IOC ExternalEUtranGenericCell

Mapping from NRM IOC ExternalEUtranGenericCell attributes and associations to SS equivalent MOC ExternalEUtranGenericCell attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| pci | pci | short |
| plmnIdList | plmnIdList Note: the first plmnId in the SS attribute plmnIdList is the primary PLMN id | genericEUTRANNRMAttributeTypes:: plmnIdListType |
| cellLocalId | cellLocalId | unsignedShort |
| eNBId | eNBId | unsignedLong |

Next Modified Section

### A.2.2.6	IOC EUtranRelation

Mapping from NRM IOC EUtranRelation attributes and associations to SS equivalent MOC EUtranRelation attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| tCI | tCI | long |
| isRemoveAllowed | isRemoveAllowed | boolean |
| isHOAllowed | isHOAllowed | boolean |
| adjacentCell | adjacentCell | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReference |
| isICICInformationSendAllowed | isICICInformationSendAllowed | boolean |
| isLBAllowed | isLBAllowed | boolean |
| isESCoveredBy | isESCoveredBy | genericEUTRANNRMAttributeTypes::IsEsCoveredByEnumType |
| cellIndividualOffset | cellIndividualOffset | genericEUTRANNRMAttributeTypes::qOffsetEnumType |
| qOffset | qOffset | genericEUTRANNRMAttributeTypes::qOffsetEnumType |
| NOTE: For all conditional qualifiers, see attribute constraints in 28.658 [4] |  |  |

Next Modified Section

### A.2.2.7	IOC Link_ENB_ENB

None.

Next Modified Section

### A.2.2.9	IOC ExternalENBFunction

Mapping from NRM IOC ExternalENBFunction attributes and associations to SS equivalent MOC ExternalENBFunction attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| eNBId | eNBId | unsignedLong |

Next Modified Section

### A.2.2.12	IOC MCEFunction

None.

Next Modified Section

### A.2.2.14	IOC RNFunction

Mapping from NRM IOC RNFunction attributes and associations to SS equivalent MOC RNFunction attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| servingCell | servingCell | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReference |
| candidateDeNBCells | candidateDeNBCells | genericEUTRANNRMAttributeTypes:: EcgiListType |

Editor’s note: the need of attribute candidateDeNBCells is for FFS.

Next Modified Section

### A.2.2.15	IOC DeNBCapability

Mapping from NRM IOC DeNBCapabilityattributes and associations to SS equivalent MOC  DeNBCapabilityattributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| servedRN | servedRN | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| maxNbrRNAllowed | maxNbrRNAllowed | unsignedShort |

Next Modified Section

### A.2.2.16	IOC ExternalRNFunction

None.

Next Modified Section

### A.2.2.17	IOC QciDscpMapping

Mapping from NRM IOC QciDscpMapping attributes and associations to SS equivalent MOC QciDscpMapping attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| qciDscpMappingList | qciDscpMappingList | genericEUTRANNRMAttributeTypes:: qciDscpMappingListType |

Next Modified Section

### A.2.2.18	IOC  CellOutageCompensationInformation

Mapping from NRM IOC CellOutageCompensationInformation attributes and associations to SS equivalent MOC CellOutageCompensationInformation attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| cOCStatus | cellOutageCompensationStatus | GenericSONPolicyNRMAttributeTypes:: cellOutageCompensationStatus |
| isCOCAllowed | isCOCAllowed | Boolean |

Next Modified Section

### A.2.2.19	IOC EUtranCellNMCentralizedSON

Mapping from NRM IOC EUtranCellNMCentralizedSON attributes and associations to SS equivalent MOC EUtranCellNMCentralizedSON attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| a1ThresholdRsrp | a1ThresholdRsrp | unsignedShort |
| a1ThresholdRsrq | a1ThresholdRsrq | unsignedShort |
| a2ThresholdRsrp | a2ThresholdRsrp | unsignedShort |
| a2ThresholdRsrq | a2ThresholdRsrq | unsignedShort |
| a3Offset | a3Offset | short |
| a4ThresholdRsrp | a4ThresholdRsrp | unsignedShort |
| a4ThresholdRsrq | a4ThresholdRsrq | unsignedShort |
| a5Threshold1Rsrp | a5Threshold1Rsrp | unsignedShort |
| a5Threshold1Rsrq | a5Threshold1Rsrq | unsignedShort |
| b1ThresholdUtraRscp | b1ThresholdUtraRscp | short |
| b1ThresholdUtraEcN0 | b1ThresholdUtraEcN0 | unsignedShort |
| b1ThresholdGeran | b1ThresholdGeran | unsignedShort |
| b1ThresholdCdma2000 | b1ThresholdCdma2000 | unsignedShort |
| b2Threshold1Rsrp | b2Threshold1Rsrp | unsignedShort |
| b2Threshold1Rsrq | b2Threshold1Rsrq | unsignedShort |
| b2Threshold2UtraRscp | b2Threshold2UtraRscp | short |
| b2Threshold2UtraEcN0 | b2Threshold2UtraEcN0 | unsignedShort |
| b2Threshold2Geran | b2Threshold2Geran | unsignedShort |
| b2Threshold2Cdma2000 | b2Threshold2Cdma2000 | unsignedShort |
| commonChannelPowerOffset | commonChannelPowerOffset | short |
| configurationIndex | configurationIndex | unsignedShort |
| contentionResolutionTimer | contentionResolutionTimer | genericEUTRANNRMAttributeTypes:: contentionResolutionTimerEnumType |
| hysteresisEutraA1 | hysteresisEutraA1 | unsignedShort |
| hysteresisEutraA2 | hysteresisEutraA2 | unsignedShort |
| hysteresisEutraA3 | hysteresisEutraA3 | unsignedShort |
| hysteresisEutraA4 | hysteresisEutraA4 | unsignedShort |
| hysteresisEutraA5 | hysteresisEutraA5 | unsignedShort |
| hysteresisIratB1 | hysteresisIratB1 | unsignedShort |
| hysteresisIratB2 | hysteresisIratB2 | unsignedShort |
| numberOfRaPreambles | numberOfRaPreambles | genericEUTRANNRMAttributeTypes:: numberOfRaPreamblesEnumType |
| preambleInitialReceivedTargetPower | preambleInitialReceivedTargetPower | genericEUTRANNRMAttributeTypes:: preambleInitialReceivedTargetPowerEnumType |
| preambleTransMax | preambleTransMax | genericEUTRANNRMAttributeTypes:: preambleTransMaxEnumType |
| pMax | pMax | short |
| powerRampingStep | powerRampingStep | genericEUTRANNRMAttributeTypes:: powerRampingStepEnumType |
| qHyst | qHyst | genericEUTRANNRMAttributeTypes:: qHystEnumType |
| qOffsetUtra | qOffsetUtra | unsignedShort |
| qOffsetGeran | qOffsetGeran | unsignedShort |
| qOffsetCdma2000 | qOffsetCdma2000 | unsignedShort |
| qQualMinUtra | qQualMinUtra | unsignedShort |
| qRxLevMinEUtraSib1 | qRxLevMinEUtraSib1 | short |
| qRxLevMinEUtraSib3 | qRxLevMinEUtraSib3 | short |
| qRxLevMinGeran | qRxLevMinGeran | unsignedShort |
| qRxLevMinUtra | qRxLevMinUtra | short |
| responseWindowSize | responseWindowSize | genericEUTRANNRMAttributeTypes:: responseWindowSizeEnumType |
| rootSequenceIndex | rootSequenceIndex | unsignedShort |
| sIntraSearch | sIntraSearch | unsignedShort |
| sizeOfRAPreamblesGroupA | sizeOfRAPreamblesGroupA | genericEUTRANNRMAttributeTypes:: sizeOfRAPreambleGroupAEnumType |
| timeToTriggerEutraA1 | timeToTriggerEutraA1 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| timeToTriggerEutraA2 | timeToTriggerEutraA2 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| timeToTriggerEutraA3 | timeToTriggerEutraA3 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| timeToTriggerEutraA4 | timeToTriggerEutraA4 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| timeToTriggerEutraA5 | timeToTriggerEutraA5 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| timeToTriggerIratB1 | timeToTriggerIratB1 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| timeToTriggerIratB2 | timeToTriggerIratB2 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| tReselectionCdma2000 | tReselectionCdma2000 | unsignedShort |
| tReselectionEUtra | tReselectionEUtra | unsignedShort |
| tReselectionGeran | tReselectionGeran | unsignedShort |
| tReselectionUtra | tReselectionUtra | unsignedShort |
| tStoreUeContext | tStoreUeContext | unsignedShort |
| Note: For all conditional qualifiers, see attribute constraints in 28.658 [4] |  |  |

Next Modified Section

## A.3.2	IDL specification "EUtranNetworkResourcesNRMDefs.idl"

//File:EUtranNetworkResourcesNRMDefs.idl

#ifndef _EUTRANNETWORKRESOURCESNRMDEFS_IDL_

#define _EUTRANNETWORKRESOURCESNRMDEFS_IDL_

#include "GenericNetworkResourcesNRMDefs.idl"

#include "EPCResourcesNRMDefs.idl"

#pragma prefix "3gppsa5.org"

/**

* This module defines constants for each MO class name and

* the attribute names for each defined MO class.

*/

module EUtranNetworkResourcesNRMDefs

{

/*

* Definitions for MO class ENBFunction

*/

interface ENBFunction: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ENBFunction";

// Attribute Names

//

const string intraANRSwitch= "intraANRSwitch";

const string iRATANRSwitch= "iRATANRSwitch";

const string eNBId = "eNBId";

const string x2BlackList= "x2BlackList";

const string x2WhiteList= "x2WhiteList";

const string x2HOBlackList= "x2HOBlackList";

const string x2IpAddressList= "x2IpAddressList";

const string tceIDMappingInfoList= "tceIDMappingInfoList";

const string sharNetTceMappingInfoList= "sharNetTceMappingInfoList";

};

/*

* Definitions for MO class RNFunction

*/

interface RNFunction: ENBFunction

{

const string CLASS = "RNFunction";

// Attribute Names

//

const string servingCell = "servingCell";

const string candidateDeNBCells = "candidateDeNBCells";

};

/*

* Definitions for MO class DeNBCapability

*/

interface DeNBCapability: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "DeNBCapability";

// Attribute Names

//

const string id= "id";

const string servedRN= "servedRN";

const string maxNbrRNAllowed= "maxNbrRNAllowed";

};

/*

* Definitions for MO class ExternalRNFunction

*/

interface ExternalRNFunction: ExternalENBFunction

{

const string CLASS = "ExternalRNFunction";

// Attribute Names

//

};

/*

/*

* Definitions for MO class EUtranGenericCell

*/

interface EUtranGenericCell: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "EUtranGenericCell";

// Attribute Names

//

const string cellLocalId = "cellLocalId";

const string cellSize = "cellSize";

const string plmnIdList = "plmnIdList";

const string tac = "tac";

const string pci = "pci";

const string pciList = "pciList";

const string operationalState = "operationalState";

const string administrativeState = "administrativeState";

const string availabilityStatus = "availabilityStatus";

const string maximumTransmissionPower = "maximumTransmissionPower";

const string referenceSignalPower = "referenceSignalPower";

const string pb = "pb";

const string partOfSectorPower = "partOfSectorPower";

const string relatedTmaList = "relatedTmaList";

const string relatedAntennaList = "relatedAntennaList";

const string relatedSector = "relatedSector";

const string allowedAccessClasses = "allowedAccessClasses";

const string isChangeForEnergySavingAllowed = "isChangeForEnergySavingAllowed";

const string cellResvInfo = "cellResvInfo";

};

/*

* Definitions for MO class ExternalEUtranGenericCell

*/

interface ExternalEUtranGenericCell: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ExternalEUtranGenericCell";

// Attribute Names

//

const string pci= "pci";

const string plmnIdList = "plmnIdList";

const string cellLocalId = "cellLocalId";

const string eNBId = "eNBId";

};

/*

* Definitions for MO class EUtranCellFDD

*/

interface EUtranCellFDD: EUtranGenericCell

{

const string CLASS = "EUtranCellFDD";

// Attribute Names

//

const string earfcnDl = "earfcnDl";

const string earfcnUl = "earfcnUl";

};

/*

* Definitions for MO class ExternalEUtranCellFDD

*/

interface ExternalEUtranCellFDD: ExternalEUtranGenericCell

{

const string CLASS = "ExternalEUtranCellFDD";

// Attribute Names

//

const string earfcnDl = "earfcnDl";

const string earfcnUl = "earfcnUl";

};

/*

* Definitions for MO class EUtranCellTDD

*/

interface EUtranCellTDD: EUtranGenericCell

{

const string CLASS = "EUtranCellTDD";

// Attribute Names

//

const string earfcn = "earfcn";

const string sfAssignment = "sfAssignment";

const string specialSfPatterns = "specialSfPatterns";

};

/*

* Definitions for MO class ExternalEUtranCellTDD

*/

interface ExternalEUtranCellTDD: ExternalEUtranGenericCell

{

const string CLASS = "ExternalEUtranCellTDD";

// Attribute Names

//

const string earfcn = "earfcn";

};

/*

* Definitions for MO class EUtranRelation

*/

interface EUtranRelation: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "EUtranRelation";

// Attribute Names

//

const string id = "id";

const string tCI = "tCI";

const string isRemoveAllowed = "isRemoveAllowed";

const string isHOAllowed = "isHOAllowed";

const string adjacentCell = "adjacentCell";

const string isICICInformationSendAllowed = "isICICInformationSendAllowed";

const string isLBAllowed = "isLBAllowed";

const string cellIndividualOffset = "cellIndividualOffset";

const string qOffset = "qOffset";

};

/*

* Definitions for MO class Link_ENB_ENB

*/

interface Link_ENB_ENB: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_ENB_ENB";

// Attribute Names

//

};

/*

* Definitions for MO class Cdma2000Relation

*/

interface Cdma2000Relation:GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "Cdma2000Relation";

// Attribute Names

//

const string id = "id";

const string adjacentSector = "adjacentSector";

};

/*

* Definitions for MO class ExternalENBFunction

*/

interface ExternalENBFunction: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ExternalENBFunction";

// Attribute Names

//

const string eNBId = "eNBId";

};

/*

* Definitions for MO class MCEFunction

*/

interface MCEFunction: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "MCEFunction";

// Attribute Names

//

};

/*

* Definitions for MO class Link_MCE_ENB

*/

interface Link_MCE_ENB: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_MCE_ENB";

// Attribute Names

//

};

/*

* Definitions for MO class Link_MCE_MME

*/

interface Link_MCE_MME: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_MCE_MME";

// Attribute Names

//

};

/*

* Definitions for MO class MBSFNArea

*/

interface MBSFNArea: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "MBSFNArea";

// Attribute Names

//

const string id = "id";

const string mbsfnAreaId= "mbsfnAreaId";

const string cellIdList= "cellIdList";

};

/*

* Definitions for MO class EnergySavingProperties

*/

interface EnergySavingProperties: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "EnergySavingProperties";

// Attribute Names

//

const string id = "id";

const string energySavingState= "energySavingState";

const string energySavingControl= "energySavingControl";

};

/*

* Definitions for MO class CellOutageCompensationInformation

*/

interface CellOutageCompensationInformation: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "CellOutageCompensationInformation";

// Attribute Names

//

const string id = "id";

const string cellOutageCompensationStatus = "cellOutageCompensationStatus";

const string isCOCAllowed =

"isCOCAllowed";

};

/*

* Definitions for MO class EUtranCellNMCentralizedSON

*/

interface EUtranCellNMCentralizedSON: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "EUtranCellNMCentralizedSON";

// Attribute Names

//

const string id = "id";

const string a1ThresholdRsrp = "a1ThresholdRsrp";

const string a1ThresholdRsrq = "a1ThresholdRsrq";

const string a2ThresholdRsrp = "a2ThresholdRsrp";

const string a2ThresholdRsrq = "a2ThresholdRsrq";

const string a3Offset = "a3Offset";

const string a4ThresholdRsrp = "a4ThresholdRsrp";

const string a4ThresholdRsrq = "a4ThresholdRsrq";

const string a5Threshold1Rsrp = "a5Threshold1Rsrp";

const string a5Threshold1Rsrq = "a5Threshold1Rsrq";

const string b1ThresholdUtraRscp = "b1ThresholdUtraRscp";

const string b1ThresholdUtraEcN0 = "b1ThresholdUtraEcN0";

const string b1ThresholdGeran = "b1ThresholdGeran";

const string b1ThresholdCdma2000 = "b1ThresholdCdma2000";

const string b2Threshold1Rsrp = "b2Threshold1Rsrp";

const string b2Threshold1Rsrq = "b2Threshold1Rsrq";

const string b2Threshold2UtraRscp = "b2Threshold2UtraRscp";

const string b2Threshold2UtraEcN0 = "b2Threshold2UtraEcN0";

const string b2Threshold2Geran = "b2Threshold2Geran";

const string b2Threshold2Cdma2000 = "b2Threshold2Cdma2000";

const string commonChannelPowerOffset = "commonChannelPowerOffset";

const string configurationIndex = "configurationIndex";

const string contentionResolutionTimer = "contentionResolutionTimer";

const string hysteresisEutraA1 = "hysteresisEutraA1";

const string hysteresisEutraA2 = "hysteresisEutraA2";

const string hysteresisEutraA3 = "hysteresisEutraA3";

const string hysteresisEutraA4 = "hysteresisEutraA4";

const string hysteresisEutraA5 = "hysteresisEutraA5";

const string hysteresisIratB1 = "hysteresisIratB1";

const string hysteresisIratB2 = "hysteresisIratB2";

const string numberOfRaPreambles = "numberOfRaPreambles";

const string preambleInitialReceivedTargetPower = "preambleInitialReceivedTargetPower";

const string preambleTransMax = "preambleTransMax";

const string pMax = "pMax";

const string powerRampingStep = "powerRampingStep";

const string qHyst = "qHyst";

const string qOffsetUtra = "qOffsetUtra";

const string qOffsetGeran = " qOffsetGeran";

const string qOffsetCdma2000 = "qOffsetCdma2000";

const string qQualMinUtra = "qQualMinUtra";

const string qRxLevMinEUtraSib1 = "qRxLevMinEUtraSib1";

const string qRxLevMinEUtraSib3 = "qRxLevMinEUtraSib3";

const string qRxLevMinGeran = "qRxLevMinGeran";

const string qRxLevMinUtra = "qRxLevMinUtra";

const string responseWindowSize = "responseWindowSize";

const string rootSequenceIndex = "rootSequenceIndex";

const string sIntraSearch = "sIntraSearch";

const string sizeOfRAPreamblesGroupA = "sizeOfRAPreamblesGroupA";

const string timeToTriggerEutraA1 = "timeToTriggerEutraA1";

const string timeToTriggerEutraA2 = "timeToTriggerEutraA2";

const string timeToTriggerEutraA3 = "timeToTriggerEutraA3";

const string timeToTriggerEutraA4 = "timeToTriggerEutraA4";

const string timeToTriggerEutraA5 = "timeToTriggerEutraA5";

const string timeToTriggerIratB1 = "timeToTriggerIratB1";

const string timeToTriggerIratB2 = "timeToTriggerIratB2";

const string tReselectionCdma2000 = "tReselectionCdma2000";

const string tReselectionEUtra = "tReselectionEUtra";

const string tReselectionGeran = "tReselectionGeran";

const string tReselectionUtra = "tReselectionUtra";

const string tStoreUeContext = "tStoreUeContext";

};

module genericEUTRANNRMAttributeTypes

{

enum CellSizeEnumType

{

verysmall,

small,

medium,

large

};

enum AllowedAccessClassesValues

{

EmergencyCall,

ForPLMNUse,

SecurityServices,

PublicUtilities,

EmergencyServices,

PLMNStaff

};

typedef sequence < AllowedAccessClassesValues,6> AllowedAccessClasses

struct PlmnIdType

{

short mcc;

short mnc;

};

const short PLMNID_LIST_LENGTH = 6;

typedef sequence<PlmnIdType > plmnIdListType;

const short NO_OF_PCIS = 504;

typedef sequence<short,NO_OF_PCIS> pciListType;

typedef sequence<string> ipAddressListType;

enum CellResvInfoType

{

reservedCell,

nonReservedCell

};

struct QciDscpMappingType

{

short qci;

short dscp;

};

typedef sequence<QciDscpMappingType> QciDscpMappingListType;

struct EcgiType

{

short mcc;

short mnc;

unsignedlong eci

};

typedef sequence <EcgiType> EcgiListType;

enum isEsCoveredByEnumType

{

no,

partial,

yes

};

enum yesNoType

{

no,

yes

};

struct TceIDMappingInfo

{

short tceID;

string tceIPAddr;

};

typedef sequence<TceIDMappingInfo> TceIDMappingInfoListType;

struct SharNetTceMappingInfo

{

long PLMNId;

short tceID;

string tceIPAddr;

};

typedef sequence<SharNetTceMappingInfo> SharNetTceMappingInfoListType;

enum CellOutageCompensationState

{

cOCActivating,

cOCActive,

cOCDeactivating,

cOCDeactive

};

enum qOffsetEnumType

{

dB-24,

dB-22,

dB-20,

dB-18,

dB-16,

dB-14,

dB-12,

dB-10,

dB-8,

dB-6,

dB-5,

dB-4,

dB-3,

dB-2,

dB-1,

dB0,

dB1,

dB2,

dB3,

dB4,

dB5,

dB6,

dB8,

dB10,

dB12,

dB14,

dB16,

dB18,

dB20,

dB22,

dB24

};

enum contentionResolutionTimerEnumType

{

sf8,

sf16,

sf24,

sf32,

sf40,

sf48,

sf56,

sf64

};

enum numberOfRaPreamblesEnumType

{

n4,

n8,

n12,

n16,

n20,

n24,

n28,

n32,

n36,

n40,

n44,

n48,

n52,

n56,

n60,

n64

};

enum preambleInitialReceivedTargetPowerEnumType

{

dBm-120,

dBm-118,

dBm-116,

dBm-114,

dBm-112,

dBm-110,

dBm-108,

dBm-106,

dBm-104,

dBm-102,

dBm-100,

dBm-98,

dBm-96,

dBm-94,

dBm-92,

dBm-90

};

enum preambleTransMaxEnumType

{

n3,

n4,

n5,

n6,

n7,

n8,

n10,

n20,

n50,

n100,

n200

};

enum powerRampingStepEnumType

{

dB0,

dB2,

dB4,

dB6

};

enum qHystEnumType

{

dB0,

dB1,

dB2,

dB3,

dB4,

dB5,

dB6,

dB8,

dB10,

dB12,

dB14,

dB16,

dB18,

dB20,

dB22,

dB24

};

enum responseWindowSizeEnumType

{

sf2,

sf3,

sf4,

sf5,

sf6,

sf7,

sf8,

sf10

};

enum sizeOfRAPreambleGroupAEnumType

{

n4,

n8,

n12,

n16,

n20,

n24,

n28,

n32,

n36,

n40,

n44,

n48,

n52,

n56,

n60,

};

enum timeToTriggerEUtraEnumType

{

ms0,

ms40,

ms64,

ms80,

ms100,

ms128,

ms160,

ms256,

ms320,

ms480,

ms512,

ms640,

ms1024,

ms1280,

ms2560,

ms5120

};

typedef sequence<string> DnList;

struct CellOutageCompensationStatus

{

CellOutageCompensationState cellOutageCompensationState;

DnList errorList;

};

};

#endif // _EUTRANNETWORKRESOURCESNRMDEFS_IDL_

End of modifications