3GPP TSG-SA WG4 Meeting #88	S4-160543

# Memphis, USA, 18-22 April, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.445 | CR | 0016 | rev | 1 | Current version: | 12.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to CMR Handling for AMR-WB IO mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVS_Codec |  |  |  |  |  |  | Date: |  | 2016-04-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The handling of a CMR received by the media sender is specified as follows: "The bit-rate in the CMR byte of EVS Primary mode indicates the highest bit-rate that the receiver wants to receive. The sender should use the bit-rate requested in the CMR and shall not use a higher bit-rate. The sender may use a lower bit-rate than the requested bit-rate within the set of negotiated bit-rates. If a range of audio bandwidths is negotiated for EVS Primary mode, the bandwidth in the CMR byte of EVS Primary mode indicates the highest bandwidth that the receiver wants to receive. The sender should use the bandwidth requested in the CMR. The sender may use a lower bandwidth within the set of negotiated bandwidths." The meaning of "bandwidth" and "bitrate" is not explicitly defined for the EVS AMR-WB IO mode, for AMR WB the terms "codec mode" and "codec rate" are often used as synonyms. It is not sufficiently specified, how the media sender in EVS primary mode shall react, when receiving a CMR for the EVS AMR-WB-IO mode and in particular if it would be permissable for a media sender to also send with EVS primary mode WB or EVS primary mode NB, when receiving a CMR for AMR-WB-IO mode. According to TS 26.114, Clause 12.3.4, CMR can be used during SRVCC to AMR-WB to request a switch from EVS primary mode to EVS AMR-WB-IO mode. It is thus essential that a media sender only sends AMR-WB-IO mode after receiving a CMR for EVS AMR-WB-IO mode. Furthermore, after SRVCC to AMR-WB it is only possible without transcoding to send EVS AMR-WB-IO mode codec modes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | After receiving a CMR for AMR-WB-IO mode, the media sender shall send AMR-WB-IO mode. The media receiver in the MTSI terminal shall be prepared to receive any speech frames within the negotiated media type parameter set, irrespective of the CMR it sends or receives. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification with potential contradictions to TS 26.114 can lead to call failures after SRVCC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Start change 1 ***

#### A.2.2.1.1	CMR byte

Codec Mode Request (CMR) byte structure is shown in Figure A.4. This CMR byte shall be present for EVS AMR-WB IO speech and SID frames in Header-Full format. For EVS Primary mode, the CMR byte is only used when a CMR needs to be transmitted or if required by session negotiation. The request indicated in the CMR byte shall comply with the media type parameters (e.g. allowed bit-rates or audio bandwidths) that are negotiated in the session.

NOTE 1:	There is no SDP MIME signalling parameter defined that can be used to disallow all CMRs with T-bits "001". However, the mode-set MIME parmeter can be used to restrain the allowed EVS AMR-WB IO codec modes. If this mode-set parameter is not included in the media type parameters, then all 9 modes of the EVS AMR-WB IO codes modes are allowed.

The media receiver in the MTSI terminal shall be prepared to receive any speech frames within the negotiated media type parameter set as well as SID frames, irrespective of the CMR it sends or receives.

NOTE 2:	The media receiver can receive such frames for various reasons. For instance, after a handover to AMR-WB, a MGW can send speech frames with an EVS AMR-WB IO codec mode even if it receives CMR byte of EVS Primary mode (T-bits not "001").

The bit-rate in the CMR byte of EVS Primary mode (T-bits not "001") indicates the highest bit-rate that the media receiver wants to receive. The media sender should use the bit-rate requested in the received CMR and shall not use a higher bit-rate. The media sender may use a lower bit-rate than the requested bit-rate within the set of negotiated bit-rates.

If a single audio bandwidth is negotiated for EVS Primary mode, the CMR shall indicate this bandwidth in its T-bits, unless the mode of operation is switched by a received CMR between EVS Primary and EVS AMR-WB IO.

If a range of audio bandwidths is negotiated for EVS Primary mode, then the audio bandwidth in the CMR byte of EVS Primary mode indicates the highest audio bandwidth that the media receiver wants to receive. The media sender should use the audio bandwidth requested in the received CMR.

A CMR with T-bits "001" (i.e. a CMR for the EVS AMR-WB IO mode of operation) indicates the highest EVS AMR-WB IO codec mode that the media receiver wants to receive. When receiving a CMR with T-bits "001", the media sender shall use the EVS AMR-WB IO mode of operation. The media sender should use the EVS AMR-WB IO codec mode (bit rate) requested in the received CMR and shall not use a higher codec mode (higher bit rate). The media sender may use a lower EVS AMR-WB IO codec mode within the negotiated mode-set.

When a CMR is received, requesting a bit-rate and/or audio bandwidth that does not comply with the negotiated media parameters, it shall be ignored.

The request in the received CMR is valid until a new request is received.

Figure A.4. CMR byte

H (1 bit): Header Type identification bit. For the CMR byte this bit is always set to 1.

T (3 bits): These bits indicate the Type of Request in order to distinguish EVS AMR-WB IO and EVS Primary bandwidths.

D (4 bits): These bits indicate the codec mode request.

The possible values and corresponding CMRs are defined in Table A.3.

Table A.3: Structure of CMR

| Code |  | Definition |  | Code |  | Definition |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T | D |  |  | T | D |  |  |
| 000 | 0000 | NB | 5.9 (VBR) | 010 | 0000 | WB | 5.9 (VBR) |
|  | 0001 | NB | 7.2 |  | 0001 | WB | 7.2 |
|  | 0010 | NB | 8.0 |  | 0010 | WB | 8 |
|  | 0011 | NB | 9.6 |  | 0011 | WB | 9.6 |
|  | 0100 | NB | 13.2 |  | 0100 | WB | 13.2 |
|  | 0101 | NB | 16.4 |  | 0101 | WB | 16.4 |
|  | 0110 | NB | 24.4 |  | 0110 | WB | 24.4 |
|  | 0111 |  | Not used |  | 0111 | WB | 32 |
|  | 1000 |  | Not used |  | 1000 | WB | 48 |
|  | 1001 |  | Not used |  | 1001 | WB | 64 |
|  | 1010 |  | Not used |  | 1010 | WB | 96 |
|  | 1011 |  | Not used |  | 1011 | WB | 128 |
|  | 1100 |  | Not used |  | 1100 |  | Not used |
|  | 1101 |  | Not used |  | 1101 |  | Not used |
|  | 1110 |  | Not used |  | 1110 |  | Not used |
|  | 1111 |  | Not used |  | 1111 |  | Not used |
| 001 | 0000 | IO | 6.6 | 011 | 0000 |  | Not used |
|  | 0001 | IO | 8.85 |  | 0001 |  | Not used |
|  | 0010 | IO | 12.65 |  | 0010 |  | Not used |
|  | 0011 | IO | 14.25 |  | 0011 | SWB | 9.6 |
|  | 0100 | IO | 15.85 |  | 0100 | SWB | 13.2 |
|  | 0101 | IO | 18.25 |  | 0101 | SWB | 16.4 |
|  | 0110 | IO | 19.85 |  | 0110 | SWB | 24.4 |
|  | 0111 | IO | 23.05 |  | 0111 | SWB | 32 |
|  | 1000 | IO | 23.85 |  | 1000 | SWB | 48 |
|  | 1001 |  | Not used |  | 1001 | SWB | 64 |
|  | 1010 |  | Not used |  | 1010 | SWB | 96 |
|  | 1011 |  | Not used |  | 1011 | SWB | 128 |
|  | 1100 |  | Not used |  | 1100 |  | Not used |
|  | 1101 |  | Not used |  | 1101 |  | Not used |
|  | 1110 |  | Not used |  | 1110 |  | Not used |
|  | 1111 |  | Not used |  | 1111 |  | Not used |

Table A.3: Structure of CMR (continued)

| Code |  | Definition |  | Code |  | Definition |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T | D |  |  | T | D |  |  |
| 100 | 0000 |  | Not used | 110 | 0000 | SWB | 13.2 CA-L-O2 |
|  | 0001 |  | Not used |  | 0001 | SWB | 13.2 CA-L-O3 |
|  | 0010 |  | Not used |  | 0010 | SWB | 13.2 CA-L-O5 |
|  | 0011 |  | Not used |  | 0011 | SWB | 13.2 CA-L-O7 |
|  | 0100 |  | Not used |  | 0100 | SWB | 13.2 CA-H-O2 |
|  | 0101 | FB | 16.4 |  | 0101 | SWB | 13.2 CA-H-O3 |
|  | 0110 | FB | 24.4 |  | 0110 | SWB | 13.2 CA-H-O5 |
|  | 0111 | FB | 32 |  | 0111 | SWB | 13.2 CA-H-O7 |
|  | 1000 | FB | 48 |  | 1000 |  | Not used |
|  | 1001 | FB | 64 |  | 1001 |  | Not used |
|  | 1010 | FB | 96 |  | 1010 |  | Not used |
|  | 1011 | FB | 128 |  | 1011 |  | Not used |
|  | 1100 |  | Not used |  | 1100 |  | Not used |
|  | 1101 |  | Not used |  | 1101 |  | Not used |
|  | 1110 |  | Not used |  | 1110 |  | Not used |
|  | 1111 |  | Not used |  | 1111 |  | Not used |
| 101 | 0000 | WB | 13.2 CA-L-O2 | 111 | 0000 | Reserved |  |
|  | 0001 | WB | 13.2 CA-L-O3 |  | 0001 | Reserved |  |
|  | 0010 | WB | 13.2 CA-L-O5 |  | 0010 | Reserved |  |
|  | 0011 | WB | 13.2 CA-L-O7 |  | 0011 | Reserved |  |
|  | 0100 | WB | 13.2 CA-H-O2 |  | 0100 | Reserved |  |
|  | 0101 | WB | 13.2 CA-H-O3 |  | 0101 | Reserved |  |
|  | 0110 | WB | 13.2 CA-H-O5 |  | 0110 | Reserved |  |
|  | 0111 | WB | 13.2 CA-H-O7 |  | 0111 | Reserved |  |
|  | 1000 |  | Not used |  | 1000 | Reserved |  |
|  | 1001 |  | Not used |  | 1001 | Reserved |  |
|  | 1010 |  | Not used |  | 1010 | Reserved |  |
|  | 1011 |  | Not used |  | 1011 | Reserved |  |
|  | 1100 |  | Not used |  | 1100 | Reserved |  |
|  | 1101 |  | Not used |  | 1101 | Reserved |  |
|  | 1110 |  | Not used |  | 1110 | Reserved |  |
|  | 1111 |  | Not used |  | 1111 | NO_REQ |  |

Table A.3: Structure of CMR (continued)

| Code |  | Definition |  | Code |  | Definition |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T | D |  |  | T | D |  |  |
| 100 | 0000 |  | Not used | 110 | 0000 | SWB | 13.2 CA-L-O2 |
|  | 0001 |  | Not used |  | 0001 | SWB | 13.2 CA-L-O3 |
|  | 0010 |  | Not used |  | 0010 | SWB | 13.2 CA-L-O5 |
|  | 0011 |  | Not used |  | 0011 | SWB | 13.2 CA-L-O7 |
|  | 0100 |  | Not used |  | 0100 | SWB | 13.2 CA-H-O2 |
|  | 0101 | FB | 16.4 |  | 0101 | SWB | 13.2 CA-H-O3 |
|  | 0110 | FB | 24.4 |  | 0110 | SWB | 13.2 CA-H-O5 |
|  | 0111 | FB | 32 |  | 0111 | SWB | 13.2 CA-H-O7 |
|  | 1000 | FB | 48 |  | 1000 |  | Not used |
|  | 1001 | FB | 64 |  | 1001 |  | Not used |
|  | 1010 | FB | 96 |  | 1010 |  | Not used |
|  | 1011 | FB | 128 |  | 1011 |  | Not used |
|  | 1100 |  | Not used |  | 1100 |  | Not used |
|  | 1101 |  | Not used |  | 1101 |  | Not used |
|  | 1110 |  | Not used |  | 1110 |  | Not used |
|  | 1111 |  | Not used |  | 1111 |  | Not used |
| 101 | 0000 | WB | 13.2 CA-L-O2 | 111 | 0000 | Reserved |  |
|  | 0001 | WB | 13.2 CA-L-O3 |  | 0001 | Reserved |  |
|  | 0010 | WB | 13.2 CA-L-O5 |  | 0010 | Reserved |  |
|  | 0011 | WB | 13.2 CA-L-O7 |  | 0011 | Reserved |  |
|  | 0100 | WB | 13.2 CA-H-O2 |  | 0100 | Reserved |  |
|  | 0101 | WB | 13.2 CA-H-O3 |  | 0101 | Reserved |  |
|  | 0110 | WB | 13.2 CA-H-O5 |  | 0110 | Reserved |  |
|  | 0111 | WB | 13.2 CA-H-O7 |  | 0111 | Reserved |  |
|  | 1000 |  | Not used |  | 1000 | Reserved |  |
|  | 1001 |  | Not used |  | 1001 | Reserved |  |
|  | 1010 |  | Not used |  | 1010 | Reserved |  |
|  | 1011 |  | Not used |  | 1011 | Reserved |  |
|  | 1100 |  | Not used |  | 1100 | Reserved |  |
|  | 1101 |  | Not used |  | 1101 | Reserved |  |
|  | 1110 |  | Not used |  | 1110 | Reserved |  |
|  | 1111 |  | Not used |  | 1111 | NO_REQ |  |