3GPP TSG-SA WG4 Meeting #88	S4-160556

# Memphis, USA, 18-22 April, 2016												Revision of S4-160460

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.131 | CR | 0071 | rev | 1 | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of brackets for channel-aware mode delay requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E_LTE_UED |  |  |  |  |  |  | Date: |  | 2016-04-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Delay requirements when the EVS codecs operates at 13.2 kbit/s with the channel-aware mode were introduced with delay figures in brackets; these brackets were used to indicate that requirements are provisional, pending confirmation. It has been confirmed in SA4#88 that these brackets can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of brackets in Tables 32b1 and 32b2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete requirements for channel-aware mode of operation in LTE UE delay tests. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.11.1, 7.11.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 7.11.1	Handset UE

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For MTSI-based speech-only with LTE access in error and jitter free conditions and EVS speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 157ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 197ms.

For MTSI-based speech-only with LTE access in conditions with simulated packet arrival time variations and packet loss and EVS speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 8bis, while meeting the speech quality targets defined.

NOTE: The UE delay requirements for MTSI-based speech-only with LTE access is derived from:

- A codec algorithmic delay of 32ms including speech frame buffering and encoder lookahead.

- An air interface transmission time of 1ms on receive and 1ms on the send direction.

- A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 8bis, and 80ms for test condition 2 of Table 8bis.

- A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 32a1: UE delay and speech quality requirements for LTE access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 157ms | TS + TR ≤ 197ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 157ms | TS + TR ≤ 197ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.4 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.4 |
| 3 | dly_profile_40msDRX_22pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF – 1.0 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. The delay profile for test condition 3 is theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 22%, with +/- 6ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132. When tested in EVS 13.2kbit/s channel aware mode, the UE shall meet the following additional requirements:

Table 32b1: UE delay and speech quality requirements for LTE access when tested in EVS 13.2kbit/s channel aware mode

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 157ms | TS + TR ≤ 197ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 157ms | TS + TR ≤ 197ms | MOS-LQOTEST ≥ MOS-LQOREF - 0,3 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF - 0,3 |
| 3 | dly_profile_40msDRX_22pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF - 0,5 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. The delay profiles for test condition 3 is theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 22%, with +/- 6ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

#### 7.11.2.1	Wired headset

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For MTSI-based speech-only with LTE access in error and jitter free conditions and EVS speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 157ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 197ms.

For MTSI-based speech-only with LTE access in conditions with simulated packet arrival time variations and packet loss and EVS speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 8ter, while meeting the speech quality targets defined.

NOTE: The UE delay requirements for MTSI-based speech-only with LTE access is derived from:

- A codec algorithmic delay of 32ms including speech frame buffering and encoder lookahead.

- An air interface transmission time of 1ms on receive and 1ms on the send direction.

- A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 8ter, and 80ms for test condition 2 of Table 8ter.

- A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 32a2: UE delay and speech quality requirements for LTE access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 157ms | TS + TR ≤ 197ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 157ms | TS + TR ≤ 197ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.4 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.4 |
| 3 | dly_profile_40msDRX_22pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF – 1.0 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. The delay profile for test condition 3 is theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 22%, with +/- 6ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132. When tested in EVS 13.2kbit/s channel aware mode, the UE shall meet the following additional requirements:

Table 32b2: UE delay and speech quality requirements for LTE access when tested in EVS 13.2kbit/s channel aware mode

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 157ms | TS + TR ≤ 197ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 157ms | TS + TR ≤ 197ms | MOS-LQOTEST ≥ MOS-LQOREF - 0,3 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF - 0,3 |
| 3 | dly_profile_40msDRX_22pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF - 0,5 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. The delay profiles for test condition 3 is theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 22%, with +/- 6ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

*** End of changes ***