3GPP TSG-SA WG6 Meeting #13	S6-161269

Sophia Antipolis, France, 10th – 14th October 2016	(revision of S6-161257)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.179 | CR | 0122 | rev | 3 | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location cancel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Trunking, ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCPTT |  |  |  |  |  |  | Date: |  | 2016-10-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Considering save resources, e.g. CPU, memory,UE power, when location reporting or location information is no longer needed, location reporting or location information shall be canceled. For example, the UE whithout GPS signal received location information request, and it will continue to search GPS signals with power consumption for a long time. If the requestor has not received location report for a while, the requestor may cancel this location information to save UE power. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description for location cancel by reusing location reporting configuration message in 3GPP TS23.179, and necessary clarification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lots of resources will be wasted and the specification will be incomplete due to location cancel missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.14.2.1,10.14.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | x |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

10.14.2.1	Location reporting configuration

Table 10.14.2.1-1 describes the information flow from the MCPTT server to the MCPTT client for the location reporting configuration.

Table 10.14.2.1-1: Location reporting configuration

| Information element | Status | Description |
| --- | --- | --- |
| Requested non-emergency location information | O (NOTE) | Identifies what location information is requested, except for MCPTT emergency or imminent peril calls or MCPTT emergency alerts |
| Requested emergency location information | O (NOTE) | Identifies what location information is requested, for MCPTT emergency or imminent peril calls or MCPTT emergency alerts |
| Triggering criteria in non- emergency cases | O (NOTE) | Identifies when the MCPTT client will send the location report in non-emergency cases |
| Minimum time between consecutive reports | O (NOTE) | Defaults to 0 if absent and 0 for MCPTT emergency calls, imminent peril calls and MCPTT emergency alerts |
| NOTE : .If none of the information elements is present, this represents a cancellation for location reporting. |  |  |

* * * Second Change * * * *

10.14.2.4	Event-triggered location reporting procedure

The MCPTT server provides location reporting configuration to the MCPTT clients, indicating what information the MCPTT server expects and what events will trigger the sending of this information to the MCPTT server. The decision to report location information can be triggered at the MCPTT client by different conditions, e.g., the reception of the location reporting configuration, initial registration, distance travelled, elapsed time, cell change, MBMS SAI change, tracking area change, PLMN change, MCPTT call initiation, or other types of events such as MCPTT emergency alert, MCPTT emergency call or MCPTT imminent peril calls. The location report can include information described in subclause 7.5.2.2 and other location information. If no information element is contained in location reporting configuration, it indicates a cancellation for location reporting.

Figure 10.14.2.4-1: Event-triggered location reporting procedure

- The MCPTT server sends location reporting configuration to the MCPTT client containing the initial configuration (or a subsequent update) for reporting the location of the MCPTT UE. If no information element is contained in location reporting configuration, it indicates a cancellation for location reporting, and then step 2-3 are not needed.

NOTE 1:	The location reporting configuration information can be made part of the MCPTT user profile, in which case the sending of the message is not necessary for location reporting.

NOTE 2:	Different MCPTT clients may be given different location reporting criteria.

2.	A location reporting event occurs, triggering step 3.

3.	The MCPTT client sends a location information report to the MCPTT server, containing location information identified by the MCPTT server and available to the MCPTT client.

* * * End of changes * * * *