5GAA Working Group 1 meeting	                                                        		T-170311

Conference Call

2017

Source:		Continental

Title:		Hazardous Location Warning

Agenda Item:	Safety Use Case Description

Document for:	Discussion

# Introduction

5GAA WG1 has recently defined two use cases:

- HV Forward Hazard (Slippery) Road Warning

- Hazardous Location Warning

During the post-conference call #12 discussion which focused on

- Solution agnostic approach

- Application Server role

It was determined that these use cases have significant overlap and that combining them into a single use case would be appropriate. This combined use case attempts to remove some of the inconsistencies that were observed during the review. In particular, the description below should be solution agnostic while not precluding other roles in the solution space including the application server role.

Scenario application zone is the concept used in the definition of the use cases. The scenario application zone defines the problem space or region relative to Host Vehicle (HV). Remote actors and situations present themselves as problems when they are in this region. Remote actors and situations that fall outside this region do not have a role in the use case scenario.

# Proposed Use Case Description

| Use Case Name | Hazardous Location Warning |
| --- | --- |
| User Story/Use Case Scenario | The host vehicle is warned when approaching a hazardous location. |
| Category | Safety |
| Road Environment | Intersection ¦ Urban ¦ Rural ¦ Highway |
| Short Description | The host vehicle is warned of an approaching hazardous location. |
| Actors | - Host Vehicle(HV) |
| Vehicle Roles | - HV is the vehicle approaching a hazardous location |
| Road & Roadside Infrastructure Roles | - Roads are defined by their lane designations, geometry and condition. |
| Other Actors’ Roles | Not applicable |
| Goal | - Warn the HV of a hazardous location and provide additional information to safely navigate around the hazard. |
| Needs | - HV needs to know the location, geometry and type of hazard |
| Constraints / Presumptions |  |
| Geographic Scope | - Global |
| Illustrations |  |
| Pre-Conditions | - HV is moving forward - Known hazard is defined by its location, geometry and type of hazard - The “Current Lane” scenario application zone is determined from: - HV’s location and dynamics - HV’s safe stopping distance - lane designations and geometry - road conditions (if available) - The “Adjacent Lane” scenario application zone is determined from: - HV’s location and dynamics - HV’s safe stopping distance - lane designations and geometry - road conditions (if available) - The “Navigation Route” scenario application zone is determined from: - HV’s location - HV’s planned navigation route |
| Main Event Flow | - If the hazard’s location is in the “Current Lane” scenario application zone; - Warn HV of the hazard’s location, geometry and type |
| Alternate Event Flow | - Else If hazard’s location is in the adjacent lane scenario application zone; - Caution HV of the adjacent hazard’s location, geometry and type |
| Alternate Event Flow | - Else If the hazard’s location is in the Navigation Route scenario application zone; - Alert HV of the hazard’s location and type |
| Post-Conditions | - HV is aware of the hazard’s location, geometry and type. |
| Service-Level Key Performance Indicators | - Communications Range - Positioning Accuracy - Data rate - Download size - Latency - Data validity |
| Information Requirements | - HV’s location and dynamics - HV’s safe stopping distance - HV’s planned navigation route (if available) - Lane designations and geometry Hazard location, geometry, type, cause, possible impact, etc. |

# Conclusion

In this contribution we have proposed a description of the use case “Hazardous Location Warning” based on the temperate T-170109 [1].

# References

- 5GAA T-170109 New template for use cases definitions, June 2017