3GPP TSG-SA2 Meeting #127bis	S2-186158

# Newport Beach, United States, 28th May 2018 - 1st Jun 2018		revision of S2-186010

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0416 | rev | 2 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify the relationship between GFBR and MDBV |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For a delay critical QoS flow, the PDB and PER requirements should be satisfied for the traffic not exceeding the MDBV. And as the delay critical QoS flow is GBR resource type, the PDB and PER requirements should be satisfied for the traffic not exceeding the GFBR. Althgh the GFBR is a granteed bit rate while MDBV is a traffic volume, and the time scales for GFBR and MDBV are different, there’s relationship between the two parameters. For example, if the GFBR of a QoS flow is a very large value, the traffic of the QoS flow may always exceed the MDBV, in this case although the bit rate of the traffic is under GFBR, the PDB and PER requirements can’t be satisfied due to the MDBV limitation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For a delay critical QoS flow, the GFBR setting shouldn’t conflict with the MDBV limit. I.e.the GFBR shouldn’t be too large, so that it’s possible for the traffic not exceeding GFBR can meet the MDBV limit. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the GFBR is set to a large value, the traffic of the QoS flow may always exceed the MDBV, the PDB and PER requirements can’t be satisfied although it’s a GBR QoS flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 CR S2-185604 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************************************Start of 1st change*****************************************

5.7.2.5	Flow Bit Rates

For GBR QoS Flows, the 5G QoS profile additionally include the following QoS parameters:

-	Guaranteed Flow Bit Rate (GFBR) - UL and DL;

-	Maximum Flow Bit Rate (MFBR) -- UL and DL.

The GFBR denotes the bit rate that is guaranteed to be provided by the network to a GBR and Delay Critical GBR QoS Flow, over the Averaging Time Window. The MFBR limits the bit rate that is expected by a GBR QoS Flow (e.g. excess traffic may get discarded or delayed by a rate shaping or policing function at the UE, RAN, UPF).

GFBR and MFBR are signalled on N2 and N11 for each of the GBR QoS Flows.

NOTE:	For each QoS Flow of Delay Critical GBR resource type, the SMF can ensure that the GFBR of the QoS Flow can be achieved with the MDBV of the QoS Flow using the QoS Flow binding functionality described in clause 6.1.3.2.4 in TS 23.503 [45].

**************************************End of changes*******************************************