3GPP TSG-SA WG3 Meeting #92 	S3-182206

# Dalian (China), 20-24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0258 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Align NAS connection identifier with access type identifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanhai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-07-19 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently the NAS connection identifier is different from access type identifier. In Annex A.9, for KgNB computation and KN3IWF computataion, in the Table A.9-1: Access type distinguishers have been defined as, Access type distinguisher , 3GPP access: 0x01 and Non 3GPP access: 0x02 But in clause 6.4.2.2, for NAS connection identifier different values have been defined, (“0’” for 3GPP access and “1” for Non 3GPP access). There is no reason for these values to be different, it is better to align both of these values to avoid confusion in implementations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 6.4.2.2 change the value of the unique NAS connection identifier over 3GPP access to "0x01" and identifier for non-3GPP access to “0x02”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This may lead to confusing implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

******************************* Start of Changes ***************************************

### 6.4.2	Security for multiple NAS connections

#### 6.4.2.1	Multiple active NAS connections with different PLMNs

TS;23.501 [2] has a scenario when the UE is registered to a VPLMN's serving network via 3GPP access and to another VPLMN's or HPLMN's serving network via non-3GPP access at the same time. When the UE is registered in one PLMN's serving network over a certain type of access (e.g. 3GPP) and is registered to another PLMN's serving network over another type of access (e.g. non-3GPP), then the UE has two active NAS connections with different AMF's in different PLMNs. As described in clause 6.3.2.1, the UE shall independently maintain and use two different 5G security contexts, one per PLMN serving network. Each security context shall be established separately via a successful primary authentication procedure with the Home PLMN. All the NAS and AS security mechanisms defined for single registration mode are applicable independently on each access using the corresponding 5G security context.x01

NOTE: 	The UE belongs to a single HPLMN.

#### 6.4.2.2	Multiple active NAS connections in the same PLMN's serving network

When the UE is registered in a serving network over two types of access (e.g. 3GPP and non-3GPP), then the UE has two active NAS connections with the same AMF. A common 5G NAS security context is created during the registration procedure over the first access type.

In order to realize cryptographic separation and replay protection, the common NAS security-context shall have parameters specific to each NAS connection. The connection specific parameters include a pair of NAS COUNTs for uplink and downlink and unique NAS connection identifier. The value of the unique NAS connection identifier shall be set to "0x01" for 3GPP access and set to "0x02" for non-3GPP access.

In mobility and interworking scenarios, a newly created partial NAS security context is activated only on the NAS connection reporting the mobility, NAS context is enabled one by one for multiple connections.

When the UE is simulanteously registered over both types of accesses, and if an authentication procedure followed by a NAS SMC run takes place over one of the accesses (say access A), then the new NAS security context shall only be activated over that access (access A). The UE and the AMF shall retain and continue to use the old NAS security context over the other access (say access B). In order to activate the new NAS security context over the other access (access B), the AMF shall trigger a NAS SMC run over that access. During the second NAS SMC run (on access B), the AMF shall include the same ngKSI associated with the new NAS security context. After a successful second NAS SMC procedure over the other access (access B), both the UE and the AMF shall delete the old NAS security context.

******************************** End of changes ****************************************