3GPP TSG-SA Meeting #82	SP-181238

# Sorrento, Italy, 12 - 14 December 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 22.125 | CR | 0001 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of requirement on enforcement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Deutsche Telekom, Vodafone |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2018-12-13 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Using of ‘enforce’ place the MNO to be responsible for the UAS behavior and make sure that authorizations and instructions from UTM are followed. This goes beyond MNO responsibilities and brings additional burden on the MNO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the requirements on enforcement in Chapter 5.1 and 5.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements added which go beyond the responsibilities of MNOs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

************** first change **************

## 5.1	General

The 3GPP system should enable UTM to associate the UAV and UAV controller, identify them as a UAS.

The 3GPP system shall enable a UAS to send UTM the UAV data which can contain: unique identity (this may be unique 3GPP identity), UE capability of the UAV, make & model, serial number, take-off weight, position, owner identity, owner address, owner contact details, owner certification, take-off location, mission type, route data, operating status.

The 3GPP system shall enable a UAS to send UTM the UAV controller data which can contains: unique identity, UE capability of the UAV controller, position, owner identity, owner address, owner contact details, owner certification, UAV operator identity, UAV operator license, UAV operator certification, UAV pilot identity, UAV pilot license, UAV pilot certification and flight plan.

The 3GPP system shall enable a UAS to send different UAS data to UTM based on the different authentication and authorizations level which are applied to the UAS.

NOTE: 	Subject to the regional regulation, the different authentication and authorization levels can be: the initial network access authentication and authorization, UAS identity authentication, UAV flight plan authorization, additional UMT service authentications, such as flight monitoring, collision avoidance services, so on.

The 3GPP system shall support capability to extend UAS data being sent to UTM with the evolution of UTM and its support applications in future.

Based on regulations and security protection, the 3GPP system shall enable a UAS to send UTM the identifiers which can be: IMEI, MSISDN, or IMSI, or IP address.

The 3GPP system shall enable a UE in a UAS to send the following identifiers to a UTM: IMEI, MSISDN, or IMSI, or IP address

The 3GPP system should enable an MNO to augment the data sent to a UTM with the following: network-based positioning information of UAV and UAV controller.

NOTE: 	This augmentation may be trust-based (i.e. the MNO informs the UTM that the UAV position information is trusted) or it may be additional location information based on network information.

NOTE: 	This requirement will not be applied to the case which the UAS and UTM has direct control communication connection without going through MNO.

The 3GPP system shall enable UTM to inform an MNO of the outcome of an authorisation to operate.

The 3GPP system shall enable an MNO to allow a UAS authorisation request only if appropriate subscription information is present.

The 3GPP system shall be able to provide UTM with the identity/identities of a UAS.

The 3GPP system shall enable a UAS to update a UTM with the live location information of a UAV and its UAV controller.

The 3GPP network should be able to provide supplement location information of UAV and its controller to a UTM.

NOTE: 	This supplement may be trust-based (i.e. the MNO informs the UTM that the UAV position information is trusted) or it may be additional location information based on network information.

The 3GPP network shall support UAVs and the corresponding UAV controller are connecting to different PLMNs at the same time.

The 3GPP system shall provide the capability for network to obtain the UAS information regarding its support of 3GPP communication capabilities designed for UAS operation.

The 3GPP system shall support the UAS identification and subscription data which can differentiate the UAS with UAS-capable UE and the UAS with non-UAS-capable UE.

NOTE: 	UAS-capable UE refers to the UE which support interaction capability with UTM and certain 3GPP communication features which 3GPP provides for UAS

The 3GPP system shall support detection, identification and reporting of problematic UAVs to a UTM.

NOTE: 	A problematic UAV can be a UAV flying without successful authorization from UTM;

********* end of first change **********

********* second change *************

## 5.2	Centralised UAV traffic management

The 3GPP system shall provide a mechanism for a UTM to provide route data, along with flight clearance, to a UAV.

The 3GPP system shall enable a UTM to send route modification information to a UAS with a latency of less than 500ms.

The 3GPP system shall enable a UTM to send a notification to a UAV controller with a latency of less than 500ms.

********* end of second change **********