3GPP TSG-SA5 Meeting #125 	S5-193416

# Newport Beach, US, 8-12 April 2019												Revision of S5-193143

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0019 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editor’s change for configuration management service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-PRO_NS |  |  |  |  |  | Date: |  |  | 2019-03-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The term “configuration management service” is used in some places in the specification, however, currently there is no definition and description for configuration management service. Other usecases in this sepecification use term management service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change configuration management service to management service to align with other use cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will be published with incorrect term. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.12, 5.1.13, 5.1.18, 5.1.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.12	Network slice subnet instance modification

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To modify an existing network slice subnet instance |  |
| Actors and Roles | NetworkSliceSubnet management service consumer. For example, NSMF or NSSMF plays the role of NetworkSliceSubnet management service consumer. |  |
| Telecom resources | Network slice subnet instance NetworkSliceSubnet management service provider. For example, NSSMF plays the role of NetworkSliceSubnet management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | N/A |  |
| Begins when | The NetworkSliceSubnet management service provider receives request from its authorized consumer with new sets of network slice subnet related requirements and decides to modify an existing NSSI. |  |
| Step 1 (M) | The NetworkSliceSubnet management service provider identifies the NSSI constituents as well as the transport network (TN) part within the NSSI that needs to be modified, and generates new sets of requirements for the NSSI constituents and transport network if needed. |  |
| Step 2 (M) | The NetworkSliceSubnet management service provider checks whether the requirements for the identified NSSI constituents managed by itself could be satisfied, and then triggers the modification of the corresponding NSSI constituents if needed. |  |
| Step 3 (M) | If the NSSI consists of constituent NSSI managed by other NetworkSliceSubnet management service provider, and the constituent NSSI is identified to be modified, the NetworkSliceSubnet management service provider sends modification request to other NetworkSliceSubnet management service provider which manages the constituent NSSI with new sets of constituent NSSI requirements. | Network slice subnet instance modification use case |
| Step 4 (M) | If the NS instance associated with the NSSI needs to be modified, the NetworkSliceSubnet management service provider derives the new sets of NS related requirements and triggers corresponding NS instance request to NFVO with Os-Ma-nfvo interface as described in clause 6.4.3 in TS 28.525 [2]. | TS 28.525 [2] Clause 6.4.3 NS instance use cases |
| Step 5 (M) | If the related TN part of the NSSI is identified to be modified, the NetworkSliceSubnet management service provider derives new sets of requirements for the TN part and coordinates with the corresponding TN management system. |  |
| Step 6 (M) | The NetworkSliceSubnet management service provider generates the modification result based on the received response and send response to its authorized consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NSSI is modified. |  |
| Traceability | REQ-PRO_NSSI-FUN-11 |  |

5.1.13	Network slice subnet configuration

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To provide service for slice-specific (re)configuration of NSSI. |  |
| Actors and Roles | NSS management service consumer (e.g., the operator or NSMF) |  |
| Telecom resources | NSS management service provider (e.g., NSSMF) Network slice subnet instance NF(s) |  |
| Assumptions | Authorized NSS management service consumer provide slice operation information (see 4.2) for (re-)configuring NSSI constituents. |  |
| Pre-conditions | NSSI exists. |  |
| Begins when | NSS management service consumer wants to (re-)configure the constituents of a NSSI. |  |
| Step 1 (M) | NSS management service consumer sends requests to NSS management service provider with slice operation information for (re-)configuring a network slice subnet. |  |
| Step 2 (M) | NSS management service provider (derives and) decomposes the received slice operation information, and then makes them as separate CM requests for each constituent if necessary and applicable. These (decomposed) requests may be delegated to other CM service providers (e.g., other NSS service providers, CM of NFs) with corresponding slice operation information. These requests may contain configuration for specific NFs such as 1) Configuration of dedicated NFs (e.g., configure the SMF with the information of new instantiated UPFs, see 6.3.2, 6.3.3 in [3]) and 2) Configuration of shared NFs (see 4.2 so that this information can be accessed by other constituents of the NSS (e.g., NSSF, AMF, SMF). |  |
| Step 3 (M) | NSS management service provider sends the processing result to NSS management service consumer (might be based on applicable processing results from other CM service providers). |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The required (re)configuration is configured at the corresponding constituent(s). |  |
| Traceability | REQ-PRO_NSSI-FUN-16 |  |

Second change

5.1.18	Configuration of a 3GPP NF instance

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request configuration of a 3GPP NF instance. |  |
| Actors and Roles | An authorized consumer of NF management service. |  |
| Telecom resources | ETSI NFV MANO system; NF management service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The NF to be configured has been instantiated; The MOI of the NF has been created. |  |
| Begins when | The authorized consumer needs to configure a 3GPP NF instance. |  |
| Step 1 (M) | The consumer requests the NF management service producer to modify the attribute(s) of the MOI of the 3GPP NF instance. |  |
| Step 2 (O) | If the 3GPP NF contains virtualized part and the corresponding VNF instance(s) need to be updated, the NF configuration service producer interacts, or requests another NF management service producer to interact, with ETSI NFV MANO system to update the corresponding VNF instance(s). |  |
| Step 3 (M) | The NF management service producer configures the 3GPP NF instance, per the MOI attribute modification request received from the consumer. |  |
| Step 4 (M) | The NF management service producer modifies the attributes of the MOI and informs the consumer that the 3GPP NF instance has been configured successfully. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The 3GPP NF instance has been configured. |  |
| Traceability | REQ-PRO_NF-FUN-4, REQ-PRO_NF-FUN-5, REQ-PRO_NF-FUN-6, REQ-PRO_NF-FUN-3 |  |

Third change

5.1.20	Configuration of a 3GPP sub-network

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request configuration of a 3GPP sub-network. |  |
| Actors and Roles | An authorized consumer of network configuration service. |  |
| Telecom resources | 3GPP network; 3GPP NFs; ETSI NFG MANO system; Network management service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The 3GPP sub-network has been created; The MOI(s) related to the sub-network has been created. |  |
| Begins when | The authorized consumer needs to configure a 3GPP sub-network. |  |
| Step 1 (M) | The authorized consumer requests to configure a 3GPP sub-network. |  |
| Step 2 (M) | The consumer requests the network management service producer to modify the attribute of the MOI(s) related to the 3GPP sub-network. |  |
| Step 3 (O) | If the 3GPP network is realized by NS(s) (ETSI ISG NFV concept), the network management service producer requests (directly or indirectly via another) ETSI NFV MANO system to update the NS(s) realizing the 3GPP sub-network. |  |
| Step 4 (O) | If there are new VNFs instantiated by the NS update, ETSI NFV MANO system informs the NF management service producer about the instantiation of VNFs. |  |
| Step 5 (O) | The NF management service producer creates the MOI(s) of the 3GPP NFs that are realized by the newly instantiated VNF(s). |  |
| Step 6 (M) | The network management service producer consumes the NF management service to configure the impacted 3GPP NF instance(s). | NF configuration service |
| Step 7 (M) | The network management service producer configures the 3GPP sub-network, per the MOI attribute modification request received from the consumer. |  |
| Step 8 (M) | The NF management service producer modifies the attributes of the MOI(s) of the 3GPP network and informs the consumer that the 3GPP sub-network has been configured successfully. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The 3GPP network has been configured. |  |
| Traceability | REQ-PRO_NW-FUN-3, REQ-PRO_NW-FUN-4 |  |

End