3GPP TSG-SA5 Meeting #125	S5-193382

Revision of S5-193058

# Newport Beach, U.S.A., 08 - 12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0074 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and definitions of packet loss measurement over N3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, KT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019/4/2 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to provide performance assurance of 5G services, it is important to measure end-to-end performance. Performance measurement at gNB level may not satisfy this requirement, Packet loss, delay, or latency can happen between gBN and UPF and their measurement thus is also very important. Performance measurements from UE to gNB is well defined but those of N3 is currently missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following measurements: - Incoming GTP data packet loss on the N3 interface, from (R)AN to UPF - Outgoing GTP data packet loss on the N3 interface, from UPF to (R)AN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance measurement besides data volume for UP traffic on N3 cannot be made. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.X (new), 5.4.1.Y (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.4.1.x  Incoming GTP Data Packet Loss

a)	This measurement provides the numer of GTP data packets which are not successfully received at UPF. It is a measure of the incoming GTP data packet loss per N3 on an UPF interface.  The measurement is split into subcounters per QoS level (5QI).

b)	CC

c)	This measurement is obtained by a counter: Number of missing incoming GTP sequence numbers (TS 29.281) among all GTP packets delivered by a gNB to an UPF interface. Separate counter is maintained for each 5QI.

d)	Each measurement is an integer value representing the number of the lost GTP pakets. If the QoS level measurement is perfomed, the measurements are equal to the number of 5QIs.

e)	The measurement name has the form GTP.InDataPktPacketLossN3UPF or GTP.InDataPktPacketLossN3UPF.QoS where QoS identifies the target quality of service class.

f)	EP_N3

g)	Valid for packet switched traffic

h)	5GS

5.4.1.y  Outgoing GTP Data Packet Loss

a)	This measurement provides the number of GTP data packets which are not successfully received at gNB over N3. It is a measure of the outgoing GTP data packet loss per N3 on an UPF interface.  The measurement is split into subcounters per QoS level (5QI).

b)	CC

c)	This measurement is obtained by a counter: Number of missing outgoing GTP sequence numbers (TS 29.281) among all GTP packets delivered by an UPF interface to a gNB. Separate counter is maintained for each 5QI.

d)	Each measurement is an integer value representing the lost GTP packets.. If the QoS level measurement is perfomed, the measurements are equal to the number of 5QIs.

e)	The measurement name has the form GTP.OutDataPktPacketLossN3UPF or GTP.OutDataPktPacketLossN3UPF.QoS where QoS identifies the target quality of service class.

f)	EP_N3

g)	Valid for packet switched traffic

h)	5GS

Next modified section

A.x	Monitoring of incoming/outgoing GTP packet loss on N3

Keeping track of GTP data packet loss over N3 is essential, since for certain services packets that are lost along the way through the system may have a noticeable impact on the end user. Incoming/outgoing GTP data packet loss measurements can be useful for evaluation, optimization, and performance assurance between gNB and UPF in the core.  It is also important for the performance measurement of end-to-end point of view from UE to UPF. Performance degradation can happen any point although the focus is centered more at UE and RAN.

End of modifications