# 3GPP TSG SA WG5 (Telecom Management) Meeting #125	S5-193363

# 08 April – 12 April 2019, Newport Beach, USA	Revision of S5-193129

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.425 | CR | 0188 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| c |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Add measurement on RRC connection usage per UE multi RAT capability |  |  |  |  |  |  |  |  |  |
| Source to WG: | P.I. Works |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | An objective for an operator is to utilize installed network functions as much as possible for efficiency reasons because each additional technology layer constitutes an important portion of total capex expenditures. A generic followed approach is to configure network parameters so that UEs that support both new and old radio access technology layers would camp on and use services from new radio access technology for better cost per bit characteristics. Therefore, it is of utmost importance for an operator to know if new technology capable UEs are served by old radio access technology in geographical areas where new technology is overlayed on top. There are no performance measurements defined for actual volume of calls per UE multi-RAT capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added use case and measurement on RRC connection usage per UE multi-RAT capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No performance measurements available for actual volume of calls per UE multi-RAT capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3.x (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First modified section

#### 4.1.3.x	RRC connection usage per UE multi-RAT capability

a)	This measurement provides RRC connection usage per UE multi-RAT capability. The measurement is split into subcounters per UE multi-RAT capability.

b)	DER (n=1)

c)	This measurement is obtained by accumulating successful RRC connections per UE multi-RAT capability after the receipt of a RRC CONNECTION SETUP COMPLETE message by the eNodeB/RN. The possible UE multi-RAT capabilities are included in TS 36.331 [8]. One or more subcounters are stepped based on received UE multi-RAT capabilities.

d)	Each measurement is an integer value.

e)	RRC.ConnUsage.geranCs

RRC.ConnUsage.geranPs

RRC.ConnUsage.cdma20001xRTT

RRC.ConnUsage.utra

RRC.ConnUsage.eutra

RRC.ConnUsage.eutra-nr

RRC.ConnUsage.nr

f)	EUtranCellFDD
EUtranCellTDD

g)	Valid for packet switching.

h)	EPS

Second modified section

# A.x	Monitoring of RRC connection usage per UE multi-RAT capability

An objective for an operator is to utilize installed network functions as much as possible for efficiency reasons because each additional technology layer constitutes an important portion of total capex expenditures. A generic followed approach is to configure network parameters so that UEs that support both new and old radio access technology layers would camp on and use services from new radio access technology for better cost per bit characteristics. Therefore, it is of utmost importance for an operator to know if new technology capable UEs are served by old radio access technology in geographical areas where new technology is overlayed on top.

RRC connection usage per UE multi-RAT capability related measurement will be helpful for operators to identify how efficient they utilize their deployed radio access technology layers and perform corrective actions when needed.

End of modifications